RGIK guidelines for compiling consistent rock glacier inventories. Brardinoni, F., Vivero, S., Barboux, C., Bodin, X., Cicoira, A., Echelard, T., Hu, Y., Jones, N., Lambiel, C., MacDonell, S., Pellet, C., Rouyet, L., Ruiz, L., Schaffer, N., Wehbe, M., and Delaloye, L.

# Supplementary Material

Figure S1. Specific examples of front types: a) talus-like (Les Cliosses, Switzerland, 07/08/2016); b) exaggerated (Olivares Theta, Chile, 23/03/2012); c) truncated (Tsarmine, Switzerland, 07/08/2016); d) bulgy (Gemmi, Switzerland, 22/09/2022). See text in Section 3.2 for more information. ©Google Earth (a-d).

S1. Location

Each RGU and RGS, as referenced in Section 3.3, must be marked by a Primary Marker (PM) at a specific point. The PM is an essential element in any RoGI. All additional characteristics associated with a RGU or RGS are then connected to this PM. The primary attributes of a PM are to:

- locate the RGU/RGS within a georeferenced coordinate system;

- distinguish each RGU/RGS from adjacent ones; and

- associate a RGS with its constituent RGUs, and vice versa.

The positioning of the PM on the RGU/RGS should minimize any temporal changes and updates. It should solely indicate the location of the RGU/RGS and clearly distinguish it from adjacent ones. The only guideline for positioning is that the point must be situated in the lower half of the RGU/RGS. It ought to be positioned arbitrarily upslope but still close to the rock glacier front, within the defined boundaries of the RGU/RGS.

Each PM is assigned a unique code (primary ID) based on its hierarchical classification as a unit (RGU) or a system (RGS), accompanied by its World Geodetic System 1984 (WGS84) coordinates in decimal degrees down to four decimal places. The primary ID code will always consist of 18 characters. For instance, an RGU situated at 46.5379°N, 0.9660°E (decimal degrees) would be labelled as RGU465379N0096605E (Figure S2). It is important to remember that each RGU should be linked to one RGS only.

Figure S2. The primary marker is situated in the lower half of the rock glacier unit or system and is distinct from other nearby rock glaciers. Each primary marker is designated by a unique code (primary ID) based on its hierarchical status as a unit (RGU) or system (RGS), followed by the relevant WGS84 coordinates expressed in decimal degrees down to four decimal places. In this example, the three RGUs (red circles) are parts of a single RGS (yellow circle). Tellers Davains, Switzerland (46.5383°N, 9.6775°E, 2200–2800 m a.s.l.). Arrows indicate main flow directions. ©Swisstopo.

S2.	Characterization

Any rock glacier unit (RGU) identified by a PM may be assigned a total of thirteen attributes (Table S1). Besides the attribution of three mandatory attributes (i.e., primary ID, hosting RGS (if any), and metadata), the remaining attributes (e.g., morphology, the type of upslope connection, and the degree of activity) are optional, although recommended. Similarly, a given rock glacier system (RGS) may be assigned a total of six attributes, three mandatory (i.e., primary ID, associated RGUs, and metadata) and three optional ones (i.e., composition, delineation, and delineation type) (Table S2). In the attribute table of an RGU (or RGS) the term "n.a." (not available) applies to any field that has not been evaluated.

During the compilation and/or the update of a RoGI, careful and exhaustive compilation of the relevant metadata (e.g., data source, date of mapping, name of operator(s), and review) is critical for providing adequate context of the methodological workflow, as well as for ensuring transparency and reproducibility of the data collection process. Detailed and complete metadata enable end users to appraise data sources and reliability (or limitations), thereby facilitating optimal utilization of RoGI products by the broader international community (Rouyet et al., 2025).

The conceptual characterizations illustrated in Tables S1 and S2 are meant for supporting the development of a RoGI database or GIS tools.

Table S1. Main attributes of a Rock Glacier Unit (RGU) in a RoGI.

| Attribute | Values / Units | Description |
| --- | --- | --- |
| Primary (mandatory) |  |  |
| Primary ID | RGU + 15 characters | Unique identifier for the RGU associated by its PM (Figure S2). |
| Associated RGS | RGS + 15 characters | Indicates the associated RGS which must be unique. |
| Metadata |  | Source data, date of mapping, mapper’s name, reviewer’s name. |
| Optional |  |  |
| Rock Glacier Morphology | Simple, Complex, n.a. | Provides a description of the morphology of the RGU (see Section 3.3). |
| Rock Glacier Completeness | Yes, No, n.a. | Indicates if the RGU encompasses the full landform (yes) or just a portion (no). This pertains to whether the RGU includes the complete sequence from its rooting zone to its front. Masking usually occurs when the RGU is (partly) buried by another RGU or by more recent gravity-driven deposits. |
| Upslope Connection | Talus, Debris-mantled slope, Landslide, Glacier, Glacier-forefield, Other, Poly-connected, Uncertain, Unknown, n.a. | Describes the connection between the RGU and its geomorphological upslope unit. The value "uncertain" is assigned when the geomorphological assessment cannot be confidently performed. "Unknown" is used when the RGU is overridden (or partly buried) by another, making it difficult to confidently assess the original connection to the upslope unit. |
| Upslope Currently | Yes, No, Uncertain, Unknown | Indicates if the rock glacier is currently attached to the upslope unit (noting only talus-connected situations). |
| Activity Assessment | Geomorphological, Kinematic, n.a. | Specifies whether the activity assessment is conducted only using geomorphological criteria or with the use of kinematic data. In the second case, it is necessary to include the type and date of the source data. |
| Kinematic Attribute | Undefined, < cm/yr, cm/yr, cm/yr to dm/yr, dm/yr, dm/yr to m/yr, m/yr, >m/yr | Indicates categorical (semi-quantitative) velocity information (Bertone et al., 2022). Not to be confused with RGV. |
| Activity | Active, Transitional, Relict, Active uncertain, Relict uncertain, Uncertain, n.a. | Describes the spatial extent and degree of surface displacement of the RGU. "Active uncertain" indicates that the RGU is not relict, yet there is inadequate data or geomorphological evidence to classify it as either "active" or "transitional." Conversely, "relict uncertain" implies that the unit is not active, but that there is insufficient data or evidence to determine whether it is "transitional" or "relict." The term "uncertain" is applied when the data quality is inadequate to assign any state of activity. |
| Destabilization | Yes (ongoing), Yes (completed), No, n.a. | Indicates the process of a destabilization phase of the RGU. The term "ongoing" indicates that the geomorphological evidence and/or kinematic data indicate a current phase of destabilization, while "completed" indicates that the geomorphological evidence and/or kinematic data corroborate a finished phase of destabilization. This latter condition may also pertain to relict RGUs. For both ongoing and completed destabilization phases, it is necessary that the data utilized for evaluation are dated. |
| Delineation type | Restricted, Extended, Restricted and extended, Other, n.a. | Explains the process used to define and establish the boundaries of the RGU. When the delineation has been carried out, it is necessary to specify the kind and date of the source data as well. The values "restricted and extended" indicate that both outlines are accessible. The term "other" indicates that none of the rules for extended and restricted outlining have been adhered to strictly. |
| Reliability | Low, Medium, High | Indicates the degree of uncertainty associated with the delineation of the outline of a rock glacier. |

Table S2. Main attributes of a Rock Glacier System (RGS) according to a RoGI.

| Attribute | Values / Units | Description |
| --- | --- | --- |
| Primary (mandatory) |  |  |
| Primary ID | RGS + 15 characters | Unique identifier for the RGS associated by its PM (Figure S2). |
| Associated RGU(s) | RGU + 15 characters | Indicates the associated RGU which can be multiple. |
| Metadata |  | Source data, geographic and map coordinates, date of mapping, mapper’s name, reviewer’s name, etc. |
| Optional |  |  |
| Rock Glacier Composition | Mono-unit, Multi-unit, n.a. | Describes the components of the RGS. |
| Delineation | Yes, Partial, n.a. | Describes the completion status of the delineation process for the RGS. The term "Yes" indicates that all constituent parts are well defined, whereas "Partial" indicates that only a portion of them are detailed. |
| Delineation Type | Restricted, Extended, Other, Various, n.a. | Explains the process used to define and establish the boundaries of the RGS. When the delineation has been carried out, it is necessary to specify the kind and date of the source data as well. The values "extended and restricted" indicate that both outlines are accessible. The term "other" indicates that none of the rules for extended and restricted outlining have been adhered to strictly. The term "various" indicates that different rules have been used based on the constituent units, specifically limited to RGS. |

S.3	Outline uncertainty

Based on geomorphological evidence and imagery resolution, a rock glacier outline is assigned a categorical level of “reliability”. Accordingly, the reliability of an outline that can be consistently drawn within an accuracy of 20 meters is considered as “high”; a counterpart consistently associated with greater uncertainty is assigned “low” reliability; and finally, an outline displaying mixed conditions of uncertainty across different portions is labelled as “medium” (Rouyet et al., 2025).

This uncertainty may stem from a lack of clear geomorphological evidence (not related to the complexity or size of the rock glacier) or from the poor quality of available imagery (for instance, due to snow, clouds, shadows, or inadequate georeferencing). The specific causes of uncertainty must be identified (such as rock glacier morphology and/or image quality). Furthermore, the date (dd/mm/yyyy) of the data acquisition for the delineation of the rock glacier should be noted, especially the images used to define the front of active rock glaciers.

If a rock glacier unit is identifiable and locatable, but significant portions of its front, lateral margins, or upslope connections cannot be reliably outlined (meaning the uncertainty in delineation changes the rock glacier area by more than roughly 10%), then no outline should be drawn. In such instances, the PM remains the sole georeferenced information linked to the landform.