Evaluation of occupational exposure to airborne quartz in the cutting and grinding of ceramic tiles

Francesca Borghi1*+, Francesca Graziosi1*+, Silvia Contessi1*; David C. Christiani2, Francesco Decataldo1, Deborah Glass3, Francesco S. Violante1,4

1Occupational Medicine Unit, Department of Medical and Surgical Sciences, Alma Mater Studiorum University of Bologna, Bologna (Italy)

2Harvard Medical School and Harvard TH Chan School of Public Health - Boston (Massashusetts, USA)

3Monash School of Public Health and Preventive Medicine - Melbourne (Australia)

4Division of Occupational Medicine, IRCCS Azienda Ospedaliero-Universitaria di Bologna, Bologna (Italy)

* These authors contributed equally to this work

+ Corresponding authors: FG (francesca.graziosi@unibo.it)

Table S1. Gravimetric analyses performed pre- and post-sampling. Each individual sample was collected while processing a single tile type and thickness.

| Dust in the Respirable fraction (mg/membrane) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N* | Min. | 5th per. | Mean | Median | 95th per. | Max. | SD |
|  |  | Total by material+ |  |  |  |  |  |  |  |  |
| Personal Samples |  | W12 mm + W20 mm+ | 7 | 0.069 | 0.103 | 0.273 | 0.244 | 0.425 | 0.444 | 0.122 |
|  |  | B12 mm + B20 mm+ | 7 | 0.272 | 0.281 | 0.876 | 0.731 | 1.640 | 1.715 | 0.567 |
|  |  | Detailed Material |  |  |  |  |  |  |  |  |
|  |  | W12 mm | 4 | 0.183 | 0.188 | 0.254 | 0.230 | 0.354 | 0.373 | 0.072 |
|  |  | W20 mm | 3 | 0.069 | 0.100 | 0.298 | 0.380 | 0.438 | 0.444 | 0.164 |
|  |  | B12 mm | 3 | 0.302 | 0305 | 0.700 | 0.334 | 1.352 | 1.465 | 0.541 |
|  |  | B20 mm | 4 | 0.272 | 0.341 | 1.007 | 1.021 | 1.654 | 1.715 | 0.550 |
|  |  | Activity |  |  |  |  |  |  |  |  |
|  |  | Cutting | 7 | 0.215 | 0.262 | 0.843 | 0.444 | 1.640 | 1.715 | 0.580 |
|  |  | Grinding | 7 | 0.069 | 0.103 | 0.305 | 0.272 | 0.612 | 0.731 | 0.192 |
|  | Total |  | 14 | 0.069 | 0.143 | 0.574 | 0.354 | 1.553 | 1.715 | 0.509 |
|  |  | Total by material+ |  |  |  |  |  |  |  |  |
| Environmental Samples |  | W12 mm + W20 mm+ | 13 | 0.034 | 0.036 | 0.102 | 0.067 | 0.256 | 0.450 | 0.104 |
|  |  | B12 mm + B20 mm+ | 16 | 0.046 | 0.050 | 0.095 | 0.092 | 0.160 | 0.164 | 0.036 |
|  |  | Detailed Material |  |  |  |  |  |  |  |  |
|  |  | W12 mm | 7 | 0.034 | 0.042 | 0.083 | 0.087 | 0.121 | 0.126 | 0.029 |
|  |  | W20 mm | 6 | 0.038 | 0.040 | 0.124 | 0.058 | 0.362 | 0.450 | 0.147 |
|  |  | B12 mm | 8 | 0.046 | 0.048 | 0.076 | 0.072 | 0.112 | 0.119 | 0.024 |
|  |  | B20 mm | 8 | 0.069 | 0.070 | 0.115 | 0.109 | 0.162 | 0.164 | 0.036 |
|  | Total |  | 29 | 0.034 | 0.040 | 0.098 | 0.087 | 0.162 | 0.450 | 0.075 |

+The rows in italics indicate the total number of samples collected for each material (Black: B and White: W), combining both 12 mm and 20 mm thicknesses. No combined processing of different thicknesses or materials was performed during a single sampling session.

* Valid N

Table S2. (a) Time-weighted exposure calculation, for cutting and grinding tasks, considering a APFs (Assigned Protection Factors) equal to 10.  (b) Time-weighted exposure calculation, for cutting and grinding tasks, considering a APFs (Assigned Protection Factors) equal to 20.

| (a) APF: 10 |  | CUTTING |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minutes/Day | 20 | 30 | 40 | 60 | 90 | 120 | 180 | 240 | 300 | 360 | 420 | 480 |
|  | Mean values [µg/m3] | 1.4 | 2.1 | 2.8 | 4.2 | 6.2 | 8.3 | 12.5 | 16,7 | 20.8 | 25.0 | 29.1 | 33.3 |
|  | Median values [µg/m3] | 0.7 | 1.0 | 1.4 | 2.1 | 3.1 | 4.1 | 6.2 | 8.2 | 10.3 | 12.3 | 14.4 | 16.4 |
|  | Max. values [µg/m3] | 3.7 | 5.6 | 7.4 | 11.1 | 16.7 | 22.2 | 33.3 | 44.5 | 55.6 | 66.7 | 77.8 | 88.9 |
|  |  | GRINDING |  |  |  |  |  |  |  |  |  |  |  |
|  | Minutes/Day | 20 | 30 | 40 | 60 | 90 | 120 | 180 | 240 | 300 | 360 | 420 | 480 |
|  | Mean values [µg/m3] | 0.6 | 0.9 | 1.2 | 1.9 | 2.8 | 3.7 | 5.6 | 7.4 | 9.3 | 11.2 | 13.0 | 14.9 |
|  | Median values [µg/m3] | 0.5 | 0.8 | 1.1 | 1.6 | 2.4 | 3.2 | 4.8 | 6.4 | 8.0 | 9.6 | 11.2 | 12.8 |
|  | Max. values [µg/m3] | 1.7 | 2.6 | 3.5 | 5.2 | 7.8 | 10.4 | 15.6 | 20.8 | 26.0 | 31.2 | 36.4 | 41.6 |
| (b) APF: 20 |  | CUTTING |  |  |  |  |  |  |  |  |  |  |  |
|  | Minutes/Day | 20 | 30 | 40 | 60 | 90 | 120 | 180 | 240 | 300 | 360 | 420 | 480 |
|  | Mean values [µg/m3] | 0.7 | 1.0 | 1.4 | 2.1 | 3.1 | 4.2 | 6.2 | 8.3 | 10.4 | 12.5 | 14.6 | 16.7 |
|  | Median values [µg/m3] | 0.3 | 0.5 | 0.7 | 1.0 | 1.5 | 2.1 | 3.1 | 4.1 | 5.1 | 6.2 | 7.2 | 8.2 |
|  | Max. values [µg/m3] | 1.9 | 2.8 | 3.7 | 5.6 | 8.3 | 11.1 | 16.7 | 22.2 | 27.8 | 33.3 | 38.9 | 44.5 |
|  |  | GRINDING |  |  |  |  |  |  |  |  |  |  |  |
|  | Minutes/Day | 20 | 30 | 40 | 60 | 90 | 120 | 180 | 240 | 300 | 360 | 420 | 480 |
|  | Mean values [µg/m3] | 0.3 | 0.5 | 0.6 | 0.9 | 1.4 | 1.9 | 2.8 | 3.7 | 4.6 | 5.6 | 6.5 | 7.4 |
|  | Median values [µg/m3] | 0.3 | 0.4 | 0.5 | 0.8 | 1.2 | 1.6 | 2.4 | 3.2 | 4.0 | 4.8 | 5.6 | 6.4 |
|  | Max. values [µg/m3] | 0.9 | 1.3 | 1.7 | 2.6 | 3.9 | 5.2 | 7.8 | 10.4 | 13.0 | 15.6 | 18.2 | 20.8 |

Key. Green: 0-25 µg/m3 (< TLV ACGIH Exposure Limit); Yellow: 25-50 µg/m3 (< OSHA Permissible Exposure Limit); Orange: 50-100 µg/m3 (<: European Exposure Limit).

Table S3. Maximum minutes of tasking per day (cutting/ grinding) not to exceed the limit of occupational exposure considering a APFs (Assigned Protection Factors) equal to 10 (Table S3(a)) and 20 (Table S3(b)).

| (a) APF=10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Exposure Limit |  | CUTTING (min) |  |  | GRINDING (min) |  |  |
| Source | µg /m3 | Mean | Median | Maximum | Mean | Median | Maximum |
| ACGIH TLV | 25 | 360 | >480 | 135 | >480 | >480 | 288 |
| OSHA PEL | 50 | >480 | >480 | 270 | >480 | >480 | >480 |
| European OEL | 100 | >480 | >480 | >480 | >480 | >480 | >480 |

| (b) APF=20 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Exposure Limit |  | CUTTING (min) |  |  | GRINDING (min) |  |  |
| Source | µg /m3 | Mean | Median | Maximum | Mean | Median | Maximum |
| ACGIH TLV | 25 | >480 | >480 | 270 | >480 | >480 | >480 |
| OSHA PEL | 50 | >480 | >480 | >480 | >480 | >480 | >480 |
| European OEL | 100 | >480 | >480 | >480 | >480 | >480 | >480 |