Code availability

The Matlab™ script used to develop the hydrogeological model shown in Figure 10 is provided below:

%%

% Saltwater-Freshwater Interface in Confined aquifer

%

% from Oude Essink (2001)

%

%

%

%

%

%

%

% We want to estimate the length of the salt wedge in meters

rhof = 1000; % freshwater density (kg/m3)

rhos = 1035; % saltwater density (kg/m3)

alpha = (rhos-rhof) / rhof; % BGH coefficient (-)

%%

% Variable parameters - Can be modified to fit data

D_B = 500; % thickness aquifer B in section B (m)

D_C = 500; % thickness aquifer A in section C (m)

SGR_B = 0.6; % Sand/shale ratio in aquifer section B

SGR_C = 0.6; % Sand/shale ratio in aquifer section C

k_shale = 0.000001; % hydraulic conductivity shale (m/d)

k_sand_B = 0.1; % hydraulic conductivity sand (m/d)

k_sand_C = 0.1; % hydraulic conductivity sand (m/d)

%%

% The permeability of the reservoir in Adriatic range from 2 to 20 millidarcies,

% which is equivalent to 0.002 t0 0.2 m/d (Pitone et al. 20121)

%

% Well tests in surface aquifers give 0.8 to 8.0 m/day

k_B = SGR_B * k_sand_B + (1-SGR_B) * k_shale; % hydraulic conductivity section B (m/day)

k_C = SGR_C * k_sand_C + (1-SGR_C) * k_shale; % hydraulic conductivity lsection C (m/day)

%%

% Aquifer flow per unit width in mm/ year and m/day due to recharge

q_B = 250 % mm/year

q_C = 250 % mm/year

q_B = ((q_B/1000) / 365) % m/day

q_C = ((q_C/1000) / 365) % m/day

%%

% Estimate depth of the interface H from bottom confining layer (m)

x = 0:100:70000;

H_B = -(((2*q_B) / (k_B *alpha))*x).^(1/2);

H_C = -(((2*q_C) / (k_C *alpha))*x).^(1/2);

%%

% Compute Distance of the wedge from tip point

L_B = (k_B * (D_B^2) * alpha) / (2 * q_B) % length saltwater wedge aquifer in section B (m)

L_C = (k_C * (D_C^2) * alpha) / (2 * q_C) % length saltwater wedge aquifer in section C (m)

%%

% Plot results

x = x* (-1);

plot(x-32000, H_B,'LineWidth', 3)

xlabel('Distance from offshore sand tippoint');

ylabel('Interface depth from bottom of confining layer (m)');

title('Depth of Saltwater-Freshwater Interface');

hold on;

plot(x-25000, H_C,'LineWidth', 3)

legend('SW-FW Interface Aquifer sez B', 'SW-FW Interface Aquifer sez C')

hold off

%%

%