Effects of the emissions of vehicles ahead on in-car exposure to traffic-related air pollutants: a Multiple Statistical Analysis approach.

Davide Campagnoloa,*, Andrea Cattaneoa, Simona Iodiceb, Chiara Faverob, Simone Lioia, Luca Boniardib,c, Francesca Borghia,d, Giacomo Fantia, Marta Kellera, Sabrina Rovellia, Carolina Zellinoa, Giovanni De Vitoe, Andrea Spinazzèa, Silvia Fustinonib,c, Valentina Bollatib,f, Domenico M. Cavalloa

a Department of Science and High Technology, University of Insubria, Como, Italy

b EPIGET Epidemiology, Epigenetics and Toxicology Lab, Department of Clinical Sciences and Community Health, University of Milan, Milan, Italy

c Environmental and Industrial Toxicology Unit, Fondazione IRCCS Cà Granda, Ospedale Maggiore Policlinico, Milan, Italy

d Department of Medical and Surgical Sciences, University of Bologna, Bologna, Italy

e Department of Medicine and Surgery, University of Insubria, Varese, Italy

f Epidemiology Unit, Fondazione IRCCS Cà Granda Ospedale Maggiore Policlinico, Milan, Italy

*Corresponding author:

Davide Campagnolo

via Valleggio, 11 – 22100 Como – Italy

Tel.: +39 031 2386621; Fax: +39 031 2386630

E-mail address: davide.campagnolo@uninsubria.it

SUPPLEMENTARY MATERIALS

Figure S1-The selected route from Cuveglio (a town in Varese Province) to Como. The trip was the same both for outwards and return-trips. The different colors represented the four types of roads and, at the same time, the four sections into which the route was divided. The selected monitoring and air quality fixed monitoring stations were also reported.

Text S1 – The monitoring instruments.

CO concentrations [ppm] were collected with a passive T15v Langan electrochemical sensor (Langan Products Inc., San Francisco, CA, USA). The numeric concentrations [particle/cm3] of ambient air particles > 0.3 μm (PM0.3-1 and PM1-2.5) were monitored using an optical particle counter (OPC) (model Handheld 3016 IAQ, Lighthouse Worldwide Solutions, Fremont, CA, USA) with a counting efficiency of 50% at 0.3 μm and 100% for particles > 0.45 μm. This instrument allows the separation of particles as a function of their size by detecting the light scattered by individual particles. Furthermore, the UFPs numeric concentrations [particle/cm3] were detected using a miniature diffusion size classifier (DiSCmini, Matter Aerosol AG, Wohlen AG, Swiss). It is a compact, battery-powered handheld instrument that measures the number of the particles in the size range of approximately 10 < Dp < 700 nm (an impactor is used to remove particles larger than 700 nm). This device is based on unipolar charging of the aerosol, which is followed by detection in two electrometer stages (Fierz et al., 2007; Fierz et al., 2011). The range of particles that could be detected by DiSCmini is 103-106 particle/cm3; more details regarding the instrument sensitivity are reported in a dedicated publication (Fierz et al., 2011). Concerning data quality, the study performed by Spinazzè and co-workers (2017) highlighted a good agreement between DiSCmini and a condensation particle counter, however this accounted for an over-estimation effect of UFPs by condensation particle counter, contrary to some previous studies (Fierz et al., 2011; Meier et al., 2013; Spinazzè et al., 2015). It is known that DiSCmini is less accurate than traditional aerosol instruments, but its smaller size makes it easier to handle and suitable for in-cabin studies (Fierz et al., 2011). The eBC concentrations [μg/m3] were monitored by a micro-aethalometer (model AE51, AethLabs, San Francisco, CA, USA). To estimate the eBC concentrations, this optical device leads the sampled air towards a filter and it measures the rate of attenuation of a light beam (at a wavelength equal to 880 nm) between before and after the filter. The measurement resolution and precision are 0.001 μg/m3 and ±0.1 μg/m3 (1 min average and 150 mL/min flow rate), respectively. The measurements were performed with a flow of 100 mL/min. To be up against the loading effect-related bias, the instrument filter was changed every day (Weingartner et al., 2003). Since the concentrations are derived by applying an assumed mass absorption cross-section (Cheng and Lin, 2013), the black carbon estimated by the micro-aethalometer is defined as eBC (i.e., equivalent black carbon) (Petzold et al., 2013).

References of Text S1

Fierz, M.; Burtscher, H.; Steigmeier, P.; Kasper, M. Field Measurement of Particle Size and Number Concentration with the Diffusion Size Classifier (Disc); 2008; pp 2008-01–1179. https://doi.org/10.4271/2008-01-1179.

Fierz, M.; Houle, C.; Steigmeier, P.; Burtscher, H. Design, Calibration, and Field Performance of a Miniature Diffusion Size Classifier. Aerosol Sci. Technol. 2011, 45 (1), 1–10. https://doi.org/10.1080/02786826.2010.516283.

Spinazzè, A.; Fanti, G.; Borghi, F.; Del Buono, L.; Campagnolo, D.; Rovelli, S.; Cattaneo, A.; Cavallo, D. M. Field Comparison of Instruments for Exposure Assessment of Airborne Ultrafine Particles and Particulate Matter. Atmos. Environ. 2017, 154, 274–284. https://doi.org/10.1016/j.atmosenv.2017.01.054.

Meier, R.; Clark, K.; Riediker, M. Comparative Testing of a Miniature Diffusion Size Classifier to Assess Airborne Ultrafine Particles Under Field Conditions. Aerosol Sci. Technol. 2013, 47 (1), 22–28. https://doi.org/10.1080/02786826.2012.720397.

Weingartner, E.; Saathoff, H.; Schnaiter, M.; Streit, N.; Bitnar, B.; Baltensperger, U. Absorption of Light by Soot Particles: Determination of the Absorption Coefficient by Means of Aethalometers. J. Aerosol Sci. 2003, 34 (10), 1445–1463. https://doi.org/10.1016/S0021-8502(03)00359-8.

Cheng, Y.-H.; Lin, M.-H. Real-Time Performance of the microAeth® AE51 and the Effects of Aerosol Loading on Its Measurement Results at a Traffic Site. Aerosol Air Qual. Res. 2013, 13 (6), 1853–1863. https://doi.org/10.4209/aaqr.2012.12.0371.

Petzold, A.; Ogren, J. A.; Fiebig, M.; Laj, P.; Li, S.-M.; Baltensperger, U.; Holzer-Popp, T.; Kinne, S.; Pappalardo, G.; Sugimoto, N.; Wehrli, C.; Wiedensohler, A.; Zhang, X.-Y. Recommendations for Reporting &quot;Black Carbon&quot; Measurements. Atmospheric Chem. Phys. 2013, 13 (16), 8365–8379. https://doi.org/10.5194/acp-13-8365-2013.

Text S2 - Differences between individual exposure and personal exposure.

The individual exposure measurement strategy allowed the measurement of fine PM and UFPs concentrations without important accuracy losses with respect to personal (breathing zone) measurements. Moreover, although the correlation between individual and personal CO exposure was lower than that for fine PM, individual exposure to CO can be considered the best proxy for personal exposures.

Adapted from: Cattaneo, A.; Taronna, M.; Garramone, G.; Peruzzo, C.; Schlitt, C.; Consonni, D.; Cavallo, D. M. Comparison between Personal and Individual Exposure to Urban Air Pollutants. Aerosol Sci. Technol. 2010, 44 (5), 370–379.

Text S3 - Procedures to obtain and process data concerning the leading vehicles.

The leading vehicle characteristics were retrieved using their license plate numbers that were matched against the Italian vehicle registration database (available on the Italian Ministry of Infrastructure and Transport website:

https://www.ilportaledellautomobilista.it/web/portale-automobilista/verifica-classe-ambientale-veicolo). This public database provides, among others, information concerning the emission standard level and the fuel type of each vehicle. […] Moreover, to account for possible time delays from leading vehicle emissions to in-cabin TRAPs diffusion and for the possible time mismatch between the measured concentrations and vehicle type attribution, only those cases in which the study vehicle followed the leading vehicle for a minimum residence time (which was fixed at 30s) were used in the analysis. Finally, possible episodes in which other vehicles could have directly caused a mix of different fuel emission types (e.g., overtaking) were excluded from the treatment.

Adapted from: Campagnolo, D.; Cattaneo, A.; Corbella, L.; Borghi, F.; Del Buono, L.; Rovelli, S.; Spinazzé, A.; Cavallo, D. M. In-Vehicle Airborne Fine and Ultra-Fine Particulate Matter Exposure: The Impact of Leading Vehicle Emissions. Environ. Int. 2019, 123, 407–416.

Table S1 Descriptive statistics for CO, eBC, fine PM fractions and UFPs concentrations monitored in the study vehicle during the summer campaign.

| In-cabin TRAPs | n | Min. | 25th perc. | Mean | SD | Median | 75th perc. | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO [ppm] | 4565 | 0.20 | 0.82 | 1.2 | 0.51 | 1.2 | 1.5 | 3.9 |
| eBC [µg/m3] | 4565 | 0.2 | 2.6 | 6.2 | 6.6 | 4.5 | 7.4 | 127 |
| PM0.3-1 [particle/cm3] | 4565 | 6.1 | 29 | 46 | 23 | 44 | 58 | 170 |
| PM1-2.5 [particle/cm3] | 4565 | 0.01 | 0.20 | 0.33 | 0.15 | 0.31 | 0.44 | 1.3 |
| UFPs [particle/cm3] | 4565 | 1.6x103 | 8.8x103 | 2.6x104 | 6.8x104 | 1.3x104 | 2.0x104 | 1.0x106 |

### Min= Minimum; 25th perc.= 25th percentile; 75th perc.= 75th percentile; Max= Maximum; SD= Standard deviation.

Table S2 Descriptive statistics for CO, eBC, fine PM fractions and UFPs concentrations monitored in the study vehicle during the autumn campaign.

| In-cabin TRAPs | n | Min. | 25th perc. | Mean | SD | Median | 75th perc. | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO [ppm] | 5465 | 0.71 | 1.4 | 1.8 | 0.65 | 1.7 | 2.1 | 9.1 |
| eBC [µg/m3] | 5465 | 0.3 | 3.0 | 8.2 | 9.8 | 5.2 | 9.1 | 145 |
| PM0.3-1 [particle/cm3] | 5465 | 1.8 | 17 | 32 | 19 | 29 | 45 | 176 |
| PM1-2.5 [particle/cm3] | 5465 | 0.02 | 0.13 | 0.23 | 0.14 | 0.20 | 0.30 | 1.5 |
| UFPs [particle/cm3] | 5465 | 365 | 8.4x103 | 2.2x104 | 2.6x104 | 1.4x104 | 2.5x104 | 3.1x105 |

Min= Minimum; 25th perc.= 25th percentile; 75th perc.= 75th percentile; Max= Maximum; SD= Standard deviation.

Table S3 Descriptive statistics for in-cabin T and RH, speed of the study vehicle, ambient pollutants concentrations and meteorological variables – Both campaigns. Wind direction data are not reported.

| Variables | n | Min | 25th perc. | Mean | SD | Median | 75th perc. | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T (in-cabin) [°C] | 10030 | 17 | 23 | 26 | 4.4 | 25 | 29 | 35 |
| RH (in-cabin) [%] | 10030 | 27 | 42 | 50 | 9.9 | 51 | 58 | 75 |
| Speed [Km/h] | 9928 | 0.18 | 22 | 38 | 20 | 41 | 54 | 88 |
| PM2.5 (Fixed stations) [µg/m3] | 10030 | 2 | 11 | 15 | 4.8 | 14 | 17 | 25 |
| PM10 (Fixed stations) [µg/m3] | 10030 | 6 | 13 | 19 | 5.9 | 20 | 24 | 28 |
| CO (Fixed stations) [ppm] | 10030 | 0.16 | 0.24 | 0.42 | 0.17 | 0.40 | 0.56 | 0.80 |
| NO2 (Fixed stations) [ppb] | 9150 | 9.8 | 15 | 22 | 7.5 | 22 | 26 | 42 |
| T (Ambient) [°C] | 10030 | 11 | 16 | 20 | 5.2 | 20 | 24 | 31 |
| RH (Ambient) [%] | 10030 | 24 | 47 | 63 | 18 | 65 | 76 | 99 |
| Wind velocity [m/s] | 10030 | 0.07 | 1.1 | 1.9 | 1.2 | 1.6 | 2.8 | 6 |

### Min= Minimum; 25th perc.= 25th percentile; 75th perc.= 75th percentile; Max= Maximum; SD= Standard deviation.

Table S4 Descriptive statistics for in-cabin T and RH, speed of the study vehicle, ambient pollutants concentrations and meteorological variables – Summer campaign. Wind direction data are not reported.

| Numerical variables | n | Min | 25th perc. | Mean | SD | Median | 75th perc. | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T (in-cabin) [°C] | 4565 | 23 | 27 | 29 | 3.1 | 29 | 32 | 35 |
| RH (in-cabin) [%] | 4565 | 34 | 41 | 47 | 7.1 | 48 | 53 | 61 |
| Speed [Km/h] | 4565 | 0 | 27 | 41 | 20 | 44 | 56 | 88 |
| PM2.5 (Fixed stations) [µg/m3] | 4565 | 13 | 14 | 17 | 3.7 | 16 | 18 | 25 |
| PM10 (Fixed stations) [µg/m3] | 4565 | 16 | 18 | 22 | 4.0 | 21 | 25 | 28 |
| CO (Fixed stations) [ppm] | 4565 | 0.24 | 0.24 | 0.41 | 0.14 | 0.40 | 0.56 | 0.64 |
| NO2 (Fixed stations) [ppb] | 4565 | 11 | 15 | 21 | 6.4 | 21 | 24 | 33 |
| T (Ambient) [°C] | 4565 | 20 | 23 | 25 | 2.6 | 24 | 26 | 31 |
| RH (Ambient) [%] | 4565 | 32 | 44 | 53 | 12 | 52 | 61 | 97 |
| Wind velocity [m/s] | 4565 | 0.77 | 1.2 | 2.2 | 1.1 | 1.9 | 3.0 | 5.3 |

### Min= Minimum; 25th perc.= 25th percentile; 75th perc.= 75th percentile; Max= Maximum; SD= Standard deviation.

Table S5 Descriptive statistics for in-cabin T and RH, speed of the study vehicle, ambient pollutants concentrations and meteorological variables – Autumn campaign. Wind direction data are not reported.

| Numerical variables | n | Min | 25th perc. | Mean | SD | Median | 75th perc. | Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T (in-cabin) [°C] | 5465 | 17 | 21 | 23 | 2.7 | 23 | 24 | 27 |
| RH (in-cabin) [%] | 5465 | 27 | 44 | 53 | 11 | 54 | 61 | 75 |
| Speed [Km/h] | 5363 | 0 | 18 | 36 | 21 | 38 | 52 | 84 |
| PM2.5 (Fixed stations) [µg/m3] | 5465 | 2.0 | 10 | 12 | 4.5 | 13 | 16 | 19 |
| PM10 (Fixed stations) [µg/m3] | 5465 | 6.0 | 12 | 16 | 5.9 | 18 | 20 | 24 |
| CO (Fixed stations) [ppm] | 5465 | 0.16 | 0.24 | 0.43 | 0.19 | 0.40 | 0.56 | 0.80 |
| NO2 (Fixed stations) [ppb] | 4585 | 9.8 | 19 | 23 | 8.3 | 23 | 26 | 42 |
| T (Ambient) [°C] | 5465 | 11 | 13 | 16 | 3.0 | 16 | 19 | 22 |
| RH (Ambient) [%] | 5465 | 24 | 67 | 71 | 18 | 74 | 78 | 99 |
| Wind velocity [m/s] | 5465 | 0.07 | 0.90 | 1.8 | 1.3 | 1.5 | 2.1 | 6.0 |

### Min= Minimum; 25th perc.= 25th percentile; 75th perc.= 75th percentile; Max= Maximum; SD= Standard deviation.

Table S6 Number of leading vehicles encountered per each categorical variable (Total frequency= 182).

| Variable | Category | n (%) |
| --- | --- | --- |
| Road Types | E.U. cities connection | 52 (28.6) |
|  | E.U. by-pass | 34 (18.7) |
|  | Rural | 31 (17.0) |
|  | Urban | 65 (35.7) |
| Traffic Jam | No | 133 (73.1) |
|  | Yes | 49 (26.9) |
| Stop at traffic light | No | 152 (83.5) |
|  | Yes | 30 (16.5) |
| Leading vehicle Types | Heavy-duty | 11 (6.1) |
|  | Light-duty | 158 (86.8) |
|  | No data | 13 (7.1) |
| Leading vehicle Emission Standard Levels | Euro 0 | 1 (0.6) |
|  | Euro 1 | 2 (1.1) |
|  | Euro 2 | 12 (6.6) |
|  | Euro 3 | 21 (11.5) |
|  | Euro 4 | 38 (20.9) |
|  | Euro 5 | 40 (22.0) |
|  | Euro 6 | 49 (26.9) |
|  | No data | 19 (10.4) |
| Leading vehicle fuel types | Diesel | 65 (35.7) |
|  | Hybrid | 10 (5.5) |
|  | No data | 24 (13.2) |
|  | Petrol | 77 (42.3) |
|  | Petrol&CNG | 6 (3.3) |

Table S7 Characteristics of each roundtrip in terms of number of observations, duration of the trip and number of leading vehicles.

| Monitoring campaign | Day | Roundtrip | n (observations) | Trip duration (min) | n of leading vehicles |
| --- | --- | --- | --- | --- | --- |
| Summer | 1 | Outward | 455 | 75.8 | 11 |
|  |  | Return-trip | 511 | 85.0 | 11 |
|  | 2 | Outward* | 188 | 31.2 | 3 |
|  |  | Return-trip | 521 | 86.7 | 8 |
|  | 3 | Outward | 445 | 74.0 | 7 |
|  |  | Return-trip | 468 | 77.8 | 10 |
|  | 4 | Outward | 484 | 80.5 | 11 |
|  |  | Return-trip | 485 | 80.7 | 8 |
|  | 5 | Outward | 482 | 80.2 | 11 |
|  |  | Return-trip | 526 | 87.5 | 10 |
| Autumn | 1 | Outward | 465 | 77.3 | 6 |
|  |  | Return-trip | 528 | 88.0 | 10 |
|  | 2 | Outward | 588 | 97.8 | 8 |
|  |  | Return-trip | 592 | 98.5 | 9 |
|  | 3 | Outward | 530 | 88.2 | 9 |
|  |  | Return-trip | 606 | 100.8 | 9 |
|  | 4 | Outward | 478 | 79.5 | 10 |
|  |  | Return-trip | 613 | 102.0 | 13 |
|  | 5 | Outward | 464 | 77.2 | 9 |
|  |  | Return-trip | 601 | 100.0 | 9 |

* Instrumentation problems occurred.

Table S8 Summary of the results of the multiple regression using GEE models relating in-cabin CO measurement data. The ∆% of the statistically significant results are reported in bold.

| Variable | Category | ∆% | 95% CI | p-value |
| --- | --- | --- | --- | --- |
| Campaign | Autumn | 130.6 | (87.4; 183.7) | <0.0001 |
|  | Summer | Ref |  |  |
| Leading vehicle | Yes (Presence) | 20.1 | (9.0; 32.3) | 0.0002 |
|  | No (Absence) | Ref |  |  |
| Leading vehicle Emission Standard Level | Euro 4 | -9.8 | (-19.4; 1.0) | 0.074 |
|  | Euro 5 | -20.5 | (-27.2; -13.2) | <0.0001 |
|  | Euro 6 | -17.4 | (-23.4; -11.0) | <0.0001 |
|  | Euro 0123 | Ref |  |  |
| Leading vehicle fuel type | Diesel | -7.2 | (-12.2; -1.9) | 0.009 |
|  | Petrol&CNG +Hybrid | 0 | (-11.1; 12.6) | 0.995 |
|  | Petrol | Ref |  |  |
| Traffic Jam | Presence | 5.8 | (2.2; 9.5) | 0.001 |
|  | Absence | Ref |  |  |
| Road Types | E.U. by-pass | -11.8 | (-17.9; -5.3) | 0.001 |
|  | Rural | -26.2 | (-34.6; -16.6) | <0.0001 |
|  | Urban | 13.9 | (4.9; 23.6) | 0.002 |
|  | E.U. cities connection | Ref |  |  |
| Traffic Light Stops | Presence | 6.7 | (2.3; 11.3) | 0.003 |
|  | Absence | Ref |  |  |
| T (in-vehicle) |  | 4.9 | (3.1; 6.8) | <0.0001 |

Table S9 In-cabin CO: complete results of the univariate linear regression using GEE models.

| TRAP | Variable | Category | β | SE | 95% CI |  | p-value | Delta % | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Log CO | Campaign | 0 - summer | Ref |  |  |  |  |  |  |  |
|  | Campaign | 1 - autumn | 0.525 | 0.101 | 0.327 | 0.723 | <0.0001 | 69.1 | 38.7 | 106.1 |
|  | Day | 1 | Ref |  |  |  |  |  |  |  |
|  | Day | 2 | -0.096 | 0.264 | -0.614 | 0.421 | 0.716 | -9.2 | -45.9 | 52.4 |
|  | Day | 3 | -0.120 | 0.264 | -0.638 | 0.397 | 0.648 | -11.3 | -47.2 | 48.8 |
|  | Day | 4 | -0.058 | 0.133 | -0.317 | 0.202 | 0.664 | -5.6 | -27.2 | 22.4 |
|  | Day | 5 | -0.110 | 0.191 | -0.484 | 0.265 | 0.566 | -10.4 | -38.4 | 30.3 |
|  | Roundtrip | Outward | Ref |  |  |  |  |  |  |  |
|  | Roundtrip | Return-trip | 0.261 | 0.144 | -0.021 | 0.542 | 0.070 | 29.8 | -2.1 | 72.0 |
|  | Leading Vehicles | ABSENCE | Ref |  |  |  |  |  |  |  |
|  | Leading Vehicles | PRESENCE | 0.072 | 0.030 | 0.012 | 0.132 | 0.019 | 7.4 | 1.2 | 14.1 |
|  | Leading vehicle Types | ABSENCE | -0.062 | 0.027 | -0.115 | -0.009 | 0.023 | -6.0 | -10.8 | -0.9 |
|  | Leading vehicle types | HEAVY-DUTY | -0.035 | 0.054 | -0.141 | 0.072 | 0.526 | -3.4 | -13.2 | 7.5 |
|  | Leading vehicle types | LIGHT-DUTY | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | ABSENCE | -0.213 | 0.057 | -0.324 | -0.101 | <0.0001 | -19.2 | -27.7 | -9.6 |
|  | Leading vehicle E. S._Level | EURO 0123 | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | EURO 4 | -0.062 | 0.073 | -0.206 | 0.082 | 0.399 | -6.0 | -18.6 | 8.5 |
|  | Leading vehicle Em. Std._Level | EURO 5 | -0.241 | 0.064 | -0.367 | -0.116 | <0.0001 | -21.4 | -30.7 | -10.9 |
|  | Leading vehicle Em. Std._Level | EURO 6 | -0.176 | 0.061 | -0.295 | -0.056 | 0.004 | -16.1 | -25.6 | -5.5 |
|  | Leading vehicle fuel type | ABSENCE | -0.128 | 0.039 | -0.204 | -0.051 | 0.001 | -12.0 | -18.5 | -5.0 |
|  | Leading vehicle fuel type | DIESEL | -0.117 | 0.042 | -0.199 | -0.035 | 0.005 | -11.0 | -18.1 | -3.4 |
|  | Leading vehicle fuel type | PETROL | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle fuel type | P.&CNG+ HYBRID | -0.025 | 0.065 | -0.152 | 0.103 | 0.704 | -2.4 | -14.1 | 10.8 |
|  | Traffic Jam | NO | Ref |  |  |  |  |  |  |  |
|  | Traffic Jam | YES | 0.120 | 0.025 | 0.072 | 0.169 | <0.0001 | 12.8 | 7.4 | 18.4 |
|  | Traffic Light Stops | NO | Ref |  |  |  |  |  |  |  |
|  | Traffic Light Stops | YES | 0.160 | 0.023 | 0.115 | 0.206 | <0.0001 | 17.4 | 12.2 | 22.8 |
|  | Road Types | E.-U.Conn. | 0.353 | 0.075 | 0.206 | 0.500 | <0.0001 | 42.4 | 22.9 | 64.9 |
|  | Road Types | E.-U. by-pass | 0.190 | 0.056 | 0.079 | 0.300 | 0.001 | 20.9 | 8.3 | 35.0 |
|  | Road Types | Rural | Ref |  |  |  |  |  |  |  |
|  | Road Types | Urban | 0.470 | 0.089 | 0.295 | 0.645 | <0.0001 | 60.0 | 34.3 | 90.5 |
|  | T (in-cabin) |  | 0.046 | 0.023 | 0.001 | 0.091 | 0.043 | 4.7 | 0.1 | 9.5 |
|  | RH (in-cabin) |  | -0.009 | 0.009 | -0.026 | 0.008 | 0.304 | -0.9 | -2.6 | 0.8 |
|  | Speed |  | -0.003 | 0.001 | -0.004 | -0.001 | <0.0001 | -0.3 | -0.4 | -0.1 |
|  | PM2.5_FixedStations |  | 0.018 | 0.014 | -0.010 | 0.046 | 0.214 | 1.8 | -1.0 | 4.7 |
|  | PM10_FixedStations |  | 0.023 | 0.016 | -0.009 | 0.054 | 0.161 | 2.3 | -0.9 | 5.5 |
|  | CO_FixedStations |  | 0.917 | 0.233 | 0.461 | 1.374 | <0.0001 | 150.3 | 58.5 | 295.2 |
|  | NO2_FixedStations |  | 0.016 | 0.008 | 0.001 | 0.031 | 0.035 | 1.6 | 0.1 | 3.1 |
|  | Temperature Ambient |  | 0.073 | 0.015 | 0.043 | 0.102 | <0.0001 | 7.5 | 4.4 | 10.8 |
|  | Relative Humidity Ambient |  | 0.012 | 0.006 | -0.001 | 0.024 | 0.073 | 1.2 | -0.1 | 2.4 |
|  | Wind direction |  | -0.001 | 0.001 | -0.002 | 0.000 | 0.250 | -0.1 | -0.2 | 0.0 |
|  | Wind velocity |  | 0.041 | 0.054 | -0.066 | 0.148 | 0.453 | 4.2 | -6.4 | 15.9 |

Table S10 In-cabin CO: complete results of the multiple linear regression using GEE models.

| Variable | Category | β | SE | 95% CI |  | p-value | Delta% | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intercept |  | -1.327 | 0.303 | -1.921 | -0.732 | <0.0001 |  |  |  |
| Campaign | Autumn | 0.835 | 0.106 | 0.628 | 1.043 | <0.0001 | 130.6 | 87.4 | 183.7 |
| Campaign | Summer | Ref |  |  |  |  |  |  |  |
| Leading Vehicle | Presence | 0.183 | 0.050 | 0.086 | 0.280 | <0.0001 | 20.1 | 9.0 | 32.3 |
|  | Absence | Ref |  |  |  |  |  |  |  |
| Leading vehicle Emission Standard Level | EURO 4 | -0.103 | 0.058 | -0.216 | 0.010 | 0.074 | -9.8 | -19.4 | 1.0 |
|  | EURO 5 | -0.230 | 0.045 | -0.318 | -0.142 | <0.0001 | -20.5 | -27.2 | -13.2 |
|  | EURO 6 | -0.192 | 0.039 | -0.267 | -0.116 | <0.0001 | -17.4 | -23.4 | -11.0 |
|  | EURO 0123 | Ref |  |  |  |  |  |  |  |
| Leading vehicle fuel type | DIESEL | -0.075 | 0.028 | -0.130 | -0.019 | 0.009 | -7.2 | -12.2 | -1.9 |
|  | P.&CNG+ HYBRID | 0.000 | 0.060 | -0.118 | 0.118 | 0.995 | 0.0 | -11.1 | 12.6 |
|  | PETROL | Ref |  |  |  |  |  |  |  |
| Traffic Jam | Yes | 0.056 | 0.018 | 0.022 | 0.091 | 0.001 | 5.8 | 2.2 | 9.5 |
|  | No | Ref |  |  |  |  |  |  |  |
| Road Types | E.-U. by-pass | -0.126 | 0.037 | -0.198 | -0.054 | 0.001 | -11.8 | -17.9 | -5.3 |
|  | Rural | -0.303 | 0.062 | -0.425 | -0.182 | <0.0001 | -26.2 | -34.6 | -16.6 |
|  | Urban | 0.130 | 0.042 | 0.048 | 0.212 | 0.002 | 13.9 | 4.9 | 23.6 |
|  | E.-U. Conn. | Ref |  |  |  |  |  |  |  |
| Traffic Light Stops | Yes | 0.065 | 0.022 | 0.022 | 0.107 | 0.003 | 6.7 | 2.3 | 11.3 |
|  | No | Ref |  |  |  |  |  |  |  |
| T (in-vehicle) |  | 0.048 | 0.009 | 0.031 | 0.066 | <0.0001 | 4.9 | 3.1 | 6.8 |

Table S11 Summary of the results of the multiple regression using GEE models relating in-cabin eBC measurement data. The ∆% of the statistically significant results are reported in bold.

| Variable | Category | ∆% | 95% CI | p-value |
| --- | --- | --- | --- | --- |
| Leading Vehicle | Presence | 67.5 | (27.1; 120.8) | <0.0001 |
|  | Absence | Ref |  |  |
| Leading vehicles Emission Standard Level | Euro 4 | -32.9 | (-49.6; -10.8) | 0.006 |
|  | Euro 5 | -39.4 | (-54.4; -19.5) | 0.001 |
|  | Euro 6 | -46.7 | (-61.6; -26.0) | <0.0001 |
|  | Euro 0123 | Ref |  |  |
| Leading vehicles fuel type | Diesel | 45.3 | (20.0; 75.9) | <0.0001 |
|  | Petrol&CNG +Hybrid | -3.8 | (-21.3; 17.5) | 0.702 |
|  | Petrol | Ref |  |  |
| Road Types | E.U. by-pass | 36.4 | (6.7; 74.2) | 0.013 |
|  | Rural | -28.5 | (-43.2; -9.9) | 0.005 |
|  | Urban | 20.8 | (1.1; 44.3) | 0.038 |
|  | E.U. cities connection | Ref |  |  |

Table S12 In-cabin eBC: complete results of the univariate linear regression using GEE models.

| TRAP | Variable | Category | β | SE | 95% CI |  | p-value | Delta % | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Log eBC | Campaign | 0 - summer | Ref |  |  |  |  |  |  |  |
|  | Campaign | 1 - autumn | 0.212 | 0.108 | 0.000 | 0.423 | 0.050 | 23.6 | 0.0 | 52.7 |
|  | Day | 1 | Ref |  |  |  |  |  |  |  |
|  | Day | 2 | -0.009 | 0.237 | -0.473 | 0.455 | 0.968 | -0.9 | -37.7 | 57.6 |
|  | Day | 3 | -0.152 | 0.061 | -0.272 | -0.033 | 0.012 | -14.1 | -23.8 | -3.3 |
|  | Day | 4 | -0.380 | 0.064 | -0.505 | -0.255 | <0.0001 | -31.6 | -39.7 | -22.5 |
|  | Day | 5 | -0.045 | 0.071 | -0.184 | 0.094 | 0.528 | -4.4 | -16.8 | 9.9 |
|  | Roundtrip | Outward | Ref |  |  |  |  |  |  |  |
|  | Roundtrip | Return-trip | -0.081 | 0.117 | -0.309 | 0.148 | 0.489 | -7.7 | -26.6 | 15.9 |
|  | Leading Vehicle | ABSENCE | Ref |  |  |  |  |  |  |  |
|  | Leading Vehicle | PRESENCE | 0.414 | 0.073 | 0.271 | 0.556 | <0.0001 | 51.3 | 31.2 | 74.4 |
|  | Leading vehicle types | ABSENCE | -0.314 | 0.069 | -0.448 | -0.180 | <0.0001 | -26.9 | -36.1 | -16.4 |
|  | Leading vehicle types | HEAVY-DUTY | 0.616 | 0.299 | 0.030 | 1.203 | 0.039 | 85.2 | 3.0 | 232.9 |
|  | Leading vehicle types | LIGHT-DUTY | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | ABSENCE | -0.883 | 0.231 | -1.335 | -0.430 | <0.0001 | -58.6 | -73.7 | -34.9 |
|  | Leading vehicle Em. Std._Level | EURO 0123 | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | EURO 4 | -0.455 | 0.231 | -0.908 | -0.003 | 0.049 | -36.6 | -59.7 | -0.3 |
|  | Leading vehicle Em. Std._Level | EURO 5 | -0.502 | 0.224 | -0.941 | -0.063 | 0.025 | -39.5 | -61.0 | -6.1 |
|  | Leading vehicle Em. Std._Level | EURO 6 | -0.631 | 0.253 | -1.127 | -0.135 | 0.013 | -46.8 | -67.6 | -12.7 |
|  | Leading vehicle fuel type | ABSENCE | -0.225 | 0.072 | -0.367 | -0.083 | 0.002 | -20.1 | -30.7 | -7.9 |
|  | Leading vehicle fuel type | DIESEL | 0.390 | 0.113 | 0.169 | 0.612 | 0.001 | 47.7 | 18.4 | 84.3 |
|  | Leading vehicle fuel type | PETROL | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle fuel type | P.&CNG+ HYBRID | -0.013 | 0.109 | -0.226 | 0.200 | 0.904 | -1.3 | -20.3 | 22.2 |
|  | Traffic Jam | No | Ref |  |  |  |  |  |  |  |
|  | Traffic Jam | Yes | 0.265 | 0.057 | 0.153 | 0.377 | <0.0001 | 30.3 | 16.5 | 45.8 |
|  | Traffic Light Stops | No | Ref |  |  |  |  |  |  |  |
|  | Traffic Light Stops | Yes | -0.089 | 0.087 | -0.259 | 0.081 | 0.304 | -8.5 | -22.8 | 8.4 |
|  | Road Types | E.-U. conn. | 0.386 | 0.124 | 0.143 | 0.628 | 0.002 | 47.0 | 15.4 | 87.4 |
|  | Road Types | E.-U. by-pass | 0.788 | 0.138 | 0.517 | 1.059 | <0.0001 | 119.9 | 67.6 | 188.4 |
|  | Road Types | Rural | Ref |  |  |  |  |  |  |  |
|  | Road Types | Urban | 0.598 | 0.137 | 0.329 | 0.867 | <0.0001 | 81.8 | 38.9 | 137.9 |
|  | T (in-cabin) |  | -0.036 | 0.031 | -0.097 | 0.025 | 0.243 | -3.6 | -9.2 | 2.5 |
|  | RH (in-cabin) |  | 0.025 | 0.015 | -0.005 | 0.055 | 0.107 | 2.5 | -0.5 | 5.6 |
|  | Speed |  | -0.001 | 0.001 | -0.003 | 0.002 | 0.479 | -0.1 | -0.3 | 0.2 |
|  | PM2.5_FixedStations |  | 0.001 | 0.019 | -0.036 | 0.039 | 0.950 | 0.1 | -3.6 | 3.9 |
|  | PM10_FixedStations |  | -0.012 | 0.023 | -0.057 | 0.032 | 0.589 | -1.2 | -5.5 | 3.3 |
|  | CO_FixedStations |  | -0.088 | 0.579 | -1.224 | 1.048 | 0.879 | -8.4 | -70.6 | 185.1 |
|  | NO2_FixedStations |  | 0.023 | 0.009 | 0.006 | 0.040 | 0.008 | 2.3 | 0.6 | 4.1 |
|  | Temperature Ambient |  | 0.045 | 0.031 | -0.015 | 0.106 | 0.144 | 4.7 | -1.5 | 11.2 |
|  | Relative Humidity Ambient |  | 0.002 | 0.006 | -0.010 | 0.014 | 0.731 | 0.2 | -1.0 | 1.4 |
|  | Wind direction |  | 0.001 | 0.000 | 0.000 | 0.002 | 0.237 | 0.1 | 0.0 | 0.2 |
|  | Wind velocity |  | -0.163 | 0.085 | -0.329 | 0.003 | 0.055 | -15.0 | -28.0 | 0.3 |

Table S13 In-cabin eBC: complete results of the multiple linear regression using GEE models.

| Variable | Category | β | SE | 95% CI |  | p-value | Delta% | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intercept |  | 8.206 | 0.092 | 8.025 | 8.386 | <0.0001 |  |  |  |
| Leading Vehicle | Presence | 0.516 | 0.141 | 0.240 | 0.792 | <0.0001 | 67.5 | 27.1 | 120.8 |
|  | Absence | Ref |  |  |  |  |  |  |  |
| Leading vehicle Emission Standard Level | EURO 4 | -0.399 | 0.145 | -0.685 | -0.114 | 0.006 | -32.9 | -49.6 | -10.8 |
|  | EURO 5 | -0.501 | 0.145 | -0.786 | -0.217 | 0.001 | -39.4 | -54.4 | -19.5 |
|  | EURO 6 | -0.629 | 0.168 | -0.957 | -0.301 | <0.0001 | -46.7 | -61.6 | -26.0 |
|  | EURO 0123 | Ref |  |  |  |  |  |  |  |
| leading vehicle fuel type | DIESEL | 0.374 | 0.098 | 0.182 | 0.565 | <0.0001 | 45.3 | 20.0 | 75.9 |
|  | P.&CNG+ HYBRID | -0.039 | 0.102 | -0.240 | 0.161 | 0.702 | -3.8 | -21.3 | 17.5 |
|  | PETROL | Ref |  |  |  |  |  |  |  |
| Road Types | E.-U. by-pass | 0.310 | 0.125 | 0.065 | 0.555 | 0.013 | 36.4 | 6.7 | 74.2 |
|  | Rural | -0.335 | 0.118 | -0.566 | -0.104 | 0.005 | -28.5 | -43.2 | -9.9 |
|  | Urban | 0.189 | 0.091 | 0.011 | 0.367 | 0.038 | 20.8 | 1.1 | 44.3 |
|  | E.-U. conn. | Ref |  |  |  |  |  |  |  |

Table S14 Summary of the results of the multiple regression using GEE models relating in-cabin PM0.3-1 measurement data. The ∆% of the statistically significant results are reported in bold.

| Variable | Category | ∆% | 95% CI | p-value |
| --- | --- | --- | --- | --- |
| Leading Vehicle | Presence | 21.7 | (2.3; 44.8) | 0.027 |
|  | Absence | Ref |  |  |
| Leading vehicle Emission Standard Level | Euro 4 | -14.0 | (-26.1; -0.1) | 0.049 |
|  | Euro 5 | -13.9 | (-25.2; -1.0) | 0.036 |
|  | Euro 6 | -18.7 | (-29.1; -6.9) | 0.003 |
|  | Euro 0123 | Ref |  |  |
| Road Types | E.U. by-pass | 0.9 | (-15.4; 20.2) | 0.923 |
|  | Rural | -25.5 | (-38.4; -9.9) | 0.002 |
|  | Urban | -0.8 | (-12.6; 12.5) | 0.896 |
|  | E.U. cities connection | Ref |  |  |
| RH (in-vehicle) |  | 21.7 | (2.3; 44.8) | 0.019 |

Table S15 In-cabin PM0.3-1: complete results of the univariate linear regression using GEE models.

| TRAP | Variable | Category | β | SE | 95% CI |  | p-value | Delta % | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Log PM0.3-1 | Campaign | 0 - summer | Ref |  |  |  |  |  |  |  |
|  | Campaign | 1 - autumn | -0.392 | 0.241 | -0.864 | 0.079 | 0.103 | -32.4 | -57.8 | 8.2 |
|  | Day | 1 | Ref |  |  |  |  |  |  |  |
|  | Day | 2 | 0.066 | 0.302 | -0.526 | 0.657 | 0.828 | 6.8 | -40.9 | 92.9 |
|  | Day | 3 | -0.083 | 0.382 | -0.832 | 0.665 | 0.827 | -8.0 | -56.5 | 94.4 |
|  | Day | 4 | -0.434 | 0.363 | -1.145 | 0.278 | 0.233 | -35.2 | -68.2 | 32.1 |
|  | Day | 5 | 0.333 | 0.153 | 0.033 | 0.633 | 0.029 | 39.6 | 3.4 | 88.4 |
|  | Roundtrip | Outward | Ref |  |  |  |  |  |  |  |
|  | Roundtrip | Return-trip | 0.237 | 0.251 | -0.254 | 0.728 | 0.344 | 26.8 | -22.4 | 107.1 |
|  | Leading Vehicle | ABSENCE | Ref |  |  |  |  |  |  |  |
|  | Leading Vehicle | PRESENCE | 0.156 | 0.045 | 0.067 | 0.244 | 0.001 | 16.8 | 6.9 | 27.6 |
|  | Leading vehicle types | ABSENCE | -0.116 | 0.041 | -0.196 | -0.035 | 0.005 | -10.9 | -17.8 | -3.5 |
|  | Leading vehicle types | HEAVY-DUTY | 0.223 | 0.077 | 0.072 | 0.373 | 0.004 | 24.9 | 7.5 | 45.2 |
|  | Leading vehicle types | LIGHT-DUTY | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | ABSENCE | -0.342 | 0.100 | -0.538 | -0.146 | 0.001 | -29.0 | -41.6 | -13.6 |
|  | Leading vehicle Em. Std._Level | EURO 0123 | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | EURO 4 | -0.183 | 0.080 | -0.339 | -0.026 | 0.022 | -16.7 | -28.8 | -2.6 |
|  | Leading vehicle Em. Std._Level | EURO 5 | -0.191 | 0.091 | -0.368 | -0.013 | 0.035 | -17.4 | -30.8 | -1.3 |
|  | Leading vehicle Em. Std._Level | EURO 6 | -0.271 | 0.072 | -0.411 | -0.130 | <0.0001 | -23.7 | -33.7 | -12.2 |
|  | Leading vehicle fuel type | ABSENCE | -0.116 | 0.047 | -0.208 | -0.025 | 0.012 | -11.0 | -18.7 | -2.5 |
|  | Leading vehicle fuel type | DIESEL | 0.075 | 0.063 | -0.048 | 0.199 | 0.233 | 7.8 | -4.7 | 22.0 |
|  | Leading vehicle fuel type | PETROL | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle fuel type | P.&CNG+ HYBRID | -0.046 | 0.122 | -0.285 | 0.193 | 0.706 | -4.5 | -24.8 | 21.3 |
|  | Traffic Jam | No | Ref |  |  |  |  |  |  |  |
|  | Traffic Jam | Yes | 0.065 | 0.028 | 0.010 | 0.120 | 0.021 | 6.7 | 1.0 | 12.7 |
|  | Traffic Light Stops | No | Ref |  |  |  |  |  |  |  |
|  | Traffic Light Stops | Yes | -0.001 | 0.057 | -0.113 | 0.111 | 0.985 | -0.1 | -10.7 | 11.7 |
|  | Road Types | E.-U. conn. | 0.286 | 0.103 | 0.084 | 0.487 | 0.005 | 33.1 | 8.8 | 62.8 |
|  | Road Types | E.-U. by-pass | 0.357 | 0.075 | 0.211 | 0.504 | <0.0001 | 42.9 | 23.5 | 65.5 |
|  | Road Types | Rural | Ref |  |  |  |  |  |  |  |
|  | Road Types | Urban | 0.292 | 0.131 | 0.036 | 0.547 | 0.026 | 33.8 | 3.6 | 72.9 |
|  | T (in-cabin) |  | -0.040 | 0.031 | -0.100 | 0.020 | 0.192 | -3.9 | -9.5 | 2.0 |
|  | RH (in-cabin) |  | 0.027 | 0.011 | 0.006 | 0.049 | 0.011 | 2.8 | 0.6 | 5.0 |
|  | Speed |  | 0.000 | 0.001 | -0.003 | 0.002 | 0.707 | 0.0 | -0.3 | 0.2 |
|  | PM2.5_FixedStations |  | -0.006 | 0.021 | -0.048 | 0.036 | 0.779 | -0.6 | -4.7 | 3.6 |
|  | PM10_FixedStations |  | -0.005 | 0.023 | -0.049 | 0.040 | 0.836 | -0.5 | -4.8 | 4.1 |
|  | CO_FixedStations |  | 0.030 | 0.347 | -0.651 | 0.711 | 0.932 | 3.0 | -47.8 | 103.5 |
|  | NO2_FixedStations |  | 0.007 | 0.007 | -0.008 | 0.021 | 0.361 | 0.7 | -0.8 | 2.1 |
|  | Temperature Ambient |  | 0.024 | 0.028 | -0.031 | 0.080 | 0.389 | 2.5 | -3.1 | 8.3 |
|  | Relative Humidity Ambient |  | -0.002 | 0.005 | -0.011 | 0.007 | 0.658 | -0.2 | -1.1 | 0.7 |
|  | Wind direction |  | -0.001 | 0.001 | -0.002 | 0.001 | 0.337 | -0.1 | -0.2 | 0.1 |
|  | Wind velocity |  | -0.136 | 0.072 | -0.276 | 0.005 | 0.058 | -12.7 | -24.1 | 0.5 |

Table S16 In-cabin PM0.3-1: complete results of the multiple linear regression using GEE models.

| Variable | Category | β | SE | 95% CI |  | p-value | Delta% | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intercept |  | 2.099 | 0.564 | 0.993 | 3.204 | 0.000 |  |  |  |
| Leading Vehicle | Presence | 0.197 | 0.089 | 0.023 | 0.370 | 0.027 | 21.7 | 2.3 | 44.8 |
|  | Absence | Ref |  |  |  |  |  |  |  |
| Leading vehicle Emission Standard Level | EURO 4 | -0.151 | 0.077 | -0.302 | -0.001 | 0.049 | -14.0 | -26.1 | -0.1 |
|  | EURO 5 | -0.150 | 0.072 | -0.290 | -0.010 | 0.036 | -13.9 | -25.2 | -1.0 |
|  | EURO 6 | -0.208 | 0.070 | -0.344 | -0.071 | 0.003 | -18.7 | -29.1 | -6.9 |
|  | EURO 0123 | Ref |  |  |  |  |  |  |  |
| Road Types | E.-U. by-pass | 0.009 | 0.090 | -0.167 | 0.184 | 0.923 | 0.9 | -15.4 | 20.2 |
|  | Rural | -0.294 | 0.097 | -0.484 | -0.105 | 0.002 | -25.5 | -38.4 | -9.9 |
|  | Urban | -0.008 | 0.064 | -0.134 | 0.117 | 0.896 | -0.8 | -12.6 | 12.5 |
|  | E.-U. conn. | Ref |  |  |  |  |  |  |  |
| RH (in-vehicle) |  | 0.027 | 0.011 | 0.005 | 0.049 | 0.019 | 21.7 | 2.3 | 44.8 |

Table S17 Summary of the results of the multiple regression using GEE models relating in-cabin PM1-2.5 measurement data. The ∆% of the statistically significant results are reported in bold.

| Variable | Category | ∆% | 95% CI | p-value |
| --- | --- | --- | --- | --- |
| Leading Vehicle | Presence | 14.6 | (3.4; 27.1) | 0.010 |
|  | Absence | Ref |  |  |
| Leading vehicle types | Heavy-duty | 30.4 | (10; 54.6) | 0.002 |
|  | Light-duty | Ref |  |  |
| Leading vehicle Emission Standard Level | Euro 4 | -10.2 | (-19.5; 0.3) | 0.056 |
|  | Euro 5 | -6.3 | (-16; 4.6) | 0.247 |
|  | Euro 6 | -12.5 | (-20.4; -3.9) | 0.005 |
|  | Euro 0123 | Ref |  |  |
| Road Types | E.U. by-pass | 5.0 | (-6.8; 18.4) | 0.421 |
|  | Rural | -26.2 | (-35.7; -15.2) | <0.0001 |
|  | Urban | 3.7 | (-6.8; 15.3) | 0.507 |
|  | E.U. cities connection | Ref |  |  |
| sin of wind direction |  | 8.3 | (1.1; 16.1) | 0.024 |

Table S18 In-cabin PM1-2.5: complete results of the univariate linear regression using GEE models.

| TRAP | Variable | Category | β | SE | 95% CI |  | p-value | Delta % | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Log PM1-2.5 | Campaign | 0 - summer | Ref |  |  |  |  |  |  |  |
|  | Campaign | 1 - autumn | -0.379 | 0.198 | -0.767 | 0.009 | 0.055 | -31.5 | -53.5 | 0.9 |
|  | Day | 1 | Ref |  |  |  |  |  |  |  |
|  | Day | 2 | 0.339 | 0.247 | -0.145 | 0.823 | 0.170 | 40.3 | -13.5 | 127.7 |
|  | Day | 3 | 0.059 | 0.333 | -0.593 | 0.711 | 0.859 | 6.1 | -44.7 | 103.6 |
|  | Day | 4 | -0.402 | 0.273 | -0.938 | 0.134 | 0.142 | -33.1 | -60.9 | 14.3 |
|  | Day | 5 | 0.089 | 0.144 | -0.192 | 0.370 | 0.535 | 9.3 | -17.5 | 44.8 |
|  | Roundtrip | Outward | Ref |  |  |  |  |  |  |  |
|  | Roundtrip | Return-trip | 0.228 | 0.209 | -0.182 | 0.637 | 0.277 | 25.6 | -16.7 | 89.2 |
|  | Leading Vehicle | ABSENCE | Ref |  |  |  |  |  |  |  |
|  | Leading Vehicle | PRESENCE | 0.172 | 0.034 | 0.105 | 0.239 | <0.0001 | 18.8 | 11.1 | 27.0 |
|  | Leading vehicle types | ABSENCE | -0.126 | 0.029 | -0.183 | -0.070 | <0.0001 | -11.9 | -16.7 | -6.8 |
|  | Leading vehicle types | HEAVY-DUTY | 0.349 | 0.078 | 0.197 | 0.502 | <0.0001 | 41.8 | 21.7 | 65.2 |
|  | Leading vehicle types | LIGHT-DUTY | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | ABSENCE | -0.327 | 0.071 | -0.465 | -0.188 | <0.0001 | -27.9 | -37.2 | -17.1 |
|  | Leading vehicle Em. Std._Level | EURO 0123 | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle Em. Std._Level | EURO 4 | -0.182 | 0.059 | -0.298 | -0.066 | 0.002 | -16.7 | -25.8 | -6.4 |
|  | Leading vehicle Em. Std._Level | EURO 5 | -0.140 | 0.065 | -0.267 | -0.013 | 0.030 | -13.1 | -23.4 | -1.3 |
|  | Leading vehicle Em. Std._Level | EURO 6 | -0.209 | 0.064 | -0.333 | -0.084 | 0.001 | -18.8 | -28.3 | -8.1 |
|  | Leading vehicle fuel type | ABSENCE | -0.145 | 0.039 | -0.222 | -0.069 | <0.0001 | -13.5 | -19.9 | -6.6 |
|  | Leading vehicle fuel type | DIESEL | 0.046 | 0.048 | -0.048 | 0.139 | 0.341 | 4.7 | -4.7 | 14.9 |
|  | Leading vehicle fuel type | PETROL | Ref |  |  |  |  |  |  |  |
|  | Leading vehicle fuel type | P.&CNG+ HYBRID | 0.073 | 0.096 | -0.115 | 0.261 | 0.448 | 7.5 | -10.9 | 29.8 |
|  | Traffic Jam | No | Ref |  |  |  |  |  |  |  |
|  | Traffic Jam | Yes | 0.075 | 0.028 | 0.020 | 0.131 | 0.008 | 7.8 | 2.0 | 14.0 |
|  | Traffic Light Stops | No | Ref |  |  |  |  |  |  |  |
|  | Traffic Light Stops | Yes | 0.021 | 0.035 | -0.046 | 0.089 | 0.538 | 2.2 | -4.5 | 9.3 |
|  | Road Types | E.-U. conn. | 0.348 | 0.069 | 0.213 | 0.482 | <0.0001 | 41.6 | 23.8 | 61.9 |
|  | Road Types | E.-U. by-pass | 0.433 | 0.066 | 0.305 | 0.562 | <0.0001 | 54.2 | 35.6 | 75.4 |
|  | Road Types | Rural | Ref |  |  |  |  |  |  |  |
|  | Road Types | Urban | 0.405 | 0.101 | 0.207 | 0.603 | <0.0001 | 49.9 | 22.9 | 82.8 |
|  | T (in-cabin) |  | -0.040 | 0.022 | -0.083 | 0.003 | 0.070 | -3.9 | -8.0 | 0.3 |
|  | RH (in-cabin) |  | 0.017 | 0.009 | -0.001 | 0.035 | 0.062 | 1.7 | -0.1 | 3.6 |
|  | Speed |  | -0.002 | 0.001 | -0.003 | 0.000 | 0.056 | -0.2 | -0.3 | 0.0 |
|  | PM2.5_FixedStations |  | 0.003 | 0.019 | -0.034 | 0.040 | 0.871 | 0.3 | -3.3 | 4.1 |
|  | PM10_FixedStations |  | 0.009 | 0.021 | -0.033 | 0.051 | 0.669 | 0.9 | -3.2 | 5.3 |
|  | CO_FixedStations |  | 0.210 | 0.254 | -0.287 | 0.707 | 0.407 | 23.4 | -24.9 | 102.9 |
|  | NO2_FixedStations |  | 0.007 | 0.006 | -0.005 | 0.020 | 0.263 | 0.7 | -0.5 | 2.0 |
|  | Temperature Ambient |  | 0.039 | 0.022 | -0.004 | 0.083 | 0.074 | 4.0 | -0.4 | 8.6 |
|  | Relative Humidity Ambient |  | 0.000 | 0.004 | -0.008 | 0.008 | 0.973 | 0.0 | -0.8 | 1.0 |
|  | Wind direction |  | 0.000 | 0.001 | -0.002 | 0.001 | 0.436 | 0.0 | -0.2 | 0.1 |
|  | Sin Wind direction |  | 0.124 | 0.039 | 0.047 | 0.002 | 0.011 | 13.2 | 4.8 | -4.6 |
|  | Cos wind direction |  | 0.016 | 0.050 | -0.081 | 0.743 | 0.746 | 1.7 | -7.8 | 110.2 |
|  | Wind velocity |  | -0.022 | 0.026 | -0.073 | 0.029 | 0.397 | -2.2 | -7.0 | 2.9 |

Table S19 In-cabin PM1-2.5: complete results of the multiple linear regression using GEE models.

| Variable | Category | β | SE | 95% CI |  | p-value | Delta% | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intercept |  | -1.4938 | 0.1188 | -1.7266 | -1.2611 | <0.0001 |  |  |  |
| Leading Vehicle | PRESENCE | 0.1362 | 0.0526 | 0.033 | 0.2394 | 0.010 | 14.59 | 3.36 | 27.05 |
| Leading Vehicle | ABSENCE | Ref |  |  |  |  |  |  |  |
| Leading vehicle Emission S_L | EURO 4 | -0.1073 | 0.0561 | -0.2172 | 0.0027 | 0.056 | -10.17 | -19.52 | 0.27 |
| Leading vehicle Emission S_L | EURO 5 | -0.0648 | 0.0559 | -0.1744 | 0.0448 | 0.247 | -6.27 | -16.00 | 4.58 |
| Leading vehicle Emission S_L | EURO 6 | -0.1339 | 0.0481 | -0.2282 | -0.0396 | 0.005 | -12.53 | -20.40 | -3.88 |
| Leading vehicle Emission S_L | EURO 0123 | Ref |  |  |  |  |  |  |  |
| Leading vehicle types | HEAVY-DUTY | 0.2653 | 0.0869 | 0.095 | 0.4356 | 0.002 | 30.38 | 9.97 | 54.59 |
| Leading vehicle types | LIGHT-DUTY | Ref |  |  |  |  |  |  |  |
| Road Types | E.-U. by-pass | 0.0491 | 0.0611 | -0.0706 | 0.1688 | 0.421 | 5.03 | -6.82 | 18.39 |
| Road Types | Rural | -0.3033 | 0.0705 | -0.4415 | -0.1651 | <0.0001 | -26.16 | -35.69 | -15.22 |
| Road Types | Urban | 0.0361 | 0.0544 | -0.0705 | 0.1427 | 0.507 | 3.68 | -6.81 | 15.34 |
| Road Types | E.-U. conn. | Ref |  |  |  |  |  |  |  |
| sin_wind_dir |  | 0.0798 | 0.0354 | 0.0105 | 0.1492 | 0.024 | 8.31 | 1.06 | 16.09 |
| cos_wind_dir |  | -0.0005 | 0.0371 | -0.0731 | 0.0721 | 0.989 | -0.05 | -7.05 | 7.48 |

Table S20 Summary of the results of the multiple regression using GEE models relating in-cabin UFPs measurement data. The ∆% of the statistically significant results are reported in bold.

| Variable | Category | ∆% | 95% CI | p-value |
| --- | --- | --- | --- | --- |
| Leading Vehicle | Presence | 30.6 | (3.7; 64.5) | 0.023 |
|  | Absence | Ref |  |  |
| Leading vehicle Emission Standard Level | Euro 4 | -16.3 | (-35.0; 7.9) | 0.171 |
|  | Euro 5 | -28.9 | (-45.0; -8.0) | 0.010 |
|  | Euro 6 | -16.3 | (-38.5; 14.0) | 0.259 |
|  | Euro 0123 | Ref |  |  |
| Road Types | E.U. by-pass | 37.7 | (6.8; 77.5) | 0.014 |
|  | Rural | -46.2 | (-60.1; -27.4) | <0.0001 |
|  | Urban | 5.7 | (-12.4; 27.5) | 0.561 |
|  | E.U. cities connection | Ref |  |  |
| RH (in-vehicle) |  | 2.6 | (0.4; 4.9) | 0.022 |

Table S21 In-cabin UFPs: complete results of the univariate linear regression using GEE models.

| TRAP | Variable |  | Category |  | β |  | SE |  | 95% CI |  |  |  | p-value |  | Delta % |  | 95% CI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Log UFPs | Campaign |  | 0 - summer |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Campaign |  | 1 - autumn |  | 0.028 |  | 0.109 |  | -0.186 |  | 0.243 |  | 0.794 |  | 2.9 |  | -16.9 |  | 27.5 |  |
|  |  | Day |  | 1 |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Day |  | 2 |  | -0.020 |  | 0.154 |  | -0.322 |  | 0.281 |  | 0.894 |  | -2.0 |  | -27.5 |  | 32.5 |  |
|  |  | Day |  | 3 |  | -0.149 |  | 0.110 |  | -0.364 |  | 0.066 |  | 0.175 |  | -13.8 |  | -30.5 |  | 6.8 |  |
|  |  | Day |  | 4 |  | -0.381 |  | 0.161 |  | -0.697 |  | -0.065 |  | 0.018 |  | -31.7 |  | -50.2 |  | -6.3 |  |
|  |  | Day |  | 5 |  | -0.252 |  | 0.128 |  | -0.503 |  | -0.001 |  | 0.049 |  | -22.3 |  | -39.5 |  | -0.1 |  |
|  |  | Roundtrip |  | Outward |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Roundtrip |  | Return-trip |  | -0.016 |  | 0.110 |  | -0.231 |  | 0.199 |  | 0.886 |  | -1.6 |  | -20.6 |  | 22.0 |  |
|  |  | Leading Vehicle |  | ABSENCE |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Leading Vehicle |  | PRESENCE |  | 0.325 |  | 0.073 |  | 0.181 |  | 0.468 |  | <0.0001 |  | 38.3 |  | 19.9 |  | 59.6 |  |
|  |  | Leading vehicle types |  | ABSENCE |  | -0.264 |  | 0.064 |  | -0.390 |  | -0.138 |  | <0.0001 |  | -23.2 |  | -32.3 |  | -12.9 |  |
|  |  | Leading vehicle types |  | HEAVY-DUTY |  | 0.395 |  | 0.345 |  | -0.282 |  | 1.072 |  | 0.253 |  | 48.5 |  | -24.6 |  | 192.1 |  |
|  |  | Leading vehicle types |  | LIGHT-DUTY |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Leading vehicle Em. Std._Level |  | ABSENCE |  | -0.596 |  | 0.210 |  | -1.008 |  | -0.185 |  | 0.005 |  | -44.9 |  | -63.5 |  | -16.9 |  |
|  |  | Leading vehicle Em. Std._Level |  | EURO 0123 |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Leading vehicle Em. Std._Level |  | EURO 4 |  | -0.222 |  | 0.184 |  | -0.582 |  | 0.138 |  | 0.227 |  | -19.9 |  | -44.1 |  | 14.8 |  |
|  |  | Leading vehicle Em. Std._Level |  | EURO 5 |  | -0.392 |  | 0.198 |  | -0.780 |  | -0.005 |  | 0.047 |  | -32.5 |  | -54.1 |  | -0.5 |  |
|  |  | Leading vehicle Em. Std._Level |  | EURO 6 |  | -0.287 |  | 0.239 |  | -0.755 |  | 0.180 |  | 0.229 |  | -25.0 |  | -53.0 |  | 19.8 |  |
|  |  | Leading vehicle fuel type |  | ABSENCE |  | -0.217 |  | 0.070 |  | -0.354 |  | -0.080 |  | 0.002 |  | -19.5 |  | -29.8 |  | -7.7 |  |
|  |  | Leading vehicle fuel type |  | DIESEL |  | 0.238 |  | 0.118 |  | 0.007 |  | 0.470 |  | 0.044 |  | 26.9 |  | 0.7 |  | 60.0 |  |
|  |  | Leading vehicle fuel type |  | PETROL |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Leading vehicle fuel type |  | P.&CNG+ HYBRID |  | -0.085 |  | 0.134 |  | -0.347 |  | 0.177 |  | 0.525 |  | -8.2 |  | -29.3 |  | 19.4 |  |
|  |  | Traffic Jam |  | No |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Traffic Jam |  | Yes |  | 0.208 |  | 0.067 |  | 0.076 |  | 0.340 |  | 0.002 |  | 23.2 |  | 7.9 |  | 40.6 |  |
|  |  | Traffic Light Stops |  | No |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Traffic Light Stops |  | Yes |  | -0.030 |  | 0.068 |  | -0.163 |  | 0.103 |  | 0.660 |  | -2.9 |  | -15.1 |  | 10.9 |  |
|  |  | Road Types |  | E.-U. conn. |  | 0.610 |  | 0.151 |  | 0.314 |  | 0.906 |  | <0.0001 |  | 84.0 |  | 36.8 |  | 147.5 |  |
|  |  | Road Types |  | E.-U. by-pass |  | 0.982 |  | 0.166 |  | 0.657 |  | 1.308 |  | <0.0001 |  | 167.1 |  | 92.9 |  | 269.8 |  |
|  |  | Road Types |  | Rural |  | Ref |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Road Types |  | Urban |  | 0.661 |  | 0.160 |  | 0.346 |  | 0.975 |  | <0.0001 |  | 93.6 |  | 41.4 |  | 165.2 |  |
|  |  | T (in-cabin) |  |  |  | -0.021 |  | 0.031 |  | -0.081 |  | 0.040 |  | 0.500 |  | -2.1 |  | -7.8 |  | 4.0 |  |
|  |  | RH (in-cabin) |  |  |  | 0.032 |  | 0.015 |  | 0.002 |  | 0.063 |  | 0.036 |  | 3.3 |  | 0.2 |  | 6.5 |  |
|  |  | Speed |  |  |  | -0.001 |  | 0.002 |  | -0.004 |  | 0.002 |  | 0.597 |  | -0.1 |  | -0.4 |  | 0.2 |  |
|  |  | PM2.5_FixedStations |  |  |  | 0.014 |  | 0.017 |  | -0.019 |  | 0.047 |  | 0.413 |  | 1.4 |  | -1.9 |  | 4.8 |  |
|  |  | PM10_FixedStations |  |  |  | -0.002 |  | 0.023 |  | -0.046 |  | 0.042 |  | 0.926 |  | -0.2 |  | -4.5 |  | 4.3 |  |
|  |  | CO_FixedStations |  |  |  | 0.184 |  | 0.427 |  | -0.653 |  | 1.022 |  | 0.666 |  | 20.2 |  | -48.0 |  | 177.8 |  |
|  |  | NO2_FixedStations |  |  |  | 0.028 |  | 0.009 |  | 0.011 |  | 0.046 |  | 0.002 |  | 2.9 |  | 1.1 |  | 4.7 |  |
|  |  | Temperature Ambient |  |  |  | 0.040 |  | 0.034 |  | -0.027 |  | 0.106 |  | 0.242 |  | 4.0 |  | -2.6 |  | 11.2 |  |
|  |  | Relative Humidity Ambient |  |  |  | -0.007 |  | 0.009 |  | -0.024 |  | 0.011 |  | 0.439 |  | -0.7 |  | -2.4 |  | 1.1 |  |
|  |  | Wind direction |  |  |  | -0.002 |  | 0.001 |  | -0.004 |  | -0.001 |  | 0.002 |  | -0.2 |  | -0.4 |  | -0.1 |  |
|  |  | Sin Wind direction |  |  |  | -0.030 |  | 0.094 |  | -0.214 |  | 0.749 |  | 0.752 |  | -3.0 |  | -19.3 |  | 111.5 |  |
|  |  | Cos wind direction |  |  |  | -0.060 |  | 0.108 |  | -0.272 |  | 0.576 |  | 0.577 |  | -5.9 |  | -23.8 |  | 77.8 |  |
|  |  | Wind velocity |  |  |  | -0.085 |  | 0.049 |  | -0.182 |  | 0.011 |  | 0.084 |  | -8.2 |  | -16.6 |  | 1.2 |  |

Table S22 In-cabin UFPs: complete results of the multiple linear regression using GEE models.

| Variable | Category | β | SE | 95% CI |  | p-value | Delta% | 95% CI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intercept |  | 8.237 | 0.548 | 7.163 | 9.310 | <0.0001 |  |  |  |
| Leading Vehicle | PRESENCE | 0.267 | 0.118 | 0.036 | 0.498 | 0.023 | 30.6 | 3.7 | 64.5 |
| Leading Vehicle | ABSENCE | Ref |  |  |  |  |  |  |  |
| Leading Vehicle EmissionS_L | EURO 4 | -0.177 | 0.129 | -0.431 | 0.076 | 0.171 | -16.3 | -35.0 | 7.9 |
| Leading Vehicle EmissionS_L | EURO 5 | -0.341 | 0.131 | -0.598 | -0.083 | 0.010 | -28.9 | -45.0 | -8.0 |
| Leading Vehicle EmissionS_L | EURO 6 | -0.177 | 0.157 | -0.486 | 0.131 | 0.259 | -16.3 | -38.5 | 14.0 |
| Leading Vehicle EmissionS_L | EURO 0123 | Ref |  |  |  |  |  |  |  |
| Road Types | E.-U. by-pass | 0.320 | 0.130 | 0.066 | 0.574 | 0.014 | 37.7 | 6.8 | 77.5 |
| Road Types | Rural | -0.619 | 0.152 | -0.918 | -0.321 | <0.0001 | -46.2 | -60.1 | -27.4 |
| Road Types | Urban | 0.056 | 0.096 | -0.132 | 0.243 | 0.561 | 5.7 | -12.4 | 27.5 |
| Road Types | E.-U. conn. | Ref |  |  |  |  |  |  |  |
| RH |  | 0.026 | 0.011 | 0.004 | 0.048 | 0.022 | 2.6 | 0.4 | 4.9 |