Assessment of Indoor Air Pollution from Cooking and Cleaning Activities in Italian Restaurant Kitchens

Marta Keller a*, Andrea Cattaneo a, Davide Campagnolo a*, Francesca Borghi b, Alessio Carminati a, Giacomo Fanti a, Sabrina Rovelli a, Carolina Zellino a, Andrea Spinazzè a, Victor G. Mihucz c, Carlo Dossi d, Domenico M. Cavallo a

a Department of Science and High Technology, University of Insubria, via Valleggio 11, 22100 Como, Italy

b Department of Medical and Surgical Sciences, University of Bologna, via Pelagio Palagi 9, 40138 Bologna, Italy

c Institute of Chemistry, Eötvös Loránd University, Pázmány Péter sétány 1/A, H-1117 Budapest, Hungary

d Department of Theoretical and Applied Science, University of Insubria, via J.H. Dunant 3, 21100 Varese, Italy

*Corresponding authors:

Marta Keller

via Valleggio, 11 – 22100 Como – Italy

Tel.: +39 031 2386628; Fax: +39 031 2386630

E-mail address: marta.keller10@gmail.com

Davide Campagnolo

via Valleggio, 11 – 22100 Como – Italy

Tel.: +39 031 2386621; Fax: +39 031 2386630

E-mail address: davide.campagnolo@uninsubria.it

Supplementary material

Table SM1-Checklist submitted to each owner for each restaurant, to collect information on the cuisine and activities.

| n | Question | Information |
| --- | --- | --- |
| 1 | Restaurant age (year) |  |
| 2 | Kitchen area (m2) |  |
| 3 | Wall height (m) |  |
| 4 | Number doors of the kitchen |  |
| 5 | Is there a door bordering the outside? |  |
| 6 | Number people in kitchen |  |
| 7 | Outdoor environments | - Rural - Suburban - Center - Industrial area - Other (specify) |
| 8 | Outdoor sources | - Vehicular traffic - Subway / train station - Countryside - Other (specify) |
| 9 | Is there a “passe”? | - Yes - No |
| 10 | Kitchen ventilation | - Air vents - Windows - Other (specify) |
| 11 | Number windows and orientation (N/S/E/W) |  |
| 12 | Can the windows open? | - Yes - No |
| 13 | Types of window frame | - Metal - Wood - Aluminium - Other (specify) |
| 14 | Position of the air supply devices | - Absent - Floor - Walls - Ceiling - Other (specify) |
| 15 | N hoods and size |  |
| 16 | Hood types | - Telescopic (on the kitchen top) - Built-in - Integrated - Island - Other (specify) |
| 17 | Number stoves and oven |  |
| 18 | Type of stoves and oven | - Gas - Induction - Electric oven - Gas oven - Other (specify) |
| 19 | What types of heating systems are there? | - Nothing - Radiators - Fan Coil - Centralized - On the floor - Heat pumps - Other (specify) |
| 20 | What types of cooling systems are there? | - Nothing - Windows - Centralized - On the floor - Air conditioning - Fan Coil - Other (specify) |
| 21 | Presence of sunscreen on the windows | - Curtains - Shutters - Venetian blinds - Other (specify) - None |
| 22 | Control of sunscreen | - Manual - Automatic - Other (specify) |
| 23 | Floor cleaning (frequency, last time it was cleaned, product type (brand)) |  |
| 24 | Windows cleaning (frequency, last time it was cleaned, product type (brand)) |  |
| 25 | Cleaning the work surface (frequency, last time it was cleaned, product type (brand)) |  |
| 26 | Dishwasher | - Waiter - Dishwasher - Assistent Cook – Dishwasher - Dishwasher |
| 27 | Working hours of the dishwasher (start and end) |  |
| 28 | Dish soap (product type (brand)) |  |
| 29 | Dishwasher detergent (product type (brand)) |  |

Table SM2- Detection range and limits of the on-line instrumentation used to monitor the selected air pollutants.

| Pollutant | Operating principle | Range | LOD |
| --- | --- | --- | --- |
| O3 | UV adsorption at 254 nm | 0 ppb-10 ppm | 0.003 ppm |
| TVOCs | Photo Ionization Detector | 0.00-20 ppm | 0.01 ppm |
| HCHO | Electrochemical Sensor | 0.00-10 ppm | 0.01 ppm |
| UFPs | Condensation Particle Counter | 0-500,000 pt/cm3 | 0.02 µm |
| NO2 | Electrochemical Sensor | 0.00-0.25 ppm | 0.02 ppm |
| CO2 | Non-dispersive Infra-Red analysis | 0-2500 ppm | 170 ppm |

LOD= Limit of detection

Table SM3- Operating parameters of the ATD-GC-FID instrumentation.

| ATD |  |
| --- | --- |
| Carrier gas | Helium |
| Oven Temperature | 280 °C |
| Primary Desorption Time | 6 min |
| Desorb Flow | 20 mL/min |
| Trap Law Temperature | -12 °C |
| Trap High Temperature | 300 °C |
| Trap Fast | 40 °C/s |
| Valve Temperature | 200 °C |
| Transfer Line Temperature | 200 °C |
| Trap Hold | 1 min |
| GC-FID |  |
| Carrier gas | Helium, 1 mL/min |
| Column Temperature program | 50 °C, 5 min; 2 °C/min to 100 °C; 10 °C/min to 200 °C; 200 °C, 10 min |
| FID temperature | 250 °C |
| Flame Composition | Hydrogen and Air |
| Total Run Time | 55 min |

Table SM4- Operating parameters of the HPLC-UV instrumentation.

| HPLC-PDA |  |
| --- | --- |
| Eluent A | Acetonitrile, 60% |
| Eluent B | MilliQ water, 40% |
| Pump flow | 1 mL/min |
| Pump Pressure | 80 bar |
| Oven Temperature | 40 °C |
| Injection Volume | 20 µL |
| Detector Operating Wavelength | 360 nm |

Identification and quantification of organic molecules

The qualitative identification of the target Volatile Organic Compounds (VOCs) was conducted by comparing the retention times of peaks obtained in real samples with those from standard solutions. Each analyte's retention time was determined by analyzing every individual standard solution using the same procedure applied to real samples. An aliquot (1μL) of each standard was added to a Tenax TA tube. Chrom-Card software facilitated chromatographic analysis, particularly for quantitative determination. For this purpose, a single standard stock solution was prepared for each analyte by diluting 120 μL of the pure liquid analyte in 50 mL of pure heptane (Sigma Aldrich, Milwaukee, WI, USA). From these single-standard stock solutions, four multi-standards were prepared at approximately 80, 40, 20, and 5 μg/mL. Instrument calibration involved adding an aliquot (1μL) of each multi-standard solution into four different Tenax TA test tubes, covering a mass range from approximately 5 to 80 ng.

As for the VOCs, the qualitative identification of the target aldehydes was performed by comparing the retention times of the peaks obtained in the real samples with those obtained from the standard. These were prepared by diluting the analytes of interest in a solution of milliQ water and acetonitrile (40:60) to a concentration of approximately 0.1 μg/g. LabSolutions software was used for qualitative identification and quantitative analysis. For quantification, a single stock standard solution was prepared for each analyte by diluting it with a solution of milliQ water and acetonitrile (40:60). From these single standard stock solutions, four multistandard solutions were prepared at approximately 0.125, 0.1, 0.075 and 0.05 μg/g and assayed (20 μL) to calibrate the instrument.

The VOC and aldehyde calibration curves were calculated using heights rather than areas because the complexity of the real samples caused peak overlaps between target analytes and other sampled compounds, sometimes making it difficult to properly separate peaks and obtain an accurate area value (Zhang et al. 2017). The limits of detection (LOD) and quantification (LOQ) were determined as 3 and 10 times the standard deviation of the background noise divided by the slope (m) of the calibration curves. The LODs and LOQs were expressed both as mass (pg) and concentration in air (ng/m3), considering the average volume of air sampled during the field campaigns carried out in restaurants. Considering that all selected contaminants can have adverse effects on human health (Antonelli et al. 2020; Soni et al. 2018), it is crucial to have accurate methods for their evaluation. The LODs and LOQs determined are much lower than those present in the literature (Tokumura et al. 2016; Mandin et al. 2017). This is essential to allow the determination of organics at environmental concentration levels. Furthermore, all the analytes showed a good linearity in the investigated concentration range (Table SM5).

Table SM5- List of the investigated analytes and the values of slope (m), intercept (q), correlation coefficients (R2) and the calculated limits of detection (LODs) and limits of quantification (LOQs), both as mass and concentrations in air.

| Analyte | m | q | R2 | LOD (pg) | LOQ (pg) | LOD (ng/m3) | LOQ (ng/m3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Benzene | 210.65 | 355.02 | 0.996 | 81.2 | 271 | 51 | 170 |
| Toluene | 205.7 | 742.5 | 0.993 | 83.1 | 277 | 52.2 | 174 |
| Ethylbenzene | 185.81 | 156.96 | 0.997 | 92.0 | 307 | 57.7 | 193 |
| p-Xylene | 186.64 | 216.45 | 0.997 | 91.6 | 305 | 57.5 | 191 |
| m-Xylene | 12757 | 28515 | 0.995 | 175 | 584 | 110 | 367 |
| o-Xylene | 179.5 | 194.14 | 0.995 | 95.3 | 318 | 59.8 | 200 |
| Styrene | 194.3 | 196.45 | 0.997 | 88.0 | 293 | 55.2 | 184 |
| α-Pinene | 157.3 | 169.09 | 0.997 | 109 | 362 | 68.4 | 227 |
| d-Limonene | 166.4 | 311.83 | 0.995 | 103 | 343 | 64.6 | 215 |
| p-Cymene | 182.26 | 87.37 | 0.998 | 93.8 | 313 | 58.9 | 196 |
| 2-Butoxyethanol | 157.3 | 223.5 | 0.997 | 109 | 362 | 68.4 | 227 |
| Formaldehyde | 63554 | 0 | 0.999 | 230 | 760 | 15.5 | 513 |
| Acetaldehyde | 49019 | -53.211 | 0.999 | 290 | 980 | 19.6 | 661 |
| Acrolein | 30968 | -105.23 | 0.995 | 470 | 1600 | 31.7 | 108 |
| Propionaldehyde | 30099 | 63.189 | 0.999 | 480 | 1600 | 32.4 | 108 |
| Benzaldehyde | 14320 | -80.592 | 0.999 | 10000 | 3400 | 67.4 | 229 |
| Glutaraldehyde | 7304 | 0.5835 | 0.999 | 20000 | 6600 | 135 | 445 |

Figure SM1- Violin plots of the winter and summer campaigns of indoor pollutants investigated in restaurant kitchens. Light blue: winter campaign, dark red: summer campaign; *: p < 0.01 and **: p < 0.001 indicate statistically significant differences between seasons in the M-W test.

Figure SM2- Spearman correlation matrix showing relationships between pollutants and other additional parameters (winter campaign).

Figure SM3 – Spearman correlation matrix showing relationships between pollutants and other additional parameters (summer campaign).

Table SM6-Indoor concentrations of VOCs (ug/m3) for the winter (WC) and summer (SC) campaigns for each investigated restaurant by time-integrated sampling.

| ID | Sampling time |  | Benzene |  | Toluene |  | Ethylbenzene |  | p-Xylene |  | m-Xylene |  | o-Xylene |  | Styrene |  | α-Pinene |  | d-Limonene |  | p-Cymene |  | 2-Butoxyethanol |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC | WC | SC |
| RES_1 | 158 min | 120 min | 9.50 | 2.10 | 9.31 | 2.22 | 1.10 | 0.63 | 0.801 | 0.106 | 3.17 | 1.82 | 1.09 | <LOD | 0.0395 | <LOD | 0.170 | <LOD | 11.9 | 0.902 | 1.07 | 0.299 | 0.0589 | 0.625 |
| RES_2 | 168 min | 190 min | 13.0 | 0.266 | 53.2 | 16.2 | 8.17 | 3.05 | 8.21 | 0.301 | 23.4 | 9.14 | 9.01 | 1.96 | 2.93 | 0.454 | 3.94 | <LOD | 80.1 | 44.0 | 3.88 | 1.43 | 5.38 | 0.156 |
| RES_3 | 288 min | 190 min | 2.24 | 0.0299 | 5.48 | 0.181 | 1.22 | 2.02 | 0.729 | 0.0543 | 3.80 | 2.77 | <LOD | 1.28 | <LOD | <LOD | <LOD | 0.200 | 20.9 | 1.85 | <LOD | 0.102 | 35.4 | 2.59 |
| RES_4 | 126 min | 99 min | 3.93 | 0.417 | 4.98 | 1.40 | 2.38 | 1.34 | 1.32 | <LOD | 6.21 | 1.76 | <LOD | 0.392 | <LOD | <LOD | 2.47 | 0.0279 | 48.5 | 5.73 | 2.37 | <LOD | <LOD | 0.720 |
| RES_5 | 166 min | 226 min | 3.29 | 0.898 | 6.64 | 1.86 | 0.859 | 1.38 | 0.819 | <LOD | 3.37 | 2.38 | <LOD | 2.77 | <LOD | <LOD | 18.1 | 2.84 | 43.9 | 35.3 | 8.31 | 7.17 | <LOD | 19.6 |
| RES_6 | 299 min | 284 min | 4.65 | 1.83 | 25.7 | 13.1 | 3.87 | 1.80 | 4.66 | 2.06 | 10.7 | 5.90 | 4.97 | 2.08 | 0.433 | <LOD | 2.50 | 1.33 | 40.4 | 3.32 | 10.4 | 1.18 | <LOD | 1.04 |
| RES_7 | 111 min | 154 min | <LOD | 0.0534 | 36.4 | 0.351 | <LOD | 0.377 | <LOD | <LOD | <LOD | 0.665 | <LOD | 0.232 | <LOD | 0.363 | <LOD | 0.00777 | 9.94 | 6.83 | <LOD | 0.119 | 10.3 | 4.01 |
| RES_8 | 193 min | 144 min | 4.02 | 0.0169 | 13.2 | 18.1 | 2.49 | 0.554 | 1.89 | 0.0183 | 7.64 | 2.53 | 2.25 | 1.86 | 2.18 | <LOD | 1.55 | <LOD | 30.3 | 7.77 | 1.48 | 0.100 | 114 | 5.66 |
| RES_9 | 328 min | 150 min | 4.16 | 0.502 | 35.1 | 5.48 | 0.931 | 9.42 | 0.438 | 0.215 | 3.16 | 4.45 | <LOD | 2.70 | <LOD | 0.173 | 1.85 | 0.0912 | 37.2 | 66.2 | 3.74 | 7.98 | 7.65 | 5.41 |
| RES_10 | 232 min | 90 min | 13.3 | <LOD | <LOD | <LOD | <LOD | 0.132 | <LOD | <LOD | 0.0544 | 0.773 | 1.11 | 0.782 | <LOD | <LOD | 3.34 | <LOD | <LOD | 2.29 | <LOD | <LOD | 1.48 | 1.41 |
| RES_11 | 133 min | 145 min | n.a. | 1.39 | n.a. | 1.39 | n.a. | 2.41 | n.a. | 0.0578 | n.a. | 2.27 | n.a. | 2.70 | n.a. | 2.48 | n.a. | <LOD | n.a. | 4.77 | n.a. | 2.49 | n.a. | 16.9 |
| RES_12 | 200 min | 145 min | <LOD | 0.690 | <LOD | 12.1 | 13.5 | 1.46 | <LOD | 9.94 | <LOD | <LOD | 0.917 | 6.71 | <LOD | 0.358 | <LOD | <LOD | <LOD | 6.92 | <LOD | 0.68 | 51.2 | 91.2 |
| RES_13 | 133 min | 108 min | 0.212 | 0.677 | <LOD | 1.50 | 1.11 | 0.940 | <LOD | <LOD | 1.77 | 2.77 | <LOD | 1.82 | 0.252 | <LOD | <LOD | <LOD | 11.7 | 1.75 | 0.737 | <LOD | 6.80 | 0.431 |
| RES_14 | 213 min | 114 min | 1.45 | 0.116 | 4.31 | 0.165 | 5.83 | 2.77 | 1.41 | 0.123 | 4.64 | 2.43 | 1.46 | 0.263 | <LOD | <LOD | 0.187 | <LOD | 14.6 | 6.86 | <LOD | 0.579 | 2.42 | 0.601 |
| RES_15 | 212 min | 190 min | 14.0 | 3.53 | 8.62 | 0.804 | 1.58 | 0.133 | 0.708 | <LOD | 4.63 | 1.76 | 1.20 | 2.50 | 0.727 | <LOD | 0.732 | 1.79 | 27.5 | 35.6 | 3.97 | 4.32 | 4.31 | 2.08 |

LOD= Limit of detection; n.a. = not available

Table SM7- Indoor concentrations of aldehydes (ug/m3) for the winter (WC) and summer (SC) campaigns for each investigated restaurant, by time-integrated sampling.

| ID | Sampling time |  | Formaldehyde |  | Acetaldehyde |  | Acrolein |  |  | Propionaldehyde |  | Benzaldehyde |  | Glutaraldehyde |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WC | SC | WC | SC | WC | SC | WC | SC | WC |  | SC | WC | SC | WC | SC |
| RES_1 | 150 min | 120 min | 3.30 | 2.04 | 5.53 | 9.72 | 16.2 | 99.0 | <LOD |  | <LOD | 47.6 | 17.2 | <LOD | <LOD |
| RES_2 | 150 min | 150 min | 2.65 | 5.44 | 14.9 | 13.3 | 27.3 | 19.4 | 19.3 |  | 14.7 | 40.9 | 3.52 | <LOD | 1.50 |
| RES_3 | 200 min | 150 min | 6.51 | 2.72 | 60.0 | 20.6 | 52.3 | 19.9 | 36.9 |  | 66.6 | 2.28 | 8.79 | 1.00 | <LOD |
| RES_4 | 180 min | 100 min | 3.37 | 9.06 | 7.55 | 26.8 | 11.8 | 35.6 | 15.6 |  | 116 | 7.71 | 30.8 | 1.06 | <LOD |
| RES_5 | 150 min | 120 min | 11.4 | 2.99 | 18.4 | 8.48 | 77.5 | 41.9 | 66.6 |  | 6.83 | 13.2 | 37.4 | <LOD | 5.54 |
| RES_6 | 150 min | 150 min | 7.50 | 0.772 | 79.2 | 4.62 | 23.6 | 48.4 | 58.4 |  | 36.6 | 10.4 | 49.5 | 2.90 | 5.01 |
| RES_7 | 113 min | 150 min | 1.87 | 3.03 | 1.66 | 35.4 | 7.35 | 86.4 | 6.95 |  | <LOD | 16.7 | 5.71 | <LOD | <LOD |
| RES_8 | 150 min | 145 min | 2.27 | 1.22 | 11.8 | 5.13 | 7.23 | 30.6 | 46.6 |  | 48.8 | <LOD | 3.04 | <LOD | 3.28 |
| RES_9 | 240 min | 150 min | 17.6 | 1.89 | 34.9 | 9.55 | 82.2 | 22.9 | 78.2 |  | 46.6 | 10.5 | 5.54 | <LOD | 2.04 |
| RES_10 | 180 min | 87 min | 3.17 | <LOD | 3.57 | <LOD | 11.7 | 21.4 | 3.00 |  | <LOD | 9.51 | 6.17 | <LOD | 2.56 |
| RES_11 | 150 min | 148 min | 4.35 | 4.77 | 8.38 | 7.14 | 9.53 | 10.4 | 2.88 |  | 5.68 | 9.37 | 4.46 | <LOD | 5.32 |
| RES_12 | 184 min | 144 min | 7.79 | 6.33 | 15.0 | 10.0 | 19.7 | 25.4 | 16.6 |  | 16.8 | 19.6 | 5.09 | 3.08 | 2.84 |
| RES_13 | 131 min | 110 min | 20.4 | 15.2 | 52.4 | 45.4 | 25.1 | 17.1 | 3.11 |  | 2.93 | 22.7 | 21.4 | <LOD | <LOD |
| RES_14 | 150 min | 134 min | 0.364 | 4.67 | 0.907 | 14.0 | 2.81 | 18.6 | 0.375 |  | 61.4 | 3.32 | 16.3 | <LOD | <LOD |
| RES_15 | 206 min | 190 min | 6.23 | 5.71 | 5.68 | 2.87 | 11.47 | 21.28 | 5.87 |  | 26.26 | 21.3 | 28.8 | <LOD | 4.05 |

LOD= Limit of detection; n.a. = not available

Table SM8- Guide values of pollutants measured in restaurant kitchens with reference times and sources.

| Instruments | Pollutants | Long-term limit value | Short-term limit value | Source |
| --- | --- | --- | --- | --- |
| On-line | O3 | 100 µg/m3 (0.05 ppm) (8-h) | n.a. | WHO AQG 2021 1 |
|  | TVOCs | 0.3 mg/m3 (0.09 ppm) (8-h) | n.a. | AIR - German Committee on Indoor Air Guide Values |
|  | NO2 | 25 µg/m3 (0.013 ppm) (24-h) | 200 µg/m3 (0.11 ppm) (1-h) | WHO AQG 2021 1; WHO IAQG 2010 2 |
|  | CO2 | 1000 ppm |  | ASHRAE 62.1 |
| Off-line | Benzene | 5 µg/m3 (1-y) | n.a. | European Union Directive on air quality |
|  | Toluene | 0.3 mg/m3 (GV-I) | 3 mg/m3 (GV-II) | AIR - German Committee on Indoor Air Guide Values |
|  | Ethylbenzene | 0.2 mg/m3 (GV-I) | 2 mg/m3 (GV-II) | AIR - German Committee on Indoor Air Guide Values |
|  | Styrene | 0.03 mg/m3 (GV-I) | 0.3 mg/m3 (GV-II) | AIR - German Committee on Indoor Air Guide Values |
|  | 2-Butoxyethanol | 56 mg/m³ (8-h) | n.a. | DNELs for consumers 2 |
|  | α-Pinene | 4.5 mg/m3 (8-h) | 45 mg/m3 (1-h) | EU EPHECT project 2015 |
|  | d-Limonene | 9 mg/m3 (8-h) | 90 mg/m3 (1-h) | EU EPHECT project 2015 |
|  | p-Cymene | 0.22 mg/m³ (8-h) | n.a. | DNELs for consumers 2 |
|  | Formaldehyde | n.a. | 100 g/m3 (30 min) | WHO IAQG 2010 2 |
|  | Acetaldehyde | n.a. | 1.42 mg/m3 (1-h) | Air quality: UK guidelines for volatile organic compounds in indoor spaces |
|  | Acrolein | 10 g/m3 (8-h) | 21 g/m3 (1-h) | EU EPHECT project 2015 |
|  | Propionaldehyde | 6.1 mg/m³ (8-h) | n.a. | DNELs available for workers in industrial settings 2 |
|  | Benzaldehyde | 0.02 mg/m3 (GV-I) | 0.2 mg/m3 (GV-II) | AIR - German Committee on Indoor Air Guide Values |
|  | Glutaraldehyde | 0.2 mg/m³ (8-h) | n.a. | Air quality: UK guidelines for volatile organic compounds in indoor spaces |

1: World Health Organization Air Quality Guidelines

2: World Health Organization Indoor Air Quality Guidelines

3: Derived No-Effect Level

Table SM9- On-line winter concentrations of indoor pollutants for each investigated restaurant.

| ID | Sampling time1 | O3 (ppm) |  | TVOCs (ppm) |  | HCHO (ppm) |  | UFPs * 103 (pt/cm3) |  | NO2 (ppm) |  | CO2 (ppm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max |
| RES_1 | 300 min | 0.0055 (0.0025) | 0.00060 - 0.0105 | n.a. | n.a. | 0.0016 (0.0046) | <LOD - 0.033 | 28 (24) | 8.4 – 145 | 0.023 (0.019) | <LOD - 0.13 | 598 (130) | 512-1371 |
| RES_2 | 255 min | 0.0021 (0.00095) | <LOD - 0.0086 | 0.21 (0.058) | 0.12 - 0.42 | 0.0021 (0.00093) | 0.00062 - 0.0052 | 24 (13) | 6.8 – 70.4 | 0.014 (0.005) | <LOD - 0.028 | 919 (155) | 698-1215 |
| RES_3 | 365 min | 0.0047 (0.0019) | 0.0011 - 0.010 | 1.5 (1.0) | 0.14 - 6.9 | 0.0049 (0.0027) | 0.0011 - 0.013 | 21 (25) | 3.2 – 155 | 0.0047 (0.010) | <LOD - 0.10 | 956 (297) | 612-1902 |
| RES_4 | 315 min | 0.0031 (0.0012) | 0.00010 - 0.0095 | 0.24 (0.16) | <LOD - 0.96 | 0.0022 (0.0018) | <LOD - 0.010 | 37 (48) | 4.7 – 231 | 0.0061 (0.0083) | <LOD - 0.071 | 789 (66) | 681-946 |
| RES_5 | 246 min | 0.0048 (0.0036) | 0.0015 - 0.028 | 1.3 (0.80) | <LOD - 5.7 | 0.014 (0.012) | <LOD - 0.072 | 54 (31.5) | 7.3 – 153 | 0.080 (0.052) | <LOD - 0.25 | 714 (143) | 543-1261 |
| RES_6 | 395 min | 0.0042 (0.0022) | 0.0010 - 0.017 | n.a.. | n.a. | 0.0070 (0.0051) | 0.00021 - 0.029 | 60 (55) | 7.1 – 206 | 0.049 (0.036) | 0.001 - 0.21 | 811 (150) | 591-1248 |
| RES_7 | 207 min | 0.0087 (0.0048) | <LOD - 0.019 | 0.13 (0.18) | 0.010 - 1.2 | n.a. | n.a. | 18 (17) | 3.5 – 86.1 | 0.011 (0.016) | <LOD - 0.12 | 442 (24) | 413-615 |
| RES_8 | 290 min | 0.013 (0.011) | 0.0010 - 0.058 | 0.19 (0.086) | <LOD - 0.59 | 0.0012 (0.0012) | <LOD - 0.0060 | 86 (47) | 5.6 – 186.5 | 0.015 (0.022) | <LOD - 0.25 | 619 (46) | 458-701 |
| RES_9 | 211 min | 0.020 (0.015) | 0.0051 - 0.18 | 0.52 (0.21) | <LOD - 1.3 | 0.014 (0.015) | <LOD - 0.081 | 247 (85) | 106 – 498 | 0.18 (0.070) | <LOD - 0.25 | 969 (204) | 482-1532 |
| RES_10 | 317 min | 0.013 (0.0068) | 0.0037 - 0.055 | 0.22 (0.38) | <LOD - 2.7 | 0.0018 (0.0070) | <LOD - 0.071 | 72 (59) | 1.4 – 235 | 0.0086 (0.012) | <LOD - 0.11 | 445 (26) | 387-617 |
| RES_11 | 247 min | 0.012 (0.010) | 0.0010 - 0.075 | 0.32 (0.088) | 0.18 - 0.81 | 0.0030 (0.0024) | <LOD - 0.014 | 157 (43) | 14 – 218 | 0.045 (0.041) | <LOD - 0.18 | 728 (105) | 571-978 |
| RES_12 | 295 min | 0.010 (0.019) | <LOD - 0.17 | 0.53(0.40) | <LOD - 3.4 | 0.55 (0.40) | <LOD - 0.014 | 46 (39) | 2.5 – 158 | 0.023 (0.028) | <LOD - 0.18 | 489 (57) | 402-653 |
| RES_13 | 241 min | 0.012 (0.0055) | 0.0043 - 0.034 | 0.33 (0.47) | 0.020 - 3.1 | 0.013 (0.032) | <LOD - 0.27 | 35 (25) | 2.3 – 114 | 0.016 (0.029) | <LOD - 0.18 | 546 (105) | 421-1438 |
| RES_14 | 245 min | 0.0070 (0.0016) | 0.0037 - 0.011 | 0.17 (0.11) | <LOD - 0.53 | 0.00024 (0.00039) | <LOD - 0.0020 | 6.4 (14) | 1.7 – 139 | n.a. | n.a. | 539 (49) | 400-676 |
| RES_15 | 311 min | 0.016 (0.0041) | 0.0059 - 0.026 | 1.6 (1.2) | <LOD - 6.5 | 0.0046 (0.014) | <LOD - 0.096 | 102 (49) | 9.1 – 167 | 0.083 (0.042) | <LOD - 0.25 | 539 (31) | 480-669 |

LOD= Limit of detection; n.a. = not available

1 With the aim of accounting for short-term reference times of guide values (Table SM8), the sampling time here reported correspond to the time-averaging period for IAQ assessment in the case of O3, TVOCs and CO2, while 30-min and 1-h averages were calculated for HCHO and NO2.

Table SM10- On-line summer concentrations of indoor pollutants for each investigated restaurant.

| ID | Sampling time1 | O3 (ppm) |  | TVOCs (ppm) |  | HCHO (ppm) |  | UFPs * 103 (pt/cm3) |  | NO2 (ppm) |  | CO2 (ppm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max | Mean (SD) | Min-Max |
| RES_1 | 257 min | 0.022 (0.0050) | 0.010 - 0.039 | 0.14 (0.13) | 0.010 - 1.3 | 0.00093 (0.0022) | <LOD - 0.020 | 42 (36) | 5.4-154 | 0.012 (0.015) | <LOD - 0.096 | 460 (26) | 419-593 |
| RES_2 | 284 min | 0.0036 (0.0015) | <LOD -0.013 | 0.31 (0.37) | 0.15 - 2.9 | 0.018 (0.048) | <LOD - 0.40 | 26 (26) | 3.9-172 | 0.0022 (0.0045) | <LOD - 0.038 | 678 (79) | 556-833 |
| RES_3 | 399 min | 0.015 (0.0042) | 0.0077 - 0.034 | 0.68 (0.33) | 0.22 - 2.4 | 0.0026 (0.0013) | <LOD - 0.012 | 11 (13) | 2.8-86 | 0.0035 (0.0073) | <LOD - 0.065 | 685 (134) | 460-1155 |
| RES_4 | 249 min | 0.021 (0.0041) | 0.0078 - 0.028 | 0.16 (0.068) | 0.080 - 0.49 | 0.0029 (0.030) | <LOD - 0.37 | 8.7 (12) | 3.9-109 | 0.00084 (0.00077) | <LOD - 0.0030 | 557 (36) | 502-770 |
| RES_5 | 286 min | 0.0079 (0.0038) | 0.0010 - 0.024 | 2.1 (2.0) | 0.41 - 13 | 0.0026 (0.0025) | <LOD - 0.014 | 32 (27) | 5.4-110 | 0.083 (0.059) | <LOD - 0.23 | 729 (127) | 537-1276 |
| RES_6 | 435 min | 0.022 (0.0091) | 0.0040 - 0.048 | 0.65 (1.5) | 0.070 - 26 | 0.0012 (0.0030) | <LOD - 0.023 | 42 (52) | 5.7-310 | 0.039 (0.041) | <LOD - 0.25 | 569 (61) | 475-745 |
| RES_7 | 360 min | 0.016 (0.0043) | 0.0030 - 0.042 | 0.37 (0.35) | 0.010 - 2.9 | 0.0011 (0.0026) | <LOD - 0.022 | 4.7 (4.8) | 1.1-39 | 0.011 (0.017) | <LOD - 0.12 | n.a. | n.a. |
| RES_8 | 250 min | 0.015 (0.0063) | 0.0057 - 0.030 | 0.35 (0.22) | 0.11 - 1.5 | 0.00060 (0.0012) | <LOD - 0.0074 | 18 (8.5) | 5.8-49 | 0.016 (0.016) | <LOD - 0.11 | 662 (131) | 653-871 |
| RES_9 | 240 min | 0.020 (0.0076) | 0.0062 - 0.035 | 0.53 (0.32) | 0.13 - 1.7 | 0.0012 (0.0017) | <LOD - 0.0072 | 151 (62) | 56-370 | 0.099 (0.048) | 0.0060 - 0.25 | 830 (202) | 527-1212 |
| RES_10 | 258 min | 0.028 (0.0050) | <LOD - 0.063 | 0.11 (0.14) | 0.020 - 1.5 | 0.014 (0.048) | <LOD - 0.35 | 14 (16) | 2.3-151 | 0.018 (0.019) | <LOD - 0.087 | 481 (26) | 446-631 |
| RES_11 | 265 min | 0.019 (0.0075) | 0.0075 - 0.035 | 0.43 (0.54) | <LOD - 5.4 | 0.090 (0.26) | <LOD - 2.7 | 12 (15) | 1.0-111 | 0.085 (0.049) | <LOD - 0.21 | 582 (50) | 469-706 |
| RES_12 | 230 min | 0.017 (0.0062) | 0.0070 - 0.036 | 1.4 (0.62) | 0.37 - 4.0 | 0.0087 (0.0088) | <LOD - 0.060 | 30 (29) | 7.7-223 | 0.073 (0.078) | <LOD - 0.25 | 630 (68) | 492-862 |
| RES_13 | 253 min | 0.052 (0.0064) | 0.038 - 0.067 | 0.25 (0.47) | <LOD - 4.5 | 0.010 (0.044) | <LOD - 0.47 | 18 (21) | 4.7-125 | 0.010 (0.015) | <LOD - 0.14 | 495 (16) | 470-587 |
| RES_14 | 263 min | 0.023 (0.010) | 0.0051 - 0.046 | 0.18 (0.085) | 0.060 - 0.67 | 0.0038 (0.0060) | <LOD - 0.047 | 8.9 (17) | 2.4-165 | 0.011 (0.011) | <LOD - 0.074 | 585 (40) | 460-716 |
| RES_15 | 300 min | 0.021 (0.011) | 0.0063 - 0.059 | 2.6 (2.6) | 0.090 – 14 | 0.0050 (0.0051) | <LOD - 0.029 | 129 (93) | 16-416 | 0.067 (0.058) | <LOD - 0.25 | 553 (61) | 458-849 |

LOD= Limit of detection; n.a. = not available

1 With the aim of accounting for short-term reference times of guide values (Table SM8), the sampling time here reported correspond to the time-averaging period for IAQ assessment in the case of O3, TVOCs and CO2, while 30-min and 1-h averages were calculated for HCHO and NO2.

Table SM11 – Indoor and outdoor concentrations and I/O ratios of benzene, nitrogen dioxide and ozone per each investigated restaurant.

| ID | Benzene |  |  |  |  |  | Nitrogen Dioxide (NO2) |  |  |  |  |  | Ozone (O3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Indoor WC | Outdoor WC | I/O | Indoor SC | Outdoor SC | I/O | Indoor WC | Outdoor WC | I/O | Indoor SC | Outdoor SC | I/O | Indoor WC | Outdoor WC | I/O | Indoor SC | Outdoor SC | I/O |
| RES_1 | 9.5 | 1.8 | 5.3 | 2.1 | 0.84 | 2.5 | 0.023 | 0.026 | 0.9 | 0.012 | 0.003 | 4.5 | 0.005 | 0.011 | 0.5 | 0.022 | 0.049 | 0.5 |
| RES_2 | 13.0 | 4.4 | 3.0 | 0.27 | 0.46 | 0.6 | 0.014 | 0.045 | 0.3 | 0.002 | 0.012 | 0.2 | 0.002 | 0.003 | 0.7 | 0.004 | 0.058 | 0.1 |
| RES_3 | 2.2 | 1.1 | 2.0 | 0.030 | 0.42 | 0.1 | 0.005 | 0.024 | 0.2 | 0.004 | 0.009 | 0.4 | 0.005 | 0.016 | 0.3 | 0.015 | 0.070 | 0.2 |
| RES_4 | 3.9 | 2.0 | 2.0 | 0.42 | 0.68 | 0.6 | 0.006 | 0.033 | 0.2 | 0.001 | 0.001 | 0.9 | 0.003 | 0.006 | 0.5 | 0.021 | 0.059 | 0.4 |
| RES_5 | 3.3 | 2.1 | 1.6 | 0.90 | 0.62 | 1.4 | 0.080 | 0.029 | 2.8 | 0.083 | n.a. | n.a. | 0.005 | 0.022 | 0.2 | 0.008 | 0.060 | 0.1 |
| RES_6 | 4.6 | 1.9 | 2.4 | 1.8 | 1.0 | 1.9 | 0.049 | 0.016 | 3.1 | 0.039 | 0.004 | 10 | 0.004 | 0.017 | 0.2 | 0.022 | 0.087 | 0.3 |
| RES_7 | <LOD | 1.2 | < 1 | 0.053 | 1.1 | 0.05 | 0.011 | 0.012 | 0.9 | 0.011 | 0.003 | 3.8 | 0.009 | 0.026 | 0.3 | 0.016 | 0.053 | 0.3 |
| RES_8 | 4.0 | 1.7 | 2.4 | 0.017 | 0.78 | 0.02 | 0.015 | 0.040 | 0.4 | 0.016 | 0.009 | 1.7 | 0.013 | 0.018 | 0.7 | 0.015 | 0.081 | 0.2 |
| RES_9 | 4.2 | 0.68 | 6.1 | 0.50 | 0.60 | 0.8 | 0.180 | 0.017 | 11 | 0.099 | 0.008 | 12 | 0.020 | 0.035 | 0.6 | 0.020 | 0.076 | 0.3 |
| RES_10 | 13.3 | 0.72 | 18 | <LOD | 0.74 | < 1 | 0.009 | 0.010 | 0.9 | 0.018 | 0.010 | 1.8 | 0.013 | 0.042 | 0.3 | 0.028 | 0.043 | 0.7 |
| RES_11 | n.a. | 0.82 | n.a. | 1.4 | 1.0 | 1.3 | 0.045 | 0.013 | 3.5 | 0.085 | 0.005 | 18 | 0.012 | 0.029 | 0.4 | 0.019 | 0.067 | 0.3 |
| RES_12 | <LOD | 1.2 | < 1 | 0.69 | 0.68 | 1.0 | 0.023 | 0.017 | 1.4 | 0.073 | 0.020 | 3.6 | 0.010 | 0.029 | 0.3 | 0.017 | 0.092 | 0.2 |
| RES_13 | 0.21 | 1.3 | 0.2 | 0.68 | 0.62 | 1.1 | 0.016 | 0.007 | 2.3 | 0.010 | 0.005 | 2.3 | 0.012 | 0.048 | 0.2 | 0.052 | 0.085 | 0.6 |
| RES_14 | 1.5 | 0.90 | 1.6 | 0.12 | 0.74 | 0.2 | n.a. | 0.013 | n.a. | 0.011 | 0.005 | 2.3 | 0.007 | 0.045 | 0.2 | 0.023 | 0.070 | 0.3 |
| RES_15 | 14.0 | 1.0 | 13 | 3.5 | 0.50 | 7.1 | 0.083 | 0.019 | 4.4 | 0.067 | 0.006 | 11 | 0.016 | 0.056 | 0.3 | 0.021 | 0.060 | 0.3 |

LOD= Limit of detection; n.a. = not available