Supplementary Materials

To conduct the agent-based simulation, the Mesa- python model was used on Jupyter Notebook. The following code demonstrates an example of running the code for case A for natural gas allocation to a network configuration comprising power, LNG, ammonia, and urea. The model prioritises allocating natural gas to meet the domestic power requirement prior to allocating surplus natural gas capacity for overall profitability maximisation. Additionally, the agent-based simulation optimises the allocation strategy based on 10 time periods (annual) considering forecasted input data for 2024-2033. However, some sample data has been used wherever applicable.

- instaling Mesa on Jupyter notebook

pip install -U mesa

- importing required packages

from random import random

#packages associated with agent based modelling

from mesa import Model

from mesa import Agent

# for data collection

from mesa.datacollection import DataCollector

#for agent activation

from mesa.time import RandomActivation

import pandas as pd

import numpy as np

- Defining agents and the model

- # Define the process agent

- #the agents for downstream processes

- #Always start with super().__init__() to initialize the model object properly

- class ProcessAgent(Agent):

- def __init__(self, unique_id, model, process_name, conversion_factor):

- # Pass the parameters to the parent class

- super().__init__(unique_id, model)

- self.process_name = process_name

- self.conversion_factor = conversion_factor

- # Create the agent's variable and set the initial values

- self.allocated_gas = 0

- self.produced_amount = 0

- def step(self):

- # Calculate production output based on the allocated gas and conversion factor

- self.production_output = self.allocated_gas * self.conversion_factor

- # Define the natural gas market model

- #done by creating a new class that extends mesa.Model and calls super().__init__()

- class NaturalGasMarketModel(Model):

- def __init__(self, global_demand, global_prices, process_costs, operational_constraints, total_gas_list, conversion_factors):

- self.global_demand = global_demand

- self.global_prices = global_prices

- self.process_costs = process_costs

- self.operational_constraints = operational_constraints

- # Assign the total_gas list to self.total_gas_list

- self.total_gas_list = total_gas_list

- self.conversion_factors = conversion_factors

- # Create scheduler and assign it to the model

- self.schedule = RandomActivation(self)

- self.annual_results=[]

- # Creating agents for each process

- for i, process_name in enumerate(global_demand[0].keys()):

- agent = ProcessAgent(i, self, process_name, conversion_factors[process_name])

- self.schedule.add(agent)

- # DataCollector for recording results

- self.datacollector = DataCollector(

- model_reporters={

- "Profitability": self.calculate_profitability,

- "Gas Allocations": lambda m: {agent.process_name: agent.allocated_gas for agent in m.schedule.agents},

- "Production Capacities": lambda m: {agent.process_name: agent.allocated_gas * agent.conversion_factor for agent in m.schedule.agents},

- }

- )

- def step(self,year):

- self.current_year=year #track current year

- # set total gas available for the current year

- self.total_gas = self.total_gas_list[year]  # Use the gas capacity for the current year

- self.datacollector.collect(self)

- # Allocate gas to meet power demand first

- power_agent = [agent for agent in self.schedule.agents if agent.process_name == "Power"][0]

- power_demand = self.global_demand[year]["Power"]

- power_agent.allocated_gas = min(power_demand, self.total_gas)

- self.total_gas -= power_agent.allocated_gas

- # Ensure each process meets its minimum threshold

- for agent in self.schedule.agents:

- if agent.process_name != "Power":

- min_gas_required = self.operational_constraints[agent.process_name]['min']

- max_gas_available = min(self.operational_constraints[agent.process_name]['max'], self.total_gas)

- agent.allocated_gas = max(min_gas_required, max_gas_available)

- self.total_gas -= agent.allocated_gas

- # Allocate remaining gas to maximize profitability

- for agent in sorted(self.schedule.agents, key=lambda x: self.global_prices[year][x.process_name], reverse=True):

- max_gas = min(self.total_gas, self.operational_constraints[agent.process_name]['max'] - agent.allocated_gas)

- agent.allocated_gas += max_gas

- self.total_gas -= max_gas

- # Calculate profitability

- #self.calculate_profitability(year)

- #update the schedule

- self.schedule.step()

- def calculate_profitability(self):

- total_profit = 0

- for agent in self.schedule.agents:

- production = agent.allocated_gas * agent.conversion_factor

- revenue = production * self.global_prices[self.current_year][agent.process_name]

- cost = agent.allocated_gas * self.process_costs[self.current_year][agent.process_name]

- total_profit += (revenue - cost)

- return total_profit

4. Define the input data required to run the model

global_demand = [

{'LNG': 381.4774, 'Ammonia': 297.8, 'Power': 709.289, 'Urea': 178.83},

{'LNG': 393.6356, 'Ammonia': 321.62, 'Power': 734.409, 'Urea': 180.48},

{'LNG': 405.788, 'Ammonia': 347.35, 'Power': 759.529, 'Urea': 182.14},

{'LNG': 417.945, 'Ammonia': 375.14, 'Power': 784.649, 'Urea': 183.81},

{'LNG': 430.102, 'Ammonia': 405.15, 'Power': 809.769, 'Urea': 188.22},

{'LNG': 442.258, 'Ammonia': 437.56, 'Power': 834.889, 'Urea': 192.64},

{'LNG': 454.415, 'Ammonia': 472.56, 'Power': 860.009, 'Urea': 197.05},

{'LNG': 466.571, 'Ammonia': 510.37, 'Power': 885.129, 'Urea': 205.87},

{'LNG': 466.571, 'Ammonia': 551.20, 'Power': 910.249, 'Urea': 210.28},

{'LNG': 490.884, 'Ammonia': 595.30, 'Power': 935.369, 'Urea': 214.69},

]

global_prices = [

{'LNG': 2.46E+9, 'Ammonia': 1.15E+9, 'Power': 5.724E+7, 'Urea': 4.48E+8},

{'LNG': 2.51E+9, 'Ammonia': 1.19E+9, 'Power': 5.724E+7, 'Urea': 4.58E+8},

{'LNG': 2.57E+9, 'Ammonia': 1.23E+9, 'Power': 5.724E+7, 'Urea': 4.69E+8},

{'LNG': 2.62E+9, 'Ammonia': 1.26E+9, 'Power': 5.724E+7, 'Urea': 4.80E+8},

{'LNG': 2.68E+9, 'Ammonia': 1.30E+9, 'Power': 5.724E+7, 'Urea': 4.91E+8},

{'LNG': 2.73E+9, 'Ammonia': 1.34E+9, 'Power': 5.72E+7, 'Urea': 5.01E+8},

{'LNG': 2.79E+9, 'Ammonia': 1.37E+9, 'Power': 5.724E+7, 'Urea': 5.12E+8},

{'LNG': 2.84E+9, 'Ammonia': 1.41E+9, 'Power': 5.724E+7, 'Urea': 5.23E+8},

{'LNG': 2.90E+9, 'Ammonia': 1.45E+9, 'Power': 5.724E+7, 'Urea': 5.34E+8},

{'LNG': 2.96E+9, 'Ammonia': 1.48E+9, 'Power': 5.724E+7, 'Urea': 5.44E+8},

]

process_costs = [

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 4.48E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 4.58E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 4.69E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 4.80E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 4.91E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 5.01E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 5.12E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 5.23E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 5.34E+8},

{'LNG': 1.84E+9, 'Ammonia': 1.33E+9, 'Power': 2.89E+7, 'Urea': 5.44E+8},

]

operational_constraints = {

'LNG': {'min': 6.91, 'max': 1000},

'Ammonia': {'min': 3.353, 'max': 1000},

'Power': {'min': 0, 'max': 10000},

'Urea': {'min': 3.19, 'max': 1000},

}

total_gas_list= [157.04, 161.51, 165.98, 170.44, 174.91, 179.34, 182.15, 184.93, 187.70, 190.48]

conversion_factors = {

'LNG': 0.724,     # 0.724 tonnes of LNG per tonne of NG

'Ammonia': 1.133, # 1.133 tonnes of Ammonia per tonne of NG

'Power': 10.2528,   #  1 kWh of Power (energy) per million tonne of NG per yr

'Urea': 1.2     #  1.2 tonnes of Urea per tonne of NG

}

5. Run the model and print results

# create the model

model= NaturalGasMarketModel(

global_demand=global_demand,

global_prices=global_prices,

process_costs=process_costs,

operational_constraints=operational_constraints,

total_gas_list=total_gas_list,

conversion_factors=conversion_factors)

#Run the model for 10 years

for year in range(10):

model.step(year)

# Collect results

results = model.datacollector.get_model_vars_dataframe()

# Print results

print(results)