Electronic Supplementary Information (ESI)

LCA of Precious Metals Recovery: Modelling the Secondary Supply of Gold, Silver, Platinum, Palladium and Rhodium from an Integrated Refining Plant

Cespi, D.1,2, Ciacci, L.1,2,*, Chiarini, A.3, Trova, E.3, Acciai, C.3, Ciabatti, I.3, Tirinnanzi, M.3, Passarini, F.1,2

1Department of Industrial Chemistry “Toso Montanari”, University of Bologna, via Gobetti 85, 40129 Bologna, Italy

2Interdepartmental Centre for Industrial Research – Renewable Resources, Environment, Sea and Energy, University of Bologna, via D. Campana 71, 47923, Rimini, Italy

3TCA S.p.A., Z. I. Castelluccio, 11, 52010 Capolona, Italy

*Corresponding author: luca.ciacci5@unibo.it (L.C.)

Table S 1: Excluded material flows.

| Material/Substance | Amount | Unit |
| --- | --- | --- |
| Sheet metal cutting discs | 800.0 | n° pieces* |
| Electrodes, stainless steel | 800.0 | n° pieces* |
| Membrane for nanofilters | 99.0 | n° pieces* |
| Slickt ru (commercial name) | 180.0 | kg** |
| Ramsat (commercial name) | 3750.0 | kg** |
| Insulator power | 170.0 | kg** |
| Anti-scaling | 250.0 | kg** |
| Crucibles, ceramic | 8257.0 | n° pieces* |
| Crucibles, graphite | 238.0 | n° pieces* |
| Crucibles, SiC | 378.0 | n° pieces* |
| Crucibles, zirconia | 16.0 | n° pieces* |
| Crucible zirconia, shirt | 13.0 | n° pieces* |

* Quantity (kg) consumed unknown.

**Exact composition unknown.

Table S 2: Amount of precious metals refined in 2022.

| Metal | Quantity (kg) |
| --- | --- |
| Gold | 36,820,185.17 |
| Silver | 87,909,886.01 |
| Palladium | 2,013,280.72 |
| Platinum | 784,595.41 |
| Rhodium | 253,020.41 |

Table S 3: List of proxies used for completing the LCI.

| Chemical substance | ecoinvent process (proxy) | Amount (kg) | note |
| --- | --- | --- | --- |
| Methyl orange | Chemical, organic {GLO}¦ market for ¦ APOS, U | 3 | RER process not available. |
| Disavil (liquid ligand) | Silica sand {GLO}¦ market for ¦ APOS, U (20%) | 812 | Is a “silica aqueous fluid" (page 28: https://www.rhimagnesita.com/wp-content/uploads/2017/12/Bulletin-2017-1-171130-Hyperlink_mon.pdf) RER process not available, in the case of silica sand. |
|  | water, ultrapure {RER}¦ water production, ultrapure ¦ APOS, U (80% | 3248 |  |
| Sodium metabisulfite | Chemical, inorganic {GLO}¦ market for chemicals, inorganic ¦ APOS, U | 71475 | RER process not available. |
| Heptadecafluorooctanesulfonic acid | Chlorosulfonic acid {RER}¦ market for chlorosulfonic acid ¦ APOS, U | 20 | - |
| Dichloromaleic anhydride (3,4-dichlorofuran-2,5-dione) | Maleic anhydride {GLO}¦ market for maleic anhydride ¦ APOS, U | 4 | RER process not available. |
| Potassium iodine | Chemical, inorganic {GLO}¦ market for chemicals, inorganic ¦ APOS, U | 5 | RER process not available. |
| Dimethylcarbamate | [thio]carbamate-compound {GLO}¦ market for ¦ APOS, U | 5000 | RER process not available. |
| Dimethylglyoxime | Chemical, organic {GLO}¦ market for ¦ APOS, U | 0.5 | RER process not available. |
| Iron (II) chloride | Iron (III) chloride, without water, in 40% solution state {GLO}¦ market for ¦ APOS, U | 20820 | RER process not available. |
| Ethylenediamine dichlorate | Ethylenediamine {GLO}¦ market for ¦ APOS, U | 1 | RER process not available. |

Table S 4: LCI for the transportation of the waste generated up to the first destination plant.

| First destination | Amount (t) | km from TCA plant (taken from ecotransit) | tkm | ecoinvent proxy | Operation |
| --- | --- | --- | --- | --- | --- |
| Waste manager_1 | 1630 | 472 | 770194 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_2 | 450 | 221 | 99203 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_3 | 998 | 417 | 416473 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D15 |
| Waste manager_4 | 6035 | 129 | 780174 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_5 | 493 | 79 | 38801 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_6 | 734 | 369 | 270614 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D15 |
| Waste manager_7 | 327 | 479 | 156989 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_8 | 57 | 377 | 21637 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_9 | 671 | 119 | 79939 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_10 | 327 | 429 | 140196 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | D9 |
| Waste manager_11 | 9 | 30 | 270 | Transport, freight, lorry 7.5-16 metric ton, euro6 {RER}¦ market for transport, freight, lorry 7.5-16 metric ton, EURO6 ¦ APOS, U | D15 |
| Waste manager_12 | 442 | 1321 | 583869 | Transport, freight, lorry 16-32 metric ton, euro6 {RER}¦ market for transport, freight, lorry 16-32 metric ton, EURO6 ¦ APOS, U | R4 |

Table S 5: LCI for the waste final treatment.

| CER | Description | Type | Amount (t) | distance | tkm | ecoinvent proxy for final treatment |
| --- | --- | --- | --- | --- | --- | --- |
| 060314 | Salts and their solutions, other than those mentioned in 06 03 11 and 06 03 13 | liquid | 5496 | 100 | 549624 | Wastewater, average {Europe without Switzerland} ¦ market for wastewater, average ¦ APOS, U |
| 100808 | Savage slag from primary and secondary production | solid | 1119 | 100 | 111864 | Inert waste, for final disposal {CH}¦ treatment of inert waste, inert material landfill ¦ APOS, U |
| 110112 | Aqueous rinsing solutions other than those mentioned in 10 01 11 | liquid | 849 | 100 | 84930 | Wastewater, average {Europe without Switzerland} ¦ market for wastewater, average ¦ APOS, U |
| 161002 | Aqueous liquid wastes other than those mentioned in 16 10 01 | liquid | 4267 | 100 | 426747 | Wastewater, average {Europe without Switzerland} ¦ market for wastewater, average ¦ APOS, U |
| 110109* | Sludge and filter residues, containing dangerous substances (rich in copper) | solid | 442 | - | - | Copper cake {GLO}¦ treatment of copper cake ¦ APOS, U |

* Added average distance of 100km to reach the final treatment plant (according to https://eplca.jrc.ec.europa.eu/permalink/PEFCR_guidance_v6.3-2.pdf)

** The waste is directly processed at UMICORE N.V.

Table S 6: LCIA contribution on the whole metal refining process. Method: ReCiPe 2016 Midpoint (H) V1.08 / World (2010) H.

| Flow type | I | II | III | IV | V | VI | VII |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Impact category | Plant emissions, water consumption and movement (core) | Chemicals (upstream) | Energy carriers (upstream) | Consumables (big-bags, Alukor, Compac, concrete, etc.) (upstream) | Gases (upstream) | Waste transportation (downstream) | Waste treatment (downstream) |
| Global warming | 23.03% | 20.61% | 26.85% | 2.49% | 20.16% | 5.22% | 1.64% |
| Stratospheric ozone depletion | 0.01% | 89.89% | 5.09% | 0.31% | 3.60% | 0.79% | 0.18% |
| Ionizing radiation | 0.08% | 9.97% | 17.16% | 0.38% | 71.08% | 0.65% | 0.60% |
| Ozone formation, Human health | 25.32% | 21.69% | 23.21% | 3.88% | 20.04% | 3.49% | 1.85% |
| Fine particulate matter formation | 4.83% | 11.96% | 7.81% | 2.11% | 13.81% | 1.31% | 57.98% |
| Ozone formation, Terrestrial ecosystems | 42.05% | 16.13% | 19.36% | 2.88% | 14.90% | 2.88% | 1.38% |
| Terrestrial acidification | 22.70% | 8.93% | 5.60% | 1.49% | 7.65% | 0.67% | 52.86% |
| Freshwater eutrophication | 0.06% | 22.16% | 16.95% | 4.10% | 52.88% | 1.27% | 2.40% |
| Marine eutrophication | 0.05% | 18.89% | 28.51% | 1.35% | 39.75% | 1.40% | 9.83% |
| Terrestrial ecotoxicity | 0.00% | 2.11% | 3.13% | 0.88% | 1.96% | 19.14% | 89.89% |
| Freshwater ecotoxicity | 0.05% | 30.42% | 23.18% | 7.67% | 26.67% | 3.62% | 7.87% |
| Marine ecotoxicity | 0.03% | 18.78% | 16.06% | 5.43% | 17.47% | 3.43% | 38.33% |
| Human carcinogenic toxicity | 0.03% | 14.93% | 16.10% | 33.16% | 12.71% | 4.22% | 18.36% |
| Human non-carcinogenic toxicity | 0.01% | 6.19% | 3.92% | 1.17% | 6.91% | 1.48% | 80.71% |
| Land use | 0.04% | 19.13% | 37.38% | 4.31% | 28.88% | 6.99% | 2.27% |
| Mineral resource scarcity | 1.36% | 40.62% | 13.04% | 30.33% | 10.15% | 3.45% | 0.54% |
| Fossil resource scarcity | 0.04% | 24.51% | 14.91% | 5.17% | 50.28% | 1.48% | 3.41% |
| Water consumption | 3.85% | 14.54% | 26.55% | 0.55% | 56.69% | 0.51% | -2.77% |

Table S 7: Energy consumption in 2022, per energetic vector.

| Energetic vector | Unit |  |  |  |  |  | Amount | Unit | Contribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electricity | - | - | - | - | - | - | 6,976,029 | kWh | 24.2% |
| Natural gas | m3 | 2120389 | 0.777* | kg/m3 | 13.10* | kWh/kg | 21,582,804 | kWh | 74.9% |
| Diesel | l | 24590 | 0.846* | kg/l | 11.83* | kWh/kg | 246,101 | kWh | 0.9% |
| Propane (GPL) | kg | 1125 |  |  | 12.88* | kWh/kg | 14,490 | kWh | 0.1% |
| Total Energy consumption |  |  |  |  |  |  | 28,819,424 | kWh |  |

* data source: https://www.engineeringtoolbox.com/fuels-higher-calorific-values-d_169.html

Table S 8: Emission profile of working vehicles, Scope 1. Total 2022.

| Substance | Amount | Unit |
| --- | --- | --- |
| Carbon dioxide, fossil | 71984.0a | kg |
| Carbon monoxide, fossil | 495.0a | kg |
| Nitrogen oxides, IT | 230.0a | kg |
| Particulates, < 10 um (mobile), remote, low-stack | 5.0a | kg |
| Hydrocarbons, unspecified | 217.0a | kg |

aEstimated from Komatsu by using the annual environmental declaration and reported working hours by TCA.

Table S 9: Plant emission profile, Scope 1. Total 2022.

| Substance | Amount | Unit |
| --- | --- | --- |
| Particulates, < 10 um (stationary), high-stack | 235.1 | kg |
| Organic carbon | 699.4 | kg |
| Carbon monoxide, fossil | 26.0 | kg |
| Nitrogen oxides, IT | 6562.6 | kg |
| Sulfur oxides, IT | 580.5 | kg |
| Hydrogen chloride | 297.3 | kg |
| Ammonia | 154.6 | kg |
| Hydrogen fluoride | 9.2 | kg |
| Mercury | 0.1 | kg |
| Metals, unspecified | 12.0 | kg |
| Nitrogen dioxide, IT | 907.6 | kg |
| PAH, polycyclic aromatic hydrocarbons | 3.0E-03 | kg |
| Carbon dioxide, fossil | 3.8a | kton |

aCO2 emissions from LPG and LNG, estimated from the total C mass balance and C reacted to CO from the emissions register.

Table S 10: Contribution analysis of nitric acid, liquid oxygen and aluminum oxide for selected impact categories.

| Nitric acid, without water, in 50% solution state {RER}¦ market for nitric acid, without water, in 50% solution state ¦ APOS, U | Stratospheric ozone depletion | Ionizing radiation |  | Freshwater eutrophication |  | Human carcinogenic toxicity |  | Mineral resources scarcity |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Platinum |  | 9.3% |  | 48.0% |  | 12.3% |  | 66.6% |  |
| Ammonia |  | 43.1% |  | 20.0% |  | 37.3% |  | 13.4% |  |
| Chemical factory |  | 34.2% |  | 31.7% |  | 53.6% |  | 21.6% |  |
| Nitrogen liquid |  | 8.0% |  |  |  |  |  |  |  |
| N2O emission in air | 98.0% |  |  |  |  |  |  |  |  |
| Avoided impacts (due to APOS model) |  |  |  |  |  | -3.2% |  | -1.6% |  |
| Other processes | 2.0% | 5.4% |  | 0.3% |  | 0.0% |  | 0.0% |  |
| Oxygen, liquid {RER}¦ market for ¦ APOS, U | Stratospheric ozone depletion |  | Ionizing radiation |  | Freshwater eutrophication |  | Human carcinogenic toxicity |  | Mineral resources scarcity |
| Electricity | 99.5% |  | 99.9% |  | 99.7% |  | 95.7% |  | 98.3% |
| Trasportation |  |  |  |  |  |  |  |  | 1.1% |
| Air separation factory |  |  |  |  |  |  | 2.20% |  |  |
| Other processes | 0.500% |  | 0.1% |  | 0.3% |  | 2.1% |  | 0.6% |
| Aluminium oxide {GLO}¦ market for ¦ APOS, U | Stratospheric ozone depletion |  | Ionizing radiation |  | Freshwater eutrophication |  | Human carcinogenic toxicity |  | Mineral resources scarcity |
| Bayer process including bauxite extraction | 73% |  | 75% |  | 90% |  | 91.30% |  | 95% |
| Trasportation | 10% |  | 12.60% |  |  |  |  |  |  |
| Other processes | 17.00% |  | 12.40% |  | 10.00% |  | 8.70% |  | 5.00% |

Table S 11: Mass and economic allocation values used in the study.

|  | Amount refined in 2022 (g) | 2014-2024 average price (€/g) | 2014-2024 standard deviation (€/g) | Total per metal (€) in 2022 | Economic allocation |
| --- | --- | --- | --- | --- | --- |
| Gold | 36,820,185.17 | 0.425 | 0.028 | 2,225,780,193.39 € | 91.26% |
| Silver | 87,909,886.01 | 0.005 | 0.002 | 6,536,920.20 € | 2.52% |
| Palladium | 2,013,280.72 | 0.521 | 0.207 | 54,700,837.19 € | 2.24% |
| Platinum | 784,595.41 | 0.178 | 0.095 | 45,129,928.07 € | 1.85% |
| Rhodium | 253,020.41 | 1.550 | 1.350 | 51,869,184.66 € | 2.13% |
| Total | 127,780.97 | - | - | 2,439,017,063.52 € | 100% |

Figure S1: Contribution to environmental impacts of II + III stages (Chemicals + Energy carriers) and II + III + V stages (Chemicals + Energy carriers + Gases). Method: ReCiPe 2016

Table S 12: Values for the total carbon footprint (t CO2-eq/kg metal) by considering different data sources. Method: IPCC 2021 GWP 100 (CO2 uptake, v. 1.02).

|  | ecoinvent database |  | Nuss&Eckelman, 2014 |  |  | This study |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Metal ​ | Primary (from ores) ​ | Primary (market mean)​ | Secondary (mean) | Secondary (min) | Secondary (max) | Secondary (mean) | Secondary (standard deviation) |
| Gold ​ | 17.2 | 50.0 | 0.92 | 0.35 | 2.26 | 0.43 | 0.028 |
| Silver ​ | 0.3 | 0.5 | 0.02 | 0.01 | 0.04 | 0.01 | 0.002 |
| Palladium ​ | 13.5 | 13.0 | 0.18 | 0.13 | 0.24 | 0.54 | 0.27 |
| Platinum ​ | 32.8 | 68.7 | 0.63 | 0.47 | 0.85 | 0.18 | 0.096 |
| Rhodium ​ | 56.9 | 80.9 | 1.78 | 1.32 | 2.40 | 1.55 | 1.35 |

Table S 13: Values for the total cumulative energy demand (GJ/kg metal) by considering different data sources. Method: CED (v. 1.12).

|  | Nuss&Eckelman, 2014 |  |  | This study |  |
| --- | --- | --- | --- | --- | --- |
| Metal | Secondary (mean) | Secondary (min) | Secondary (max) | Secondary (mean) | Secondary (standard deviation) |
| Gold | 208.00 | 165.00 | 263.00 | 7.59 | 0.51 |
| Silver | 3.28 | 2.74 | 3.95 | 0.09 | 0.04 |
| Palladium | 72.70 | 55.50 | 95.50 | 3.08 | 1.82 |
| Platinum | 243.00 | 178.00 | 326.00 | 9.36 | 3.79 |
| Rhodium | 683.00 | 504.00 | 911.00 | 27.58 | 25.07 |

Table S 14: Uncertainty analysis through the Monte Carlo method ReCiPe 2016 Midpoint (H) V1.08 / World (2010) H: Characterization.

| Impact category | Unit | Mean | Median | SD* | CV** | 2.5% | 97.5% | SEM*** |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fine particulate matter formation | kg PM2.5 eq | 4.0E+04 | 3.8E+04 | 9.5E+03 | 2.4E+01 | 2.6E+04 | 6.2E+04 | 3.0E+02 |
| Fossil resource scarcity | kg oil eq | 9.1E+05 | 8.9E+05 | 1.7E+05 | 1.9E+01 | 6.6E+05 | 1.3E+06 | 5.4E+03 |
| Freshwater ecotoxicity | kg 1,4-DCB | 5.6E+05 | 5.4E+05 | 1.2E+05 | 2.2E+01 | 4.0E+05 | 8.4E+05 | 3.9E+03 |
| Freshwater eutrophication | kg P eq | 5.3E+03 | 4.8E+03 | 2.0E+03 | 3.9E+01 | 2.7E+03 | 1.1E+04 | 6.4E+01 |
| Global warming | kg CO2 eq | 1.7E+07 | 1.7E+07 | 8.5E+05 | 5.0E+00 | 1.5E+07 | 1.9E+07 | 2.7E+04 |
| Human carcinogenic toxicity | kg 1,4-DCB | 3.6E+06 | 3.4E+06 | 1.1E+06 | 3.0E+01 | 2.0E+06 | 5.9E+06 | 3.4E+04 |
| Human non-carcinogenic toxicity | kg 1,4-DCB | 5.6E+07 | 5.5E+07 | 1.0E+08 | 1.8E+02 | -1.3E+08 | 2.6E+08 | 3.2E+06 |
| Ionizing radiation | kBq Co-60 eq | 1.9E+06 | 1.1E+06 | 2.5E+06 | 1.3E+02 | 2.1E+05 | 8.0E+06 | 7.8E+04 |
| Land use | m2a crop eq | 4.4E+05 | 4.4E+05 | 9.6E+04 | 2.2E+01 | 2.6E+05 | 6.5E+05 | 3.0E+03 |
| Marine ecotoxicity | kg 1,4-DCB | 1.3E+06 | 1.2E+06 | 4.0E+05 | 3.1E+01 | 8.4E+05 | 2.3E+06 | 1.3E+04 |
| Marine eutrophication | kg N eq | 5.7E+02 | 5.6E+02 | 6.6E+01 | 1.2E+01 | 4.5E+02 | 7.1E+02 | 2.1E+00 |
| Mineral resource scarcity | kg Cu eq | 5.7E+04 | 5.7E+04 | 6.0E+03 | 1.0E+01 | 4.7E+04 | 7.0E+04 | 1.9E+02 |
| Ozone formation, Human health | kg NOx eq | 3.4E+04 | 3.4E+04 | 1.9E+03 | 5.5E+00 | 3.1E+04 | 3.8E+04 | 6.0E+01 |
| Ozone formation, Terrestrial ecosystems | kg NOx eq | 4.8E+04 | 4.7E+04 | 2.5E+03 | 5.2E+00 | 4.3E+04 | 5.3E+04 | 7.8E+01 |
| Stratospheric ozone depletion | kg CFC11 eq | 4.4E+01 | 4.3E+01 | 8.6E+00 | 2.0E+01 | 3.0E+01 | 6.4E+01 | 2.7E-01 |
| Terrestrial acidification | kg SO2 eq | 1.5E+05 | 1.4E+05 | 3.3E+04 | 2.2E+01 | 1.0E+05 | 2.3E+05 | 1.0E+03 |
| Terrestrial ecotoxicity | kg 1,4-DCB | 1.1E+09 | 8.8E+08 | 8.8E+08 | 7.9E+01 | 3.7E+08 | 3.3E+09 | 2.8E+07 |
| Water consumption | m3 | 7.8E+05 | 9.1E+05 | 1.3E+07 | 1.7E+03 | -2.9E+07 | 2.4E+07 | 4.1E+05 |

*SD = standard deviation

**CV = coefficient of variation (values in %)

***SEM = Standard error of the mean

Table S 15: Uncertainty analysis through the Monte Carlo method ReCiPe 2016 Endpoint (H/A) V1.08 / World (2010) H: Damage assessment.

| Damage category | Unit | Mean | Median | SD* | CV** | 2.5% | 97.5% | SEM*** |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ecosystems | species.yr | 1.0E-01 | 1.1E-01 | 6.3E-02 | 6.2E+01 | -3.2E-02 | 2.2E-01 | 2.0E-03 |
| Human health | DALY | 6.6E+01 | 6.6E+01 | 3.4E+01 | 5.2E+01 | -4.5E+00 | 1.3E+02 | 1.1E+00 |
| Resources | USD2013 | 6.8E+04 | 6.6E+04 | 1.2E+04 | 1.8E+01 | 5.0E+04 | 9.6E+04 | 3.8E+02 |

*SD = standard deviation

**CV = coefficient of variation (values in %)

***SEM = Standard error of the mean

Table S 16: Uncertainty analysis through the Monte Carlo method ReCiPe 2016 Endpoint (H/A) V1.08 / World (2010) H: Single score.

| Damage category | Unit | Mean | Median | SD* | CV** | 2.5% | 97.5% | SEM*** |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ecosystems | Pt | 2.8E+04 | 2.9E+04 | 1.7E+04 | 6.2E+01 | -8.7E+03 | 5.9E+04 | 5.4E+02 |
| Human health | Pt | 1.1E+06 | 1.1E+06 | 5.7E+05 | 5.2E+01 | -7.4E+04 | 2.2E+06 | 1.8E+04 |
| Resources | Pt | 4.9E+02 | 4.7E+02 | 8.7E+01 | 1.8E+01 | 3.6E+02 | 6.9E+02 | 2.7E+00 |

*SD = standard deviation

**CV = coefficient of variation (values in %)

***SEM = Standard error of the mean

Table S 17: Distances from suppliers, for the seven material flows selected.

| Substance | Amount | Unit | Supplier | Distance sup. 1 (km) | Distance sup. 2 (km) | Allocation sup. 1 (%) | Allocation sup. 2 (%) | tkm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oxygen | 4.20E+06 | kg | Supplier_1 (60%); Supplier_2 (40%) | 2192 | 26 | 60% | 40% | 5.57E+06 |
| Sodium hydroxide | 6.10E+05 | kg | Supplier_3 (100%) | 32 | 0 | 100% | 0% | 1.97E+04 |
| Soda ash | 4.54E+05 | kg | Supplier_4 (100%) | 190 | 0 | 100% | 0% | 8.63E+04 |
| Nitric acid | 4.06E+05 | kg | Supplier_5 (90%);Supplier_3 (10%) | 90 | 32 | 90% | 10% | 3.43E+04 |
| Hydrochloric acid | 3.38E+05 | kg | Supplier_5 (50%); Supplier_6 (50%) | 90 | 359 | 50% | 50% | 7.59E+04 |
| Refractory material (COMPAC) | 2.54E+05 | kg | Supplier_7.1 (50%) Supplier_7.2 (50%) | 1394 | 1173 | 50% | 50% | 3.26E+05 |
| Lime hydrated | 1.30E+05 | kg | Supplier_8 (100%) | 129 | 0 | 100% | 0% | 1.68E+04 |

Table S 18: Sensitivity analysis method ReCiPe 2016 Midpoint (H) V1.08 / World (2010) H: Characterization.

| Impact category | Unit | Total | Other processes* | Transp. Oxygen | Transp. Sodium hydroxide | Transp. Soda ash | Transp. Nitric acid | Transp. Hydrochloric acid | Transp. Refractory material (COMPAC) | Transp. Lime hydrated | Transp. Sum | Contribution to total impact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GW | kg CO2 eq | 1.8E+07 | 1.7E+07 | 8.7E+05 | 3.1E+03 | 1.4E+04 | 5.4E+03 | 1.2E+04 | 5.1E+04 | 2.6E+03 | 9.6E+05 | 5% |
| ODP | kg CFC11 eq | 4.4E+01 | 4.3E+01 | 2.0E-01 | 7.1E-04 | 3.1E-03 | 1.2E-03 | 2.8E-03 | 1.2E-02 | 6.1E-04 | 2.2E-01 | 1% |
| IRP | kBq Co-60 eq | 1.9E+06 | 1.9E+06 | 1.4E+04 | 5.1E+01 | 2.2E+02 | 8.9E+01 | 2.0E+02 | 8.4E+02 | 4.3E+01 | 1.6E+04 | 1% |
| HOFP | kg NOx eq | 3.8E+04 | 3.4E+04 | 4.2E+03 | 1.5E+01 | 6.4E+01 | 2.6E+01 | 5.7E+01 | 2.4E+02 | 1.3E+01 | 4.6E+03 | 12% |
| PMFP | kg PM2.5 eq | 4.1E+04 | 4.0E+04 | 9.6E+02 | 3.4E+00 | 1.5E+01 | 5.9E+00 | 1.3E+01 | 5.6E+01 | 2.9E+00 | 1.1E+03 | 3% |
| EOFP | kg NOx eq | 5.2E+04 | 4.7E+04 | 4.4E+03 | 1.5E+01 | 6.8E+01 | 2.7E+01 | 6.0E+01 | 2.6E+02 | 1.3E+01 | 4.8E+03 | 9% |
| TAP | kg SO2 eq | 1.5E+05 | 1.5E+05 | 2.1E+03 | 7.6E+00 | 3.3E+01 | 1.3E+01 | 2.9E+01 | 1.3E+02 | 6.4E+00 | 2.4E+03 | 2% |
| FEP | kg P eq | 5.3E+03 | 5.3E+03 | 7.7E+01 | 2.7E-01 | 1.2E+00 | 4.8E-01 | 1.1E+00 | 4.5E+00 | 2.3E-01 | 8.5E+01 | 2% |
| MEP | kg N eq | 5.7E+02 | 5.6E+02 | 9.1E+00 | 3.2E-02 | 1.4E-01 | 5.6E-02 | 1.2E-01 | 5.3E-01 | 2.7E-02 | 1.0E+01 | 2% |
| TETP | kg 1,4-DCB | 1.1E+09 | 1.1E+09 | 2.3E+07 | 8.1E+04 | 3.5E+05 | 1.4E+05 | 3.1E+05 | 1.3E+06 | 6.9E+04 | 2.5E+07 | 2% |
| FETP | kg 1,4-DCB | 5.8E+05 | 5.6E+05 | 2.2E+04 | 7.9E+01 | 3.5E+02 | 1.4E+02 | 3.1E+02 | 1.3E+03 | 6.8E+01 | 2.5E+04 | 4% |
| METP | kg 1,4-DCB | 1.3E+06 | 1.3E+06 | 4.9E+04 | 1.7E+02 | 7.5E+02 | 3.0E+02 | 6.6E+02 | 2.8E+03 | 1.5E+02 | 5.3E+04 | 4% |
| HTPc | kg 1,4-DCB | 3.7E+06 | 3.5E+06 | 1.6E+05 | 5.8E+02 | 2.5E+03 | 1.0E+03 | 2.2E+03 | 9.6E+03 | 4.9E+02 | 1.8E+05 | 5% |
| HTPnc | kg 1,4-DCB | 6.3E+07 | 6.2E+07 | 6.8E+05 | 2.4E+03 | 1.1E+04 | 4.2E+03 | 9.3E+03 | 4.0E+04 | 2.1E+03 | 7.5E+05 | 1% |
| LOP | m2a crop eq | 4.9E+05 | 4.4E+05 | 4.4E+04 | 1.5E+02 | 6.8E+02 | 2.7E+02 | 5.9E+02 | 2.6E+03 | 1.3E+02 | 4.8E+04 | 10% |
| SOP | kg Cu eq | 5.9E+04 | 5.7E+04 | 2.2E+03 | 7.7E+00 | 3.4E+01 | 1.3E+01 | 3.0E+01 | 1.3E+02 | 6.6E+00 | 2.4E+03 | 4% |
| FFP | kg oil eq | 9.3E+05 | 9.1E+05 | 1.5E+04 | 5.4E+01 | 2.4E+02 | 9.5E+01 | 2.1E+02 | 9.0E+02 | 4.6E+01 | 1.7E+04 | 2% |
| WCP | m3 | 3.0E+05 | 3.0E+05 | 1.9E+03 | 6.6E+00 | 2.9E+01 | 1.1E+01 | 2.5E+01 | 1.1E+02 | 5.6E+00 | 2.0E+03 | 1% |

*It includes flows (chemical, gases, emission, transportation, waste management, etc.) not covered elsewhere. Global Warming (GW), Stratospheric ozone depletion (ODP), Ionizing radiation (IRP), Ozone formation, human health (HOFP), Fine particulate matter formation (PMFP), Ozone formation, terrestrial ecosystems (EOFP), Terrestrial acidification (TAP), Freshwater eutrophication (FEP), Marine eutrophication (MEP), Terrestrial ecotoxicity (TETP), Freshwater ecotoxicity (FETP), Marine ecotoxicity (METP), Human carcinogenic toxicity (HTPc), Human non-carcinogenic toxicity (HTPnc), Land use (LOP), Mineral resource scarcity (SOP), Fossil resource scarcity (FFP), and Water use (WCP).

Komatsu, 2023. Dichiarazione Ambientale, Komatsu. Retrieved from https://www.komatsu.eu/it/dati-ambientali