Earth’s Future

Supporting Information for

Leverage Points and Cascading Impacts Analysis in Nexus Systems using System Dynamics Modelling

Virginia R. Coletta1, Irene Pluchinotta2, Vassilios Pisinaras3, Andreas Panagopoulos3, Raffaele Giordano1, Alessandro Pagano4, Umberto Fratino4, and Alberto Montanari5

1 Water Research Institute - National Research Council, Bari, IT

2 Institute for Environmental Design and Engineering, The Bartlett Faculty of the Built Environment, University College London, London, UK

3 Soil and Water Resources Institute, Hellenic Agricultural Organization “DIMITRA”, Thessaloniki, GR

4 Department of Civil, Environmental, Land, Construction and Chemistry, Polytechnic University of Bari, Bari, IT

5 Department of Civil, Chemical, Environmental, and Materials Engineering, Alma Mater Studiorum University of Bologna, Bologna, IT

Contents of this file

S1 to S6

Introduction

This section includes additional details related to mathematical perspective on eigenvector centrality, MICMAC (Cross-Impact Matrix Multiplication Applied to Classification) and LTM (Loops That Matter) (see Sections 1,2,3); dependence and influence scores of key factors in the PRB (Pinios River Basin) (Section 4); variables and mathematical equations behind the SF (Stock and Flow) model focused on the PRB (Section 5); ranges and probability distributions for the Uncertainty Analysis (Section 6).

S1 Mathematical perspective on eigenvector centrality

The mathematical process for calculating eigenvector centrality from CLDs typically starts by representing the CLD as an adjacency matrix A (see Newman, 2008 for further details). Each element Aij of the matrix indicates the presence and direction of a relationship from node i to node j. The centrality of node i, denoted as xi, is proportional to the sum of the centralities of i's neighbours, following the principle that connections to more influential nodes increase a node's influence more (see Eq. (1)).

xi= 1λ j=1nAij xj      (1)

The term λ in Eq. (1) is a normalization factor, ensuring the vector x remains bounded. Writing the vector of centralities as x = (x1, x2 …), the equation can be expressed in matrix form (see Eq. (2)).

λx=A ∙x    (2)

The final vector x contains the eigenvector centrality values, indicating the relative influence of each node in the CLD.

S2 Mathematical perspective on MICMAC (Cross-Impact Matrix Multiplication Applied to Classification)

A ‘classic’ Impact Matrix M={mij} represents the direct influence of element i on element j. Influence shows the cumulative impact of a factor, while dependence reflects its susceptibility to other factors. They are normalized between 0 and 1 (Walters et al., 2022). The Direct Influence vector DIi and the Direct Dependence vector DDi are calculated as follows (see Eq. (3) and (4)):

DIi = jmij    (3)

DDi = jmji    (4)

However, since 'classic' Impact Matrices account only for direct relationships between system elements and ignore indirect ones (i.e., feedback effects through paths and loops), MICMAC manipulates and analyses 'classic' Impact Matrices to assess both direct and indirect importance of key factors in complex systems (Heilgeist et al., 2022). For clarity, assume that a 'classic' Impact Matrix M contains only zeros and ones, meaning the intensity of relationships is not considered. Each element mij indicates the presence of an influence (a path of length 1) from variable i to variable j. By raising matrix M to the power of n, the element at the intersection of row i and column j represents the number of paths of length n connecting these two variables (see Eq. (5)):

Mn =M × M(n-1)   (5)

The MICMAC method calculates the matrix M raised to successive powers (1, 2, ..., up to n) until the ranking of variables’ influence and dependence stabilizes. This process results in a new Impact Matrix, where each element represents the number of propagation paths (of length ≤ n) indicating both direct and indirect influences from variable i to variable j (Arcade et al., 1999). The Total Influence TIi and Total Dependence TDi are then calculated as follows (see Eq. (6) and (7)):

TIi = n(Mn )ij   (6)

TDi = n(Mn )ji   (7)

To identify the specific roles of key factors in the Nexus system, the variables in the CLD are projected onto an influence-dependence chart based on their Total Influence and Total Dependence scores (Nazlabadi et al., 2022).

S3 Mathematical perspective on LTM (Loops That Matter)

LTM computes three metrics to measure loop dominance: the link score, loop score, and relative loop score (Schoenberg et al., 2020). The link score measures the contribution and polarity of a causal link between variables. The link score for the link xz in the equation z=f(x,y) is defined as (see Eq. (8)):

LS(xàz) =ΔzxΔz×sign ΔzxΔx    if Δz≠0 and Δx≠0 0                                   if  Δz=0 and Δx=0        (8)

where Δz represents the change in z from the previous to the current time, Δx is the change in x over the same time step, and Δzx is the change in z with respect to x.

The loop score represents the combined contribution and polarity of a loop, calculated as the product of all link scores within that loop. The loop score for a loop Lx containing n links through n variables is defined as (see Eq. (9)):

Loop score (Lx) = LSv1→v2×LSv2→v3…×LS(svn-1→vn)×LS(svn→v1) (9)

The relative loop score normalizes the loop score across all loops to identify dominant loops at any given time, indicating the polarity and fractional contribution of a feedback loop to changes in connected stocks (Schoenberg et al., 2023). The relative loop score for loop Lx normalized over all m loops is (see Eq. (10)):

Relative loop score (Lx)=Loop score Lxy=0mLoop score Ly    (10)

By combining relative loop scores with the System Change metric, analysts can identify which links and feedback loops are most influential during key periods of substantial change, enabling more effective leverage point identification for intervention. The System Change metric is defined by the following equations (Eq. (11)):

Accumulated Stock Change (As)=Time=StartStopⅆSⅆtΔT    (11)

Raw System Change (R) =ALL STOCKSⅆSⅆt1As                (12)

System Change Over Time = RmaxR                            (13)

As shown in Eq. (12), the net flow for a stock ⅆS/ⅆt at a given time is compared to its total accumulation over the period As, ensuring that only significant changes contribute to the system change metric. Eq. (13) normalizes these values to a score between 0 and 1, aiding in the identification of critical periods of system change.

S4 Normalized dependence and influence scores of key factors in the Pinios River Basin, with values ranging from 0 (lowest) to 1 (highest)

| KEY FACTORS DRIVING SYSTEM OUTCOME | MICMAC DEPENDENCE | MICMAC INFLUENCE |
| --- | --- | --- |
| Community well-being | 0.866 | 0 |
| Soil quality | 0.813 | 0 |
| Flora and fauna conservation | 0.647 | 0 |
| Soil pollution | 0.805 | 0 |
| Irrigated areas | 0.964 | 0.823 |
| Agricultural productivity | 0.809 | 0.881 |
| Economic sustainability of agriculture | 0.964 | 0 |
| Cultivated areas | 0.672 | 0.881 |
| Water demand for irrigation | 1 | 1 |
| Tourism | 0.67 | 0 |
| Use of chemicals and fertilizers | 0.869 | 0.926 |
| Nitrate pollution | 0.855 | 0.801 |
| GW (groundwater) quality | 0.784 | 0.889 |
| GW demand for irrigation | 0.783 | 0.626 |
| GW use for irrigation | 0.763 | 0.797 |
| GW level | 0.765 | 0.83 |
| GW salinization | 0.81 | 0.763 |
| SW (surface water) demand for irrigation | 0.896 | 0.773 |
| SW use for irrigation | 0.788 | 0.627 |
| SW quality | 0.795 | 0.754 |
| SW availability for agriculture | 0.842 | 0.818 |
| GW cost | 0.81 | 0.888 |

S5 Variables and mathematical equations behind the SF (Stock and Flow) model focused on the Pinios River Basin

Run Specs

Start Time	0

Stop Time	360

DT	1

Fractional DT	False

Save Interval	1

Sim Duration	0

Time Units	Month

Pause Interval	0

Integration Method	Euler

Total	Count

Variables	214

Modules	4

Stocks	21

Flows	37

Converters	156

Constants	75

Equations	118

| Array Dimension | Elements |  |
| --- | --- | --- |
| Area | Delta, Agia |  |
| Crop | Pome_fruit_trees, Stone_fruit_trees, Cereals, Corn, Nuts, Forage, Cotton, Olives, Other_trees, Other_crops |  |
| Irrigated | Pome_fruit_trees, Stone_fruit_trees, Cereals, Corn, Nuts, Forage, Cotton, Olives, Other_trees, Other_crops |  |
| Landuse | Urban, Agricultural, Natural, Other_landuse |  |
| Month | Jan, Feb, Mar, Apr, May, Jun, Jul, Aug, Sep, Oct, Nov, Dec |  |
| Nutrients | N, P, K |  |
| Use | Drinking, Irrigation, Ecosystem, Other |  |
| Variable |  | Equation | Properties | Units |
| Agricultural_areas[Crop, Area](t) |  | Agricultural_areas[Crop, Area](t - dt) + (AA_increase_rate[Crop, Area] - AA_decrease_rate[Crop, Area]) * dt | INIT Agricultural_areas[Crop, Area] = Initial_crops_area[Crop, Area] | ha |
| Agricultural_profitability[Crop, Area](t) |  | Agricultural_profitability[Crop, Area](t - dt) + (AP_increase_rate[Crop, Area] - AP_decrease_rate[Crop, Area]) * dt | INIT Agricultural_profitability[Crop, Area] = 0.2 | Dmnl |
| Desertification_risk[Area](t) |  | Desertification_risk[Area](t - dt) + (DR_increase_rate[Area] - DR_decrease_rate[Area]) * dt | INIT Desertification_risk[Area] = Initial_Desertification_Risk[Area] | Dmnl |
| Livestock_and_Pasture_Areas[Area](t) |  | Livestock_and_Pasture_Areas[Area](t - dt) + (LPA_increase_rate[Area] - LPA_decrease_rate[Area]) * dt | INIT Livestock_and_Pasture_Areas[Area] = Initial_Livestock_and_Pasture_Areas[Area] | ha |
| AA_decrease_rate[Crop, Area] |  | 0 |  | ha/Month |
| AA_increase_rate[Crop, Area] |  | 0*(1-"Landuse_planning_regulation/actions") |  | ha/Month |
| AP_decrease_rate[Crop, Area] |  | Crop_profitability[Crop]*WATER."Frequency_and_intensity_of_extremes_(droughts_and_floods)"*(1-Favorable_Market_conditions)*Land_fragmentation*Production_cost[Area] |  | Dmnl/Month |
| AP_increase_rate[Crop, Area] |  | Crop_profitability[Crop]*(1+Agricultural_productivity[Crop, Area]*Development_of_consortia[Area]*Favorable_Market_conditions)*Access_to_Funding |  | Dmnl/Month |
| DR_decrease_rate[Area] |  | ( IF "N_load_-_Area"[Area] > 0.5 AND Desertification_risk[Area] < 1 THEN 0 ELSE 0.001*"N_load_-_Area"[Area]-"Trend_AA_-_sum"[Area] ) |  | Dmnl/Month |
| DR_increase_rate[Area] |  | ( IF "N_load_-_Area"[Area] > 0.5 AND Desertification_risk[Area] < 1 THEN 0.001*"N_load_-_Area"[Area]+"Trend_AA_-_sum"[Area] ELSE 0 ) |  | Dmnl/Month |
| LPA_decrease_rate[Area] |  | 0 |  | ha/Month |
| LPA_increase_rate[Area] |  | 0 |  | ha/Month |
| Access_to_Funding |  | 0.1 |  | Dmnl |
| Accessibility_to_irrigation_water |  | 1 |  | Dmnl |
| Actual_contribution_of_capillary_rise[Crop, Area] |  | ( IF WATER.CC_scenario_YN = 0 THEN Potential_contribution_of_capillary_rise[Crop, Area] ELSE 0.88*Potential_contribution_of_capillary_rise[Crop, Area] ) |  | Dmnl |
| Agricultural_productivity[Crop, Area] |  | ( IF Average_unit_crop_yield[Crop] > "Target_-_unit_crop_yield"[Crop] THEN 1 ELSE Average_unit_crop_yield[Crop]/"Target_-_unit_crop_yield"[Crop]*((Innovation_in_agricultural_practice+ECOSYSTEMS.Soil_organic_matter[Area]+Farmers'_training_and_awareness*TIME/360+ECOSYSTEMS.Soil_quality[Area])/4) ) |  | Dmnl |
| Agricultural_sustainability[Crop, Area] |  | (Agricultural_productivity[Crop, Area]+Crop_profitability[Crop]*(1+0.3*Access_to_Funding-0.2*Land_fragmentation-0.5*Production_cost[Area]))/2 |  | Dmnl |
| Application_efficiency |  | 0.85 |  | Dmnl |
| Average_agricultural_sustainability[Area] |  | SUM(Agricultural_sustainability[*, Area])/10 |  | Dmnl |
| Average_crop_unit_water_need[Pome_fruit_trees, Delta] |  | 4200 |  | m3/ha |
| Average_crop_unit_water_need[Pome_fruit_trees, Agia] |  | 5580 |  | m3/ha |
| Average_crop_unit_water_need[Stone_fruit_trees, Delta] |  | 4200 |  | m3/ha |
| Average_crop_unit_water_need[Stone_fruit_trees, Agia] |  | 4667 |  | m3/ha |
| Average_crop_unit_water_need[Cereals, Delta] |  | 0 |  | m3/ha |
| Average_crop_unit_water_need[Cereals, Agia] |  | 0 |  | m3/ha |
| Average_crop_unit_water_need[Corn, Delta] |  | 4813 |  | m3/ha |
| Average_crop_unit_water_need[Corn, Agia] |  | 4167 |  | m3/ha |
| Average_crop_unit_water_need[Nuts, Delta] |  | 3300 |  | m3/ha |
| Average_crop_unit_water_need[Nuts, Agia] |  | 3667 |  | m3/ha |
| Average_crop_unit_water_need[Forage, Delta] |  | 4903 |  | m3/ha |
| Average_crop_unit_water_need[Forage, Agia] |  | 4167 |  | m3/ha |
| Average_crop_unit_water_need[Cotton, Delta] |  | 2650 |  | m3/ha |
| Average_crop_unit_water_need[Cotton, Agia] |  | 2208 |  | m3/ha |
| Average_crop_unit_water_need[Olives, Delta] |  | 1130 |  | m3/ha |
| Average_crop_unit_water_need[Olives, Agia] |  | 0 |  | m3/ha |
| Average_crop_unit_water_need[Other_trees, Delta] |  | 10462 |  | m3/ha |
| Average_crop_unit_water_need[Other_trees, Agia] |  | 0 |  | m3/ha |
| Average_crop_unit_water_need[Other_crops, Delta] |  | 3662 |  | m3/ha |
| Average_crop_unit_water_need[Other_crops, Agia] |  | 0 |  | m3/ha |
| Average_irrigation_water_budget |  | SUM(Irrigation_water_budget[*])/3 |  | Dmnl |
| Average_unit_crop_yield[Pome_fruit_trees] |  | 27470 |  | kg/ha |
| Average_unit_crop_yield[Stone_fruit_trees] |  | 4812 |  | kg/ha |
| Average_unit_crop_yield[Cereals] |  | 4000 |  | kg/ha |
| Average_unit_crop_yield[Corn] |  | 11018 |  | kg/ha |
| Average_unit_crop_yield[Nuts] |  | 1291 |  | kg/ha |
| Average_unit_crop_yield[Forage] |  | 3000 |  | kg/ha |
| Average_unit_crop_yield[Cotton] |  | 3000 |  | kg/ha |
| Average_unit_crop_yield[Olives] |  | 3000 |  | kg/ha |
| Average_unit_crop_yield[Other_trees] |  | 24931 |  | kg/ha |
| Average_unit_crop_yield[Other_crops] |  | 43000 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Pome_fruit_trees] |  | 43 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Stone_fruit_trees] |  | 43 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Cereals] |  | 52 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Corn] |  | 250 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Nuts] |  | 43 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Forage] |  | 8 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Cotton] |  | 112 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Olives] |  | 43 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Other_trees] |  | 43 |  | kg/ha |
| Baseline_fertilizers_application_rate[N, Other_crops] |  | 220 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Pome_fruit_trees] |  | 15 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Stone_fruit_trees] |  | 15 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Cereals] |  | 13 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Corn] |  | 24 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Nuts] |  | 15 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Forage] |  | 12 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Cotton] |  | 20 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Olives] |  | 15 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Other_trees] |  | 15 |  | kg/ha |
| Baseline_fertilizers_application_rate[P, Other_crops] |  | 76 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Pome_fruit_trees] |  | 14 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Stone_fruit_trees] |  | 14 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Cereals] |  | 0 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Corn] |  | 7 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Nuts] |  | 14 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Forage] |  | 10 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Cotton] |  | 30 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Olives] |  | 14 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Other_trees] |  | 14 |  | kg/ha |
| Baseline_fertilizers_application_rate[K, Other_crops] |  | 163 |  | kg/ha |
| Cost_of_water[Area] |  | (Impact_of_energy_cost_on_water_cost[Area]+(1-Irrigation_efficiency[Area])-"S2-Irrigation_programming")/3 |  | Dmnl |
| Crop_profitability[Crop] |  | (Unit_crop_prof[Crop]/Months_per_year)/(Target_crop_profitability[Crop]/Months_per_year)*(1+0.2*Favorable_Market_conditions-0.4*(1-"N_load_-_Crop"[Crop])) |  | Dmnl |
| Desertification_risk_of_agricultural_areas[Area] |  | ((1-WATER.SW_quality[Area])+(1-WATER.GW_quality[Area])+(1-"Landuse_planning_regulation/actions")+"N_load_-_Area"[Area]+WATER."Frequency_and_intensity_of_extremes_(droughts_and_floods)")/5 |  | Dmnl |
| Development_of_consortia[Area] |  | 0.2 |  | Dmnl |
| Environmental_labelling_schemes |  | 0 |  | Dmnl |
| Farmers'_training_and_awareness |  | 0 |  | Dmnl |
| Favorable_Market_conditions |  | 0.5 |  | Dmnl |
| Fertilizers_application_rate[Nutrients, Crop] |  | Baseline_fertilizers_application_rate[Nutrients, Crop]*(1-(0.2*Farmers'_training_and_awareness)-(0.2*Environmental_labelling_schemes)-(0.1*"Landuse_planning_regulation/actions")) |  | kg/ha |
| Fertilizers_load[Nutrients, Crop] |  | ( IF Fertilizers_application_rate[Nutrients, Crop] > Recommended_fertilizers_application_rate[Nutrients, Crop] THEN 1 ELSE Fertilizers_application_rate[Nutrients, Crop]/Recommended_fertilizers_application_rate[Nutrients, Crop] ) |  | Dmnl |
| Fertilizers_use_efficiency[N] |  | 0.5 |  | Dmnl |
| Fertilizers_use_efficiency[P] |  | 0.5 |  | Dmnl |
| Fertilizers_use_efficiency[K] |  | 0.5 |  | Dmnl |
| Impact_of_CC_on_unit_water_need |  | ( IF WATER.CC_scenario_YN = 0 THEN 1 ELSE 1.06 ) |  | Dmnl |
| Impact_of_energy_cost_on_water_cost[Area] |  | (1-WATER."SW/GW_uses_-_area"[Irrigation, Area])*(1-(WATER.GW_availability[Area]/WATER.GW_storage[Area])) |  | Dmnl |
| Initial_crops_area[Pome_fruit_trees, Delta] |  | 1.7 |  | ha |
| Initial_crops_area[Pome_fruit_trees, Agia] |  | 630 |  | ha |
| Initial_crops_area[Stone_fruit_trees, Delta] |  | 12.7 |  | ha |
| Initial_crops_area[Stone_fruit_trees, Agia] |  | 254 |  | ha |
| Initial_crops_area[Cereals, Delta] |  | 706.9 |  | ha |
| Initial_crops_area[Cereals, Agia] |  | 150 |  | ha |
| Initial_crops_area[Corn, Delta] |  | 398.9 |  | ha |
| Initial_crops_area[Corn, Agia] |  | 3.7 |  | ha |
| Initial_crops_area[Nuts, Delta] |  | 50.6 |  | ha |
| Initial_crops_area[Nuts, Agia] |  | 75.7 |  | ha |
| Initial_crops_area[Forage, Delta] |  | 863 |  | ha |
| Initial_crops_area[Forage, Agia] |  | 41.8 |  | ha |
| Initial_crops_area[Cotton, Delta] |  | 70.8 |  | ha |
| Initial_crops_area[Cotton, Agia] |  | 0.001 |  | ha |
| Initial_crops_area[Olives, Delta] |  | 112.7 |  | ha |
| Initial_crops_area[Olives, Agia] |  | 26.1 |  | ha |
| Initial_crops_area[Other_trees, Delta] |  | 596 |  | ha |
| Initial_crops_area[Other_trees, Agia] |  | 78.9 |  | ha |
| Initial_crops_area[Other_crops, Delta] |  | 136.3 |  | ha |
| Initial_crops_area[Other_crops, Agia] |  | 28.1 |  | ha |
| Initial_Desertification_Risk[Delta] |  | 0.7 |  | Dmnl |
| Initial_Desertification_Risk[Agia] |  | 0.7 |  | Dmnl |
| Initial_Livestock_and_Pasture_Areas[Delta] |  | 2000 |  | ha |
| Initial_Livestock_and_Pasture_Areas[Agia] |  | 500 |  | ha |
| Innovation_in_agricultural_practice |  | 0 |  | Dmnl |
| Institutional_coordination |  | 0 |  | Dmnl |
| Irrigation_efficiency[Delta] |  | 0.65 |  | Dmnl |
| Irrigation_efficiency[Agia] |  | 0.8 |  | Dmnl |
| Irrigation_unit_water_demand[Crop, Area] |  | Impact_of_CC_on_unit_water_need*Irrigation_water_demand_coefficient*((1-Reduction_of_irrigation_water_needs_due_to_NBS[Crop, Area])*Average_crop_unit_water_need[Crop, Area]*(1-(Actual_contribution_of_capillary_rise[Crop, Area])))*(1-0.15*(Innovation_in_agricultural_practice)-0.05*(Monitoring)-0.15*(Farmers'_training_and_awareness)) |  | m3/(Month*ha) |
| Irrigation_water_budget[Area] |  | ( IF Water_availability_for_irrigation[Area] > Irrigation_water_use_per_Area[Area] THEN 1 ELSE Water_availability_for_irrigation[Area]/Irrigation_water_use_per_Area[Area] ) |  | Dmnl |
| Irrigation_water_demand_coefficient |  | ( IF WATER.Month_of_the_year = 1 THEN 0 ELSE ( IF WATER.Month_of_the_year = 2 THEN 0 ELSE ( IF WATER.Month_of_the_year = 3 THEN 0 ELSE ( IF WATER.Month_of_the_year = 4 THEN 0 ELSE ( IF WATER.Month_of_the_year = 5 THEN 0.07 ELSE ( IF WATER.Month_of_the_year = 6 THEN 0.2 ELSE ( IF WATER.Month_of_the_year = 7 THEN 0.3 ELSE ( IF WATER.Month_of_the_year = 8 THEN 0.3 ELSE ( IF WATER.Month_of_the_year = 9 THEN 0.13 ELSE ( IF WATER.Month_of_the_year = 10 THEN 0 ELSE 0 ) ) ) ) ) ) ) ) ) ) |  | Dmnl |
| Irrigation_water_use[Crop, Area] |  | Agricultural_areas[Crop, Area]*Irrigation_unit_water_demand[Crop, Area]*(2-Irrigation_efficiency[Area])*WATER."Conversion_m3/Mm3" |  | Mm3 |
| Irrigation_water_use_per_Area[Area] |  | SUM(Irrigation_water_use[*, Area]) |  | Mm3 |
| Land_fragmentation |  | 0.6*(1-"Landuse_planning_regulation/actions") |  | Dmnl |
| "Landuse_-_Area"[Urban, Delta] |  | 1 |  | ha |
| "Landuse_-_Area"[Urban, Agia] |  | 168.7 |  | ha |
| "Landuse_-_Area"[Agricultural, Delta] |  | 1 |  | ha |
| "Landuse_-_Area"[Agricultural, Agia] |  | 2127 |  | ha |
| "Landuse_-_Area"[Natural, Delta] |  | 1 |  | ha |
| "Landuse_-_Area"[Natural, Agia] |  | 2955 |  | ha |
| "Landuse_-_Area"[Other_landuse, Delta] |  | 1 |  | ha |
| "Landuse_-_Area"[Other_landuse, Agia] |  | 1 |  | ha |
| "Landuse_planning_regulation/actions" |  | 0.5 |  | Dmnl |
| "Livestock_capacity_-_actual"[Delta] |  | 2 |  | animal/ha |
| "Livestock_capacity_-_actual"[Agia] |  | 1 |  | animal/ha |
| "Livestock_capacity_-_threshold" |  | 2 |  | animal/ha |
| Livestock_impacts[Area] |  | ( IF "Livestock_capacity_-_actual"[Area] > 2*"Livestock_capacity_-_threshold" THEN 1 ELSE "Livestock_capacity_-_actual"[Area]/"Livestock_capacity_-_threshold" )*(Livestock_and_Pasture_Areas[Area]/Initial_Livestock_and_Pasture_Areas[Area]) |  | Dmnl |
| Monitoring |  | 0 |  | Dmnl |
| Months_per_year |  | 12 |  | Month |
| "N_load_-_Area"[Area] |  | SUM("N_load_-_Crop,_Area"[*, Area])*(1-0.2*"S1-Mulching"*TIME/360) |  | Dmnl |
| "N_load_-_Crop"[Crop] |  | Fertilizers_load[N, Crop] |  | Dmnl |
| "N_load_-_Crop,_Area"[Crop, Area] |  | "N_load_-_Crop"[Crop]*Agricultural_areas[Crop, Area]/SUM(Agricultural_areas[*, Area]) |  | Dmnl |
| Potential_contribution_of_capillary_rise[Pome_fruit_trees, Delta] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Pome_fruit_trees, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Stone_fruit_trees, Delta] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Stone_fruit_trees, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Cereals, Delta] |  | 0.51 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Cereals, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Corn, Delta] |  | 0.55 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Corn, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Nuts, Delta] |  | 0.51 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Nuts, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Forage, Delta] |  | 0.55 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Forage, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Cotton, Delta] |  | 1 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Cotton, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Olives, Delta] |  | 0.51 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Olives, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Other_trees, Delta] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Other_trees, Agia] |  | 0 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Other_crops, Delta] |  | 0.51 |  | Dmnl |
| Potential_contribution_of_capillary_rise[Other_crops, Agia] |  | 0 |  | Dmnl |
| Production_cost[Area] |  | (0.6*Cost_of_water[Area]+0.2*(1-Development_of_consortia[Area])+0.2*(1-Farmers'_training_and_awareness))/3 |  | Dmnl |
| Recommended_fertilizers_application_rate[N, Pome_fruit_trees] |  | 45 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Stone_fruit_trees] |  | 45 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Cereals] |  | 101 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Corn] |  | 201 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Nuts] |  | 110 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Forage] |  | 28 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Cotton] |  | 84 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Olives] |  | 89 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Other_trees] |  | 45 |  | kg/ha |
| Recommended_fertilizers_application_rate[N, Other_crops] |  | 168 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Pome_fruit_trees] |  | 34 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Stone_fruit_trees] |  | 34 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Cereals] |  | 45 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Corn] |  | 101 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Nuts] |  | 40 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Forage] |  | 28 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Cotton] |  | 67 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Olives] |  | 45 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Other_trees] |  | 34 |  | kg/ha |
| Recommended_fertilizers_application_rate[P, Other_crops] |  | 89 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Pome_fruit_trees] |  | 34 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Stone_fruit_trees] |  | 34 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Cereals] |  | 45 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Corn] |  | 101 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Nuts] |  | 90 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Forage] |  | 68 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Cotton] |  | 67 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Olives] |  | 56 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Other_trees] |  | 34 |  | kg/ha |
| Recommended_fertilizers_application_rate[K, Other_crops] |  | 34 |  | kg/ha |
| Reduction_of_irrigation_water_needs_due_to_NBS[Crop, Area] |  | "S2-Irrigation_programming"*"S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Crop, Area]+"S1-Mulching"*"S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Crop, Area] |  | Dmnl |
| "S1-Mulching" |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Pome_fruit_trees, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Pome_fruit_trees, Agia] |  | 0.01 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Stone_fruit_trees, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Stone_fruit_trees, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Cereals, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Cereals, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Corn, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Corn, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Nuts, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Nuts, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Forage, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Forage, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Cotton, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Cotton, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Olives, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Olives, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Other_trees, Delta] |  | 0.05 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Other_trees, Agia] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Other_crops, Delta] |  | 0 |  | Dmnl |
| "S1-Reduction_of_irrigation_water_needs_due_to_mulching"[Other_crops, Agia] |  | 0 |  | Dmnl |
| "S2-Irrigation_programming" |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Pome_fruit_trees, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Pome_fruit_trees, Agia] |  | 0.225 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Stone_fruit_trees, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Stone_fruit_trees, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Cereals, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Cereals, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Corn, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Corn, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Nuts, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Nuts, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Forage, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Forage, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Cotton, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Cotton, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Olives, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Olives, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Other_trees, Delta] |  | 0.176 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Other_trees, Agia] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Other_crops, Delta] |  | 0 |  | Dmnl |
| "S2-Reduction_of_irrigation_water_needs_due_to_irrigation_programming"[Other_crops, Agia] |  | 0 |  | Dmnl |
| "Target_-_unit_crop_yield"[Crop] |  | 1.1*Average_unit_crop_yield[Crop] |  | kg/ha |
| Target_crop_profitability[Pome_fruit_trees] |  | 0.44 |  | euro/kg |
| Target_crop_profitability[Stone_fruit_trees] |  | 1.35 |  | euro/kg |
| Target_crop_profitability[Cereals] |  | 0.22 |  | euro/kg |
| Target_crop_profitability[Corn] |  | 0.2 |  | euro/kg |
| Target_crop_profitability[Nuts] |  | 2.67 |  | euro/kg |
| Target_crop_profitability[Forage] |  | 0.01 |  | euro/kg |
| Target_crop_profitability[Cotton] |  | 0.01 |  | euro/kg |
| Target_crop_profitability[Olives] |  | 0.01 |  | euro/kg |
| Target_crop_profitability[Other_trees] |  | 0.65 |  | euro/kg |
| Target_crop_profitability[Other_crops] |  | 0.01 |  | euro/kg |
| Trend_AA[Crop, Area] |  | TREND(Agricultural_areas[Crop, Area], 1, 0) |  | Dmnl |
| "Trend_AA_-_sum"[Area] |  | SUM(Trend_AA[*, Area]) |  | Dmnl |
| Unit_crop_prof[Crop] |  | Target_crop_profitability[Crop]*0.5 |  | euro/kg |
| Unit_crop_profitability[Pome_fruit_trees] |  | 0.47 |  | euro/kg |
| Unit_crop_profitability[Stone_fruit_trees] |  | 1.26 |  | euro/kg |
| Unit_crop_profitability[Cereals] |  | 0.25 |  | euro/kg |
| Unit_crop_profitability[Corn] |  | 0.23 |  | euro/kg |
| Unit_crop_profitability[Nuts] |  | 2.83 |  | euro/kg |
| Unit_crop_profitability[Forage] |  | 0.01 |  | euro/kg |
| Unit_crop_profitability[Cotton] |  | 0.01 |  | euro/kg |
| Unit_crop_profitability[Olives] |  | 0.01 |  | euro/kg |
| Unit_crop_profitability[Other_trees] |  | 0.8 |  | euro/kg |
| Unit_crop_profitability[Other_crops] |  | 0.01 |  | euro/kg |
| Water_availability_for_irrigation[Area] |  | WATER.SW_availability[Area]+WATER.GW_availability[Area] |  | Mm3 |
| Soil_erosion[Area](t) |  | Soil_erosion[Area](t - dt) + (SE_increase_rate[Area] - SE_decrease_rate[Area]) * dt | INIT Soil_erosion[Area] = 0.5 | Dmnl |
| Soil_quality[Area](t) |  | Soil_quality[Area](t - dt) + (SQ_increase_rate[Area] - SQ_decrease_rate[Area]) * dt | INIT Soil_quality[Area] = 0.5 | Dmnl |
| State_of_natural_areas[Area](t) |  | State_of_natural_areas[Area](t - dt) + (SNA_increase_rate[Area] - SNA_decrease_rate[Area]) * dt | INIT State_of_natural_areas[Area] = 0.5 | Dmnl |
| State_of_riparian_habitat_and_forest_conservation[Area](t) |  | State_of_riparian_habitat_and_forest_conservation[Area](t - dt) + (PHF_increase_rate[Area] - PHF_decrease_rate[Area]) * dt | INIT State_of_riparian_habitat_and_forest_conservation[Area] = 0.2 | Dmnl |
| PHF_decrease_rate[Area] |  | WATER.Average_water_quality[Area]*Urbanization_increase*"Agricultural_areas_-_irrigated"[Area]/"Agricultural_areas_-_irrigated_INIT"[Area] |  | Dmnl/Month |
| PHF_increase_rate[Area] |  | AGRICULTURE_LAND."Landuse_planning_regulation/actions" |  | Dmnl/Month |
| SE_decrease_rate[Area] |  | ( IF Soil_erosion[Area] > 0 THEN 0.001*((AGRICULTURE_LAND."S1-Mulching")+AGRICULTURE_LAND."Landuse_planning_regulation/actions"+AGRICULTURE_LAND.Farmers'_training_and_awareness)/3 ELSE 0 ) |  | Dmnl/Month |
| SE_increase_rate[Area] |  | ( IF Soil_erosion[Area] < 1 THEN 0.001*(("Agricultural_areas_-_sum"[Area]/"Initial_crops_area_-_sum"[Area])+WATER.CC_scenario_YN+(1-AGRICULTURE_LAND.Farmers'_training_and_awareness)+(1-AGRICULTURE_LAND."Landuse_planning_regulation/actions"))/4 ELSE 0 ) |  | Dmnl/Month |
| SNA_decrease_rate[Area] |  | ( IF State_of_natural_areas[Area] > 0 THEN State_of_natural_areas[Area]*(0.01*((1-WATER.Average_water_quality[Area])+(1-Soil_quality[Area])+Urbanization_increase)/3)*("Agricultural_areas_-_sum"[Area]/"Initial_crops_area_-_sum"[Area]) ELSE 0 ) |  | Dmnl/Month |
| SNA_increase_rate[Area] |  | State_of_natural_areas[Area]*0.01*AGRICULTURE_LAND."Landuse_planning_regulation/actions" |  | Dmnl/Month |
| SQ_decrease_rate[Area] |  | ( IF Soil_quality[Area] < 1 THEN Soil_quality[Area]*0.001*((AGRICULTURE_LAND."N_load_-_Area"[Area]+WATER.CC_scenario_YN+(1-Soil_organic_matter[Area])+Soil_erosion[Area]/4)*("Agricultural_areas_-_sum"[Area]/"Initial_crops_area_-_sum"[Area])) ELSE 0 ) |  | Dmnl/Month |
| SQ_increase_rate[Area] |  | Soil_quality[Area]*0.005*((AGRICULTURE_LAND."Landuse_planning_regulation/actions"+Soil_organic_matter[Area])/2) |  | Dmnl/Month |
| "Agricultural_areas_-_irrigated"[Area] |  | SUM(AGRICULTURE_LAND.Agricultural_areas[*:Irrigated, Area]) |  | ha |
| "Agricultural_areas_-_irrigated_INIT"[Area] |  | INIT("Agricultural_areas_-_irrigated"[Area]) |  | ha |
| "Agricultural_areas_-_sum"[Area] |  | SUM(AGRICULTURE_LAND.Agricultural_areas[*, Area]) |  | ha |
| "Initial_crops_area_-_sum"[Area] |  | SUM(AGRICULTURE_LAND.Initial_crops_area[*, Area]) |  | ha |
| Potential_for_agrotourism[Area] |  | State_of_natural_areas[Area] |  | Dmnl |
| Soil_organic_matter[Area] |  | ((1-AGRICULTURE_LAND."N_load_-_Area"[Area])+(AGRICULTURE_LAND.Farmers'_training_and_awareness+AGRICULTURE_LAND."S1-Mulching")*TIME/360)/3 |  | Dmnl |
| Urbanization_increase |  | 0.5 |  | Dmnl |
| GW_availability[Area](t) |  | GW_availability[Area](t - dt) + (GW_availability_increase_rate[Area] - GW_availability_decrease_rate[Area]) * dt | INIT GW_availability[Area] = Initial_GW_storage[Area] | Mm3 |
| "GW_availability_-_mGrowa"[Area](t) |  | "GW_availability_-_mGrowa"[Area](t - dt) + (GW_inc_rate[Area] - GW_dec_rate[Area]) * dt | INIT "GW_availability_-_mGrowa"[Area] = 0 | m3/ha |
| GW_quality[Area](t) |  | GW_quality[Area](t - dt) + (GW_quality_increase_rate[Area] - GW_quality_decrease_rate[Area]) * dt | INIT GW_quality[Area] = Initial_GW_quality[Area] | Dmnl |
| SW_availability[Area](t) |  | SW_availability[Area](t - dt) + (SW_availability_increase_rate[Area] - SW_availability_decrease_rate[Area]) * dt | INIT SW_availability[Area] = 1 | Mm3 |
| "SW_availability_-_mGrowa"[Area](t) |  | "SW_availability_-_mGrowa"[Area](t - dt) + (SW_inc_rate[Area] - SW_dec_rate[Area]) * dt | INIT "SW_availability_-_mGrowa"[Area] = 0 | m3/ha |
| SW_in_Reservoirs[Area](t) |  | SW_in_Reservoirs[Area](t - dt) + (SW_increase_rate[Area] - SW_decrease_rate[Area]) * dt | INIT SW_in_Reservoirs[Area] = Initial_SW_in_reservoirs[Area] | Mm3 |
| SW_quality[Area](t) |  | SW_quality[Area](t - dt) + (SW_quality_increase_rate[Area] - SW_quality_decrease_rate[Area]) * dt | INIT SW_quality[Area] = Initial_SW_quality[Area] | Dmnl |
| GW_availability_decrease_rate[Area] |  | GW_demand[Area]+"Capillary_rise_-_volume_per_area"[Area]-Potential_GW_withdrawal_reduction_due_to_Reservoirs[Area]+GW_outflow[Area] |  | Mm3/Month |
| GW_availability_increase_rate[Area] |  | ( IF GW_availability[Area] < GW_storage[Area] THEN GW_recharge_rate[Area]*(1+(CC_scenario_YN*"Yearly_change_-_GW_recharge_rate"[Area]/AGRICULTURE_LAND.Months_per_year)) ELSE 0 ) |  | Mm3/Month |
| GW_dec_rate[Area] |  | 0 |  | m3/(ha*Month) |
| GW_inc_rate[Area] |  | SW_dec_rate[Area] |  | m3/(ha*Month) |
| GW_quality_decrease_rate[Area] |  | ( IF GW_quality[Area] > 0 THEN 0.01*(("Diffuse_sources_of_pollution_-_Area"[Area])+0.2*(1-GW_availability_over_time[Area])+0.2*(High_Salinity_area_ratio[Area])) ELSE 0 ) |  | Dmnl/Month |
| GW_quality_increase_rate[Area] |  | ( IF GW_quality[Area] > 0 THEN 0.01*(1-"Diffuse_sources_of_pollution_-_Area"[Area])*(GW_availability_over_time[Area])*(1+Protected_area_ratio[Area]) ELSE 0 ) |  | Dmnl/Month |
| SW_availability_decrease_rate[Area] |  | CC_scenario_YN |  | Mm3/Month |
| SW_availability_increase_rate[Area] |  | "SW_inflow_-_HRUs"[Area]+Water_in_reservoirs[Area] |  | Mm3/Month |
| SW_dec_rate[Area] |  | "Total_Runoff_-_mGrowa"[Area]*BFI[Area] |  | m3/(ha*Month) |
| SW_decrease_rate[Area] |  | ( IF "S4_-_SW_reservoirs_active_YN"[Area] = 1 THEN ( IF SW_in_Reservoirs[Area] > SW_use[Area] THEN SW_use[Area] ELSE 0 ) ELSE 0 ) |  | Mm3/Month |
| SW_inc_rate[Area] |  | "Total_Runoff_-_mGrowa"[Area] |  | m3/(ha*Month) |
| SW_increase_rate[Area] |  | ( IF SW_in_Reservoirs[Area] < Max_SW_in_reservoirs[Area] THEN Reservoirs_recharge_rate[Area]*(1+"Yearly_change_-_Reservoirs_recharge_rate"[Area]) ELSE 0 ) |  | Mm3/Month |
| SW_quality_decrease_rate[Area] |  | ( IF SW_quality[Area] > 0 THEN 0.01*"Diffuse_sources_of_pollution_-_Area"[Area]*(1-Protected_area_ratio[Area]) ELSE 0 ) |  | Dmnl/Month |
| SW_quality_increase_rate[Area] |  | ( IF SW_quality[Area] < 1 THEN 0.01*(1-"Diffuse_sources_of_pollution_-_Area"[Area])*(Protected_area_ratio[Area]) ELSE 0 ) |  | Dmnl/Month |
| Additional_SW_availability[Area] |  | ( IF "S4_-_SW_reservoirs_active_YN"[Area] = 1 THEN ( IF SW_in_Reservoirs[Area] > 0.2*Initial_SW_in_reservoirs[Area] THEN SW_in_Reservoirs[Area]-0.2*Initial_SW_in_reservoirs[Area] ELSE 0 ) ELSE 0 ) |  | Mm3 |
| "Average_Pinios_river_flow_-_Delta_U"[Jan] |  | 117.28 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Feb] |  | 60.84 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Mar] |  | 102.54 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Apr] |  | 78.91 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[May] |  | 30.42 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Jun] |  | 22.67 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Jul] |  | 22.21 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Aug] |  | 23.05 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Sep] |  | 67.14 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Oct] |  | 42.2 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Nov] |  | 48.3 |  | m3/s |
| "Average_Pinios_river_flow_-_Delta_U"[Dec] |  | 100.42 |  | m3/s |
| "Average_river_discharge_-_mGrowa"[Area] |  | 10 |  | m3/s |
| Average_water_quality[Area] |  | (GW_quality[Area]+SW_quality[Area])/2 |  | Dmnl |
| "Capillary_rise_-_volume"[Crop, Area] |  | AGRICULTURE_LAND.Potential_contribution_of_capillary_rise[Crop, Area]*AGRICULTURE_LAND.Average_crop_unit_water_need[Crop, Area]*AGRICULTURE_LAND.Agricultural_areas[Crop, Area]*"Conversion_m3/Mm3"*AGRICULTURE_LAND.Irrigation_water_demand_coefficient |  | Mm3/Month |
| "Capillary_rise_-_volume_per_area"[Area] |  | SUM("Capillary_rise_-_volume"[*, Area]) |  | Mm3 |
| CC_scenario_YN |  | 1 |  | Dmnl |
| Climate_change_impacts |  | 1 |  | Dmnl |
| "Conversion_m3/Mm3" |  | 1e-06 |  | Mm3/m3 |
| "Conversion_s/Month" |  | 60*60*24*30 |  | s/Month |
| "Diffuse_sources_of_pollution_-_Area"[Area] |  | AGRICULTURE_LAND."N_load_-_Area"[Area] |  | Dmnl |
| Drinking_water_demand[Area] |  | (1+"Loss_rate_-_drinking")*Population[Area]*Unit_drinking_water_demand[Area]*"Conversion_m3/Mm3"*Fluctuating_population_coefficient[Area] |  | Mm3/Month |
| Fluctuating_population_coefficient[Delta] |  | ( IF Month_of_the_year = 1 THEN 1 ELSE ( IF Month_of_the_year = 2 THEN 1 ELSE ( IF Month_of_the_year = 3 THEN 1 ELSE ( IF Month_of_the_year = 4 THEN 1 ELSE ( IF Month_of_the_year = 5 THEN 1 ELSE ( IF Month_of_the_year = 6 THEN 2.25 ELSE ( IF Month_of_the_year = 7 THEN 2.25 ELSE ( IF Month_of_the_year = 8 THEN 2.25 ELSE ( IF Month_of_the_year = 9 THEN 1 ELSE ( IF Month_of_the_year = 10 THEN 1 ELSE 1 ) ) ) ) ) ) ) ) ) ) |  | Dmnl |
| Fluctuating_population_coefficient[Agia] |  | ( IF Month_of_the_year = 1 THEN 1 ELSE ( IF Month_of_the_year = 2 THEN 1 ELSE ( IF Month_of_the_year = 3 THEN 1 ELSE ( IF Month_of_the_year = 4 THEN 1 ELSE ( IF Month_of_the_year = 5 THEN 1 ELSE ( IF Month_of_the_year = 6 THEN 1.2 ELSE ( IF Month_of_the_year = 7 THEN 1.2 ELSE ( IF Month_of_the_year = 8 THEN 1.2 ELSE ( IF Month_of_the_year = 9 THEN 1 ELSE ( IF Month_of_the_year = 10 THEN 1 ELSE 1 ) ) ) ) ) ) ) ) ) ) |  | Dmnl |
| "Frequency_and_intensity_of_extremes_(droughts_and_floods)" |  | Climate_change_impacts*TIME/360 |  | Dmnl |
| GW_availability_over_time[Area] |  | ( IF GW_availability[Area] < 0 THEN 0 ELSE MIN(GW_availability[Area]/Initial_GW_storage[Area], 1) ) |  | Dmnl |
| GW_demand[Area] |  | "GW_use_-_drinking"[Area]+"GW_use_-_irrigation"[Area]+"GW_use_-_other"[Area] |  | Mm3/Month |
| GW_outflow[Area] |  | ( IF GW_availability[Area] > GW_storage[Area] THEN GW_availability[Area]-GW_storage[Area] ELSE 0 ) |  | Mm3 |
| GW_recharge_rate[Delta] |  | 0.87 |  | Mm3/Month |
| GW_recharge_rate[Agia] |  | 0.62 |  | Mm3/Month |
| GW_storage[Delta] |  | 37 |  | Mm3 |
| GW_storage[Agia] |  | 45 |  | Mm3 |
| "GW_use_-_drinking"[Area] |  | Drinking_water_demand[Area]*(1-"SW/GW_uses_-_area"[Drinking, Area]) |  | Mm3/Month |
| "GW_use_-_irrigation"[Area] |  | AGRICULTURE_LAND.Irrigation_water_use_per_Area[Area]*(1-"SW/GW_uses_-_area"[Irrigation, Area]) |  | Mm3/Month |
| "GW_use_-_other"[Area] |  | Water_demand_for_industrial_use[Area]*(1-"SW/GW_uses_-_area"[Other, Area]) |  | Mm3/Month |
| High_Salinity_area_ratio[Delta] |  | 0.2 |  | Dmnl |
| High_Salinity_area_ratio[Agia] |  | 0 |  | Dmnl |
| Initial_GW_quality[Delta] |  | 0.5 |  | Dmnl |
| Initial_GW_quality[Agia] |  | 0.5 |  | Dmnl |
| Initial_GW_storage[Delta] |  | 25 |  | Mm3 |
| Initial_GW_storage[Agia] |  | 30 |  | Mm3 |
| Initial_SW_in_reservoirs[Delta] |  | 1.75 |  | Mm3 |
| Initial_SW_in_reservoirs[Agia] |  | 2.5 |  | Mm3 |
| Initial_SW_quality[Delta] |  | 0.3 |  | Dmnl |
| Initial_SW_quality[Agia] |  | 0.6 |  | Dmnl |
| "Loss_rate_-_drinking" |  | 0.3 |  | Dmnl |
| Max_SW_in_reservoirs[Delta] |  | 3.5 |  | Mm3 |
| Max_SW_in_reservoirs[Agia] |  | 5 |  | Mm3 |
| Month_of_the_year |  | TIME |  | Month |
| "Monthly_Pinios_river_flow_-_Delta_Downstream" |  | "Monthly_Pinios_river_flow_-_Delta_Upstream"-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) |  | Mm3/Month |
| "Monthly_Pinios_river_flow_-_Delta_Upstream" |  | ( IF Month_of_the_year = 1 THEN "Average_Pinios_river_flow_-_Delta_U"[Jan]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 2 THEN "Average_Pinios_river_flow_-_Delta_U"[Feb]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 3 THEN "Average_Pinios_river_flow_-_Delta_U"[Mar]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 4 THEN "Average_Pinios_river_flow_-_Delta_U"[Apr]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 5 THEN "Average_Pinios_river_flow_-_Delta_U"[May]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 6 THEN "Average_Pinios_river_flow_-_Delta_U"[Jun]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 7 THEN "Average_Pinios_river_flow_-_Delta_U"[Jul]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 8 THEN "Average_Pinios_river_flow_-_Delta_U"[Aug]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 9 THEN "Average_Pinios_river_flow_-_Delta_U"[Sep]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 10 THEN "Average_Pinios_river_flow_-_Delta_U"[Oct]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 11 THEN "Average_Pinios_river_flow_-_Delta_U"[Nov]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE ( IF Month_of_the_year = 12 THEN "Average_Pinios_river_flow_-_Delta_U"[Dec]*"Conversion_s/Month"*"Conversion_m3/Mm3" ELSE 0 ) ) ) ) ) ) ) ) ) ) ) ) |  | Mm3/Month |
| "Pinios_river_flow_-_Delta_D" |  | ( IF Month_of_the_year = 1 THEN "Average_Pinios_river_flow_-_Delta_U"[Jan]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 2 THEN "Average_Pinios_river_flow_-_Delta_U"[Feb]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 3 THEN "Average_Pinios_river_flow_-_Delta_U"[Mar]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 4 THEN "Average_Pinios_river_flow_-_Delta_U"[Apr]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 5 THEN "Average_Pinios_river_flow_-_Delta_U"[May]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 6 THEN "Average_Pinios_river_flow_-_Delta_U"[Jun]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 7 THEN "Average_Pinios_river_flow_-_Delta_U"[Jul]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 8 THEN "Average_Pinios_river_flow_-_Delta_U"[Aug]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 9 THEN "Average_Pinios_river_flow_-_Delta_U"[Sep]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 10 THEN "Average_Pinios_river_flow_-_Delta_U"[Oct]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 11 THEN "Average_Pinios_river_flow_-_Delta_U"[Nov]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE ( IF Month_of_the_year = 12 THEN "Average_Pinios_river_flow_-_Delta_U"[Dec]-SW_use[Delta]*(1-"S4_-_SW_reservoirs_active_YN"[Delta]) ELSE 0 ) ) ) ) ) ) ) ) ) ) ) ) |  | Mm3/Month |
| Point_sources_of_pollution[Area] |  | AGRICULTURE_LAND.Livestock_impacts[Area]+WWT_load[Area] |  | Dmnl |
| Population[Delta] |  | 2723.93 |  | person |
| Population[Agia] |  | 5236.37 |  | person |
| Potential_GW_withdrawal_reduction_due_to_Reservoirs[Area] |  | ( IF GW_demand[Area] > 0 THEN ( IF Additional_SW_availability[Area] > GW_demand[Area] THEN GW_demand[Area] ELSE Additional_SW_availability[Area] ) ELSE 0 ) |  | Mm3/Month |
| Protected_area_ratio[Delta] |  | 0.1 |  | Dmnl |
| Protected_area_ratio[Agia] |  | 0.1 |  | Dmnl |
| QPR_ratio[Area] |  | 1 |  | Dmnl |
| Reservoirs_recharge_rate[Delta] |  | 0.41 |  | Mm3/Month |
| Reservoirs_recharge_rate[Agia] |  | 0.29 |  | Mm3/Month |
| "S4_-_SW_reservoirs_active_YN"[Area] |  | 0 |  | Dmnl |
| "SW_inflow_-_HRUs"[Delta] |  | 10 |  | Mm3/Month |
| "SW_inflow_-_HRUs"[Agia] |  | 10 |  | Mm3/Month |
| SW_use[Area] |  | "SW_use_-_drinking"[Area]+"SW_use_-_irrigation"[Area]+"SW_use_-_other"[Area] |  | Mm3/Month |
| "SW_use_-_drinking"[Area] |  | Drinking_water_demand[Area]*"SW/GW_uses_-_area"[Drinking, Area] |  | Mm3/Month |
| "SW_use_-_irrigation"[Area] |  | AGRICULTURE_LAND.Irrigation_water_use_per_Area[Area]*"SW/GW_uses_-_area"[Irrigation, Area] |  | Mm3/Month |
| "SW_use_-_other"[Area] |  | Water_demand_for_industrial_use[Area]*"SW/GW_uses_-_area"[Other, Area] |  | Mm3/Month |
| "SW/GW_uses_-_area"[Drinking, Delta] |  | 0 |  | Dmnl |
| "SW/GW_uses_-_area"[Drinking, Agia] |  | 0 |  | Dmnl |
| "SW/GW_uses_-_area"[Irrigation, Delta] |  | 0.8 |  | Dmnl |
| "SW/GW_uses_-_area"[Irrigation, Agia] |  | 0 |  | Dmnl |
| "SW/GW_uses_-_area"[Ecosystem, Delta] |  | 0 |  | Dmnl |
| "SW/GW_uses_-_area"[Ecosystem, Agia] |  | 0 |  | Dmnl |
| "SW/GW_uses_-_area"[Other, Delta] |  | 0 |  | Dmnl |
| "SW/GW_uses_-_area"[Other, Agia] |  | 0 |  | Dmnl |
| "Total_Runoff_-_mGrowa"[Delta] |  | 1 |  | m3/(ha*Month) |
| "Total_Runoff_-_mGrowa"[Agia] |  | 1 |  | m3/(ha*Month) |
| Unit_drinking_water_demand[Delta] |  | 9 |  | m3/(person*Month) |
| Unit_drinking_water_demand[Agia] |  | 7.8 |  | m3/(person*Month) |
| Water_demand_for_industrial_use[Delta] |  | 0.042 |  | Mm3/Month |
| Water_demand_for_industrial_use[Agia] |  | 0.042 |  | Mm3/Month |
| Water_in_reservoirs[Area] |  | 0 |  | Mm3/Month |
| Water_quality_monitoring |  | 0.2 |  | Dmnl |
| WWT_efficiency[Area] |  | ( IF Fluctuating_population_coefficient[Area] = 1 THEN 0.6 ELSE 0.6/Fluctuating_population_coefficient[Area] ) |  | Dmnl |
| WWT_load[Area] |  | ( IF Fluctuating_population_coefficient[Area] = 1 THEN (1-WWT_efficiency[Area]) ELSE Fluctuating_population_coefficient[Area]*(1-WWT_efficiency[Area]) ) |  | Dmnl |
| "Yearly_change_-_GW_recharge_rate"[Delta] |  | 0.005 |  | Dmnl |
| "Yearly_change_-_GW_recharge_rate"[Agia] |  | 0.001 |  | Dmnl |
| "Yearly_change_-_Reservoirs_recharge_rate"[Delta] |  | 0 |  | Dmnl |
| "Yearly_change_-_Reservoirs_recharge_rate"[Agia] |  | 0 |  | Dmnl |

S6 Summary of each parameter and variable, showing actual model values, ranges, probability distributions, and rationale for distribution choice as implemented in Vensim’s Sensitivity Simulation module

| PARAMETER NAME | MODEL VALUE |  | RANGE AND PROBABILITY DISTRIBUTION |  | RATIONALE FOR DISTRIBUTION CHOICE |
| --- | --- | --- | --- | --- | --- |
| Yearly change in Reservoirs recharge rate | DELTA | AGIA | DELTA | AGIA | Represents natural variations in recharge influenced by climatic fluctuations, typically cantered around a stable mean, justifying the Normal distribution. |
|  | 0 | 0 | NORMAL(min-0.01,max0.01, mean0,standard deviation0.005) | NORMAL(min-0.01,max0.01, mean0,standard deviation0.005) |  |
| Yearly change in GW(groundwater) recharge rat | -0.005 | -0.001 | NORMAL(min-0.01,max0, mean-0.005,standard deviation0.005) | NORMAL(min-0.002,max0, mean-0.001,standard deviation0.001) | Reflects small annual variations in groundwater dynamics, which are symmetrically distributed due to natural hydrological processes, supporting the Normal distribution. |
| GW(groundwater) recharge rate | 0.87 | 0.62 | TRIANGULAR(min0.435,max1.305, start0.435, peak0.87, stop1.305) | TRIANGULAR(min0.31,max0.93, start0.31, peak0.62, stop0.93) | Depends on varying environmental factors, creating a scenario with a plausible most frequent value and upper/lower bounds, suited for a Triangular distribution. |
| Reservoir recharge rate | 0.41 | 0.29 | TRIANGULAR(min0.205,max0.615, start0.205, peak0.87, stop0.615) | TRIANGULAR(min0.145,max1.435, start0.145, peak0.29, stop0.435) | Influenced by rainfall and runoff, naturally forming a distribution with a most probable rate and definable extremes, well-modelled by a Triangular distribution. |
| Water demand for industrial use | 0.042 | 0.042 | TRIANGULAR(min0.021,max0.063, start0.021, peak0.042, stop0.063) | TRIANGULAR(min0.021,max0.063, start0.021,peak0.042,stop0.063) | Typically centres around predictable usage levels, with known limits affected by capacity and economic activity, making the Triangular distribution appropriate. |
| High salinity area ratio | 0.2 | 0.05 | GAMMA(min0.1,max0.3,order7.11, shift0.1,stretch0.0281) | GAMMA(min0.025,max0.075,order7.1,shift0.025,stretch 0.00703) | Salinity changes occur gradually over time and are influenced by cumulative processes such as irrigation return flow and soil conditions, which align with a Gamma distribution. |
| Population | 2723.93 | 5236.37 | WEIBULL(min2603.5,max2848, shape42.5507,shift0,stretch2758.44) | WEIBULL(min5196.02,max5684, shape42.5548,shift0,stretch5505.25) | Weibull distribution offered the best fit for capturing historical data. |
| Average Pinios river flow – Delta [January] | 117.28 | / | TRIANGULAR(min58.64,max234.56, start58.64,peak117.28,stop234.56) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [February] | 61 | / | TRIANGULAR(min30.5,max122, start30.5,peak61,stop122) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [March] | 102.07 | / | TRIANGULAR(min51.04,max204.14, start51.04,peak102.07,stop204.14) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [April] | 78.91 | / | TRIANGULAR(min39.46,max157.82, start39.46,peak78.91,stop157.82) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [May] | 30.43 | / | TRIANGULAR(min15.215,max60.86, start15.215,peak30.43,stop60.86) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [June] | 22.67 | / | TRIANGULAR(min11.335,max45.34, start11.335,peak22.67,stop45.34) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [July] | 22.21 | / | TRIANGULAR(min11.105,max44.42, start11.105,peak22.21,stop44.42) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [August] | 30 | / | TRIANGULAR(min15,max60, start15,peak30,stop60) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [September] | 67.14 | / | TRIANGULAR(min33.57,max134.28, start33.57,peak67.14,stop134.28) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [October] | 42.33 | / | TRIANGULAR(min21.165,max84.66, start21.165,peak42.33,stop84.66) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [November] | 48.3 | / | TRIANGULAR(min24.15,max96.6, start24.15,peak48.3,stop96.6) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| Average Pinios river flow – Delta [December] | 100.27 | / | TRIANGULAR(min50.135,max200.54,start50.135,peak100.27,stop200.54) | / | Monthly river flows vary due to seasonal patterns, with a most probable flow influenced by climate and watershed characteristics, aligning with a Triangular distribution. |
| SW(surface water)/GW(groundwater) uses [Irrigation] | 0.846 | / | TRIANGULAR(min0.423,max1.269, start0.423,peak0.846,stop1.269) | / | Depends on resource availability and allocation strategies, with a natural tendency toward a predominant use while having defined limits, making a Triangular distribution appropriate. |
| Baseline fertilizers application rate [Nitrate, Pome fruit trees] | 43 |  | UNIFORM(min36,max50) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Stone fruit trees] | 43 |  | UNIFORM(min36,max50) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Cereals] | 52 |  | UNIFORM(min44,max60) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Corn] | 250 |  | UNIFORM(min213,max288) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Nuts] | 43 |  | UNIFORM(min36,max50) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Forage] | 8 |  | UNIFORM(min6,max9) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Cotton] | 112 |  | UNIFORM(min95,max129) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Olives] | 43 |  | UNIFORM(min36,max50) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Other trees] | 43 |  | UNIFORM(min36,max50) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Baseline fertilizers application rate [Nitrate, Other crops] | 220 |  | UNIFORM(min187,max253) |  | Fertilizer use is uniformly distributed within a range determined by agricultural practices and crop needs, reflecting no strong preference for specific values, hence a Uniform distribution. |
| Potential contribution of capillary rise [Cereals] | 0.51 | / | TRIANGULAR(min0.255,max0.765, start0.255,peak0.51,stop0.765) | / | Capillary rise depends on soil properties and water availability, with a central tendency and natural limits, justifying a Triangular distribution. |
| Potential contribution of capillary rise [Corn] | 0.55 | / | TRIANGULAR(min0.275,max0.825, start0.275,peak0.55,stop0.825) | / | Capillary rise depends on soil properties and water availability, with a central tendency and natural limits, justifying a Triangular distribution. |
| Potential contribution of capillary rise [Nuts] | 0.51 | / | TRIANGULAR(min0.255,max0.765, start0.255,peak0.51,stop0.765) | / | Capillary rise depends on soil properties and water availability, with a central tendency and natural limits, justifying a Triangular distribution. |
| Potential contribution of capillary rise [Forage] | 0.55 | / | TRIANGULAR(min0.275,max0.825, start0.275,peak0.55,stop0.825) | / | Capillary rise depends on soil properties and water availability, with a central tendency and natural limits, justifying a Triangular distribution. |
| Potential contribution of capillary rise [Cotton] | 1 | / | UNIFORM(min0.5,max1) | / | For this crop where the deterministic value is 1, the Uniform distribution reflects a lack of variability in expected contributions, ensuring equal probability across the specified range. |
| Potential contribution of capillary rise [Olives] | 0.51 | / | TRIANGULAR(min0.255,max0.765, start0.255,peak0.51,stop0.765) | / | Capillary rise depends on soil properties and water availability, with a central tendency and natural limits, justifying a Triangular distribution. |
| Potential contribution of capillary rise [Other crops] | 0.51 | / | TRIANGULAR(min0.255,max0.765, start0.255,peak0.51,stop0.765) | / | Capillary rise depends on soil properties and water availability, with a central tendency and natural limits, justifying a Triangular distribution. |
| Average crop unit water need [Pome fruit trees] | 4200 | 5580 | TRIANGULAR(min2100,max6300, start2100,peak4200,stop6300) | TRIANGULAR(min2790,max8370, start2790,peak5580,stop8370) | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Stone fruit trees] | 4200 | 4667 | TRIANGULAR(min2100,max6300, start2100,peak4200,stop6300) | TRIANGULAR(min2333.5,max7000.5,start2333.5,peak4667,stop7000.5) | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Corn] | 4813 | 4167 | TRIANGULAR(min2406.5,max7219.5 start2406.5,peak4813,stop7219.5) | TRIANGULAR(min2083.5,max6250.5 start2083.5,peak4167,stop6250.5) | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Nuts] | 3300 | 3667 | TRIANGULAR(min1650,max4950, start1650,peak3300,stop4950) | TRIANGULAR(min1833.5,max5500.5 start1833.5,peak3667,stop5500.5) | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Forage] | 4903 | 4167 | TRIANGULAR(min2451.5,max7354.5 start2451.5,peak4903,stop7354.5) | TRIANGULAR(min2083.5,max6250.5 start2083.5,peak4167,stop6250.5) | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Cotton] | 2650 | 2208 | TRIANGULAR(min1325,max3975, start1325,peak2650,stop3975) | TRIANGULAR(min1104,max3312, start1104,peak2208,stop3312) | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Olives] | 1130 | / | TRIANGULAR(min565,max1695, start565,peak1130,stop1695) | / | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Other trees] | 10462 | / | TRIANGULAR(min5231,max15693, start5231,peak10462,stop15693) | / | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Average crop unit water need [Other crops] | 3662 | / | TRIANGULAR(min831,max5493, start831,peak3662,stop5493) | / | Depends on crop physiology and growth stages, forming a central value with natural variability around minimum and maximum, supporting a Triangular distribution. |
| Loss rate drinking | 0.3 |  | TRIANGULAR(min0.15,max0.45,Start0.15,peak0.3,stop0.45) |  | Reflects infrastructure efficiency and maintenance levels, with a most likely value and identifiable minimum and maximum limits, justifying the use of a Triangular distribution. |
| Average unit crop yield [Pome fruit trees] | 27470 |  | TRIANGULAR(min13735,max41205,Start13735,peak27470,stop41205) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Stone fruit trees] | 4812 |  | TRIANGULAR(min2406,max7218,start2406,peak4812,stop7218) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Cereals] | 4000 |  | TRIANGULAR(min2000,max6000,start2000,peak4000,stop6000) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Corn] | 11018 |  | TRIANGULAR(min5509,max16527,start5509,peak11018,stop16527) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Nuts] | 1291 |  | TRIANGULAR(min645.5,max19365,start645.5,peak1291,stop19365) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Forage] | 3000 |  | TRIANGULAR(min1500,max4500,start1500,peak3000,stop4500) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Cotton] | 3000 |  | TRIANGULAR(min1500,max4500,start1500,peak3000,stop4500) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Olives] | 3000 |  | TRIANGULAR(min1500,max4500,start1500,peak3000,stop4500) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Other trees] | 24931 |  | TRIANGULAR(min12465.5,max37396.5,start12465.5,peak24931,stop37396.5) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Average unit crop yield [Other crops] | 43000 |  | TRIANGULAR(min21500,max64500,start21500,peak43000,stop64500) |  | Crop yield varies based on environmental factors, agricultural practices, and crop types, typically with a central tendency and natural bounds, well-represented by a Triangular distribution. |
| Target crop profitability [Pome fruit trees] | 0.44 |  | NORMAL(min0.22,max0.66,mean0.44,standard deviation0.11) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Stone fruit trees] | 1.35 |  | NORMAL(min0.675,max2.025,mean1.35,standard deviation0.3375) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Cereals] | 0.22 |  | NORMAL(min0.11,max0.33,mean0.22,standard deviation0.055) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Corn] | 0.2 |  | NORMAL(min0.10,max0.30,mean0.20,standard deviation0.050) |  | Profitability depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Nuts] | 2.67 |  | NORMAL(min1.335,max4.005,mean2.67,standard deviation0.6675) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Forage] | 0.01 |  | NORMAL(min0.005,max0.015,mean0.01,standard deviation0.0025) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Cotton] | 0.01 |  | NORMAL(min0.005,max0.015,mean0.01,standard deviation0.0025) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Olives] | 0.01 |  | NORMAL(min0.005,max0.015,mean0.01,standard deviation0.0025) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Other trees] | 0.65 |  | NORMAL(min0.325,max0.975,mean0.65,standard deviation0.1625) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Target crop profitability [Other crops] | 0.01 |  | NORMAL(min0.005,max0.015,mean0.01,standard deviation0.0025) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| Urbanization increase | 0.5 |  | TRIANGULAR(min0.25,max0.75,Start0.25,peak0.5,stop0.75) |  | Depends on market conditions and input costs, which vary symmetrically around an expected average, fitting a Normal distribution. |
| VARIABLE NAME | MODEL EQUATION |  | EQUATION WITH DISTRIBUTION |  | RATIONALE FOR DISTRIBUTION CHOICE |
| Irrigation water demand coefficient | IF THEN ELSE(Month of the year=1, 0 , IF THEN ELSE(Month of the year=2, 0 , IF THEN ELSE(Month of the year=3, 0 , IF THEN ELSE (Month of the year=4, 0, IF THEN ELSE(Month of the year=5, 0.07, IF THEN ELSE(Month of the year=6, 0.2 , IF THEN ELSE(Month of the year =7, 0.3, IF THEN ELSE(Month of the year=8, 0.3 , IF THEN ELSE(Month of the year=9, 0.13 , IF THEN ELSE(Month of the year= 10, 0 , 0) ) ) ) ) ) ) ) ) ) |  | IF THEN ELSE(Month of the year=1, 0 , IF THEN ELSE(Month of the year=2, 0 , IF THEN ELSE(Month of the year=3, 0 , IF THEN ELSE(Month of the year=4, 0, IF THEN ELSE(Month of the year=5, RANDOM TRIANGULAR(0.035, 0.105, 0.035 , 0.07 , 0.105 , 0 ), IF THEN ELSE(Month of the year=6, RANDOM TRIANGULAR(0.1, 0.3 , 0.1 , 0.2 , 0.3 , 0 ) , IF THEN ELSE(Month of the year =7, RANDOM TRIANGULAR(0.15, 0.45 , 0.15 , 0.3 , 0.45 , 0 ), IF THEN ELSE(Month of the year=8, RANDOM TRIANGULAR(0.15, 0.45 , 0.15 , 0.3 , 0.45 , 0 ), IF THEN ELSE(Month of the year=9, RANDOM TRIANGULAR(0.065, 0.195 , 0.065 , 0.13 , 0.195 , 0 ) , IF THEN ELSE(Month of the year=10, 0 , 0) ) ) ) ) ) ) ) ) ) |  | Fluctuates seasonally, influenced by crop water requirements and climatic conditions, naturally forming a peak usage period and defined bounds, supporting the choice of a Triangular distribution. |
| Fluctuating population coefficient [Delta] | IF THEN ELSE(Month of the year=1, 1 , IF THEN ELSE(Month of the year=2, 1 , IF THEN ELSE(Month of the year=3, 1 , IF THEN ELSE(Month of the year=4, 1, IF THEN ELSE(Month of the year=5, 1, IF THEN ELSE(Month of the year=6, 2.25 , IF THEN ELSE(Month of the year=7, 2.25, IF THEN ELSE(Month of the year=8, 2.25 , IF THEN ELSE(Month of the year=9, 1 , IF THEN ELSE(Month of the year=10, 1, 1 ) ) ) ) ) ) ) ) ) ) |  | IF THEN ELSE(Month of the year=1, 1 , IF THEN ELSE(Month of the year=2, 1 , IF THEN ELSE(Month of the year=3, 1 , IF THEN ELSE(Month of the year=4, 1, IF THEN ELSE(Month of the year=5, 1, IF THEN ELSE(Month of the year=6, RANDOM TRIANGULAR( 0.8, 2.4 , 0.8 , 1.6 , 2.4, 0) , IF THEN ELSE(Month of the year=7,RANDOM TRIANGULAR( 0.8, 2.4 , 0.8 , 1.6 , 2.4, 0), IF THEN ELSE(Month of the year=8, RANDOM TRIANGULAR( 0.8, 2.4 , 0.8 , 1.6 , 2.4, 0), IF THEN ELSE(Month of the year=9, 1 , IF THEN ELSE(Month of the year=10, 1, 1 ) ) ) ) ) ) ) ) ) ) |  | Population changes seasonally or due to socio-economic factors, with predictable central behaviour and clear limits, making the Triangular distribution suitable. |
| Fluctuating population coefficient [Agia] | IF THEN ELSE(Month of the year=1, 1 , IF THEN ELSE(Month of the year=2, 1 , IF THEN ELSE(Month of the year=3, 1 , IF THEN ELSE(Month of the year=4, 1, IF THEN ELSE(Month of the year=5, 1, IF THEN ELSE(Month of the year=6, 1.2 , IF THEN ELSE(Month of the year=7, 1.2, IF THEN ELSE(Month of the year=8, 1.2 , IF THEN ELSE(Month of the year=9, 1 , IF THEN ELSE(Month of the year=10, 1, 1 ) ) ) ) ) ) ) ) ) ) |  | IF THEN ELSE(Month of the year=1, 1 , IF THEN ELSE(Month of the year=2, 1 , IF THEN ELSE(Month of the year=3, 1 , IF THEN ELSE(Month of the year=4, 1, IF THEN ELSE(Month of the year=5, 1, IF THEN ELSE(Month of the year=6, RANDOM TRIANGULAR( 0.55, 1.65 , 0.55 , 1.1 , 1.65, 0 ), IF THEN ELSE(Month of the year=7, RANDOM TRIANGULAR( 0.55, 1.65 , 0.55 , 1.1 , 1.65, 0 ), IF THEN ELSE(Month of the year=8, RANDOM TRIANGULAR( 0.55, 1.65 , 0.55 , 1.1 , 1.65, 0 ), IF THEN ELSE(Month of the year=9, 1 , IF THEN ELSE(Month of the year=10, 1, 1 ) ) ) ) ) ) ) ) ) ) |  | Population changes seasonally or due to socio-economic factors, with predictable central behaviour and clear limits, making the Triangular distribution suitable. |

References

Arcade, J., Godet, M., Meunier, F., & Roubelat, F. (1999), Structural Analysis with the MICMAC Method & Actors' Strategy with MACTOR Method. In Futures Research Methodology, 1–69. American Council for the United Nations University: The Millennium Project.

Heilgeist, S., Sahin, O., Sekine, R., & Stewart, R. A. (2022), Mapping the Complex Journey of Swimming Pool Contaminants: A Multi-Method Systems Approach. Water, 14(13), 1–28. doi:10.3390/w14132062

Nazlabadi, E., Maknoon, R., Moghaddam, M. R. A., & Daigger, G. T. (2023), A novel MICMAC approach for cross impact analysis with application to urban water/wastewater management. Expert Systems with Applications, 230, 120667. doi:10.1016/j.eswa.2023.120667

Newman, M. E. J. (2008), The Mathematics of Networks. In: Blume, L. E., Durlauf, S. N. (eds). The New Palgrave Dictionary of Economics. Palgrave Macmillan, London. doi:10.1057/978-1-349-95121-5_2565-1

Schoenberg, W. (2020), Loops that matter. PhD thesis, Department of Geography, University of Bergen, Norway.

Schoenberg, W., Hayward, J., Eberlein, R. (2023), Improving loops that matter. System Dynamics Review, 39 (2), 140–151. doi:10.1002/sdr.1728

Walters, J., Valcourt, N., Linden, K., Javernick-Will, A., Lockwood, H. (2022), Challenges and solutions to rural water service sustainability in East African countries: a ‘systems scaffolding’ perspective. Environmental Science and Policy, 136:564–574. doi:10.1016/j.envsci.2022.07.023