Supplementary informations

Analyses of bubbling gas emissions and dissociated ion concentrations

Gas emissions from the seabed around the cave were sampled in July 2020, May 2021 and July 2023, using 50-ml thorion-tapped glass tubes that had been pre-weighted, evacuated, and filled with 20 mL of a 0.15 M Cd(OH)2 and 4 M NaOH suspension. The glass tubes were connected to a plastic funnel positioned over the bubbling gases. The silicon connection between the glass and the funnel was isolated by a plastic plug and filled with Milli-Q water to avoid seawater contamination (Capaccioni et al. 2005). Acid gases (CO2, H2S, SO2, HCl, and HF) were absorbed into the alkaline solution as dissolved ions and cadmium sulfide precipitate (by reaction between H2S and Cd), whereas non-reactive gas species (N2, Ar, O2, Ne, H2, He, CO, CH4) occupied the flask headspace. The inorganic low-soluble gas compounds (N2, Ar, O2, Ne, H2, He) were analysed with a gas-chromatograph (Shimadzu 15a) equipped with a Thermal Conductivity Detector (Capaccioni et al. 2005). Methane and light hydrocarbons were analysed with a gas-chromatograph (Shimadzu 14a) with a Flame Ionization Detector (Capaccioni et al. 2005). CO was converted to CH4 by using a Shimadzu MTN-1 methanizer (Tassi et al. 2004). Carbon dioxide (dissolved as CO32- in the caustic solution) was analysed by acidimetric titration using a 0.5 N HCl solution. F- and Cl- contents in the alkaline solution (from HF and HCl dissolution, respectively) were determined by ion chromatography (Capaccioni et al. 2005). Sulfate ion derived by (i) SO2 dissolution in the alkaline solution and (ii) CdS oxidation using H2O2, was analysed using a Dionex DX100 ion chromatograph equipped with an Ionpac AS9-HC column (Montegrossi et al. 2006). Seawater was also sampled to 12 cc plastic tubes filled with 2 mL of Cd(NH4)2 solution to analyse the reduced S species (including dissolved H2S, HS- and S2-) as SO42- after oxidation with H2O2 by ionic chromatography (Montegrossi et al. 2006). Gas analyses were performed at the Laboratory of Fluid Geochemistry of the University of Florence.

Table S1. Dissolved inorganic nutrients at the three sites within the CO2 vent cave and at the control site of Scilla. NH4+, ammonium; NO3-, nitrate; NO2-, nitrite; Si(OH)4, silicate; PO4, phosphate. Data are presented as mean ± 95% CIs, n = number of samples. NA, data not available. Different letters indicate statistical differences by Kruskal-Wallis followed by post-hoc Dunn’s test (p ≤ 0.05).

|  | CO2 vents cave |  |  |  | Control Site |
| --- | --- | --- | --- | --- | --- |
|  | Site 1 | Site 2 | Site 3 |  | Scilla |
|  | n = 4 | n = 4 | n = 4 |  | n = 12 |
| NH4+ (µM) | 5.95 ± 18.42 | 1.78 ± 3.99 | 4.35 ± 13.37 |  | 0.092 ± 0.04 |
|  | n = 3 | n = 3 | n = 3 |  | n = 12 |
| NO3- (µM) | 0.43 ± 0.53 | 0.37 ± 0.74 | 0.37 ± 0.32 |  | 0.24 ± 0.11 |
|  | n = 4 | n = 4 | n = 4 |  | n = 4 |
| NO2- (µM) | 0.04 ± 0.06 | 0.03 ± 0.02 | 0.03 ± 0.03 |  | NA |
| Si(OH)4 (µM) | 2.65 ± 2.91 | 3.45 ± 1.73 | 4.89 ± 3.17 |  | 3.53 ± 0.01 |
|  | n = 12 | n = 12 | n = 12 |  | n = 12 |
| PO4+ (µM) | 0.06 ± 0.02b | 0.07 ± 0.03b | 0.07 ± 0.03b |  | 0.01 ± 0.00a |

Table S2. Dissociated ion concentrations at the three sites within the CO2 vent cave.  TDS, Total Dissolved Solids. Data are presented as mean ± 95% CIs, n = number of samples. Different letters indicate statistical differences by ANOVA followed by post-hoc Tukey's test (p ≤ 0.05).

|  | CO2 vents cave |  |  |
| --- | --- | --- | --- |
|  | Site 1 | Site 2 | Site 3 |
|  | n = 3 | n = 3 | n = 3 |
| TDS (g L-1) | 39784 ± 9226 | 39778 ± 2134 | 39175 ± 7804 |
| Ca2+ (mg L-1) | 502 ± 209 | 523 ± 31 | 520 ± 116 |
| Mg2+ (mg L-1) | 1598 ± 449 | 1600 ± 56 | 1650 ± 199 |
| Na+ (mg L-1) | 12600 ± 3155 | 12511 ± 904 | 12178 ± 2418 |
| K+ (mg L-1) | 402 ± 40 | 418 ± 85 | 375 ± 159 |
| SO42- (mg L-1) | 3751 ± 2535 | 3667 ± 1554 | 3826 ± 2454 |
| Cl- (mg L-1) | 20719 ± 2781 | 20862 ± 2108 | 20430 ± 3009 |
| NO3- (mg L-1) | 32.20 ± 112.95 | 9.50 ± 15.27 | 10.90 ± 3.52 |
| F- (mg L-1) | 3.27 ± 5.15 | 2.80 ± 2.58 | 6.00 ± 15.99 |
| Br- (mg L-1) | 59.43 ± 4.75 | 58.90 ± 14.68 | 60.83 ± 3.59 |
| HS- (mg L-1) | 2.77 ± 1.90a | 4.00 ± 3.61a | 10.93 ± 8.76b |
|  | n = 2 | n = 2 | n = 2 |
| HCO3- (mg L-1) | 175 ± 438 | 190 ± 260 | 178 ± 419 |
| NH4+ (mg L-1) | 3.85 ± 22.24 | 2.20 ± 5.08 | 3.65 ± 9.53 |
| Li+ (mg L-1) | 2.00 ± 1.27 | 1.60 ± 0.00 | 2.35 ± 5.72 |