# Supporting material

# Methodological Challenges in Wood Carbon Accounting: A Maritime Flooring Case Study

Federico Bedogni¹,a, Eleonora Rossi¹,²,a, Francesco Arfelli1, Daniele Cespi¹,²*, Fabrizio Passarini¹,²

1Department of Industrial Chemistry “Toso Montanari”, University of Bologna, Via Piero Gobetti, 85, 40129 Bologna, Italy.

2Interdepartmental Centre of Industrial Research “Renewable Resources, Environment, Sea and Energy”, University of Bologna, Via Dario Campana, 71, 47922 Rimini RN, Italy.

*Corresponding author- daniele.cespi2@unibo.it

a Federico Bedogni and Eleonora Rossi contributed equally to this work (Co-first authors).

Table S1. Final score analysis (Hierarchist perspective) for each scenario and contribution (%) of GW, PMF, HTc, HTnc, LO.

| Method: ReCiPe 2016 Endpoint (H) V1.08 / World (2010) H/A |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Impact category | Unit | NW1 | NW2 | TW1 | TW2 | RES1 | RES2 |
| Total | Pt | 3.45E-01 | 2.04E-01 | 4.42E-01 | 2.98E-01 | 1.04E+00 | 1.01E+00 |
| Global warming, Human health | Pt | 8.86E-02 | 3.41E-02 | 1.08E-01 | 6.36E-02 | 3.39E-01 | 4.24E-01 |
| Global warming, Terrestrial ecosystems | Pt | 4.33E-03 | 1.67E-03 | 5.29E-03 | 3.11E-03 | 1.66E-02 | 2.08E-02 |
| Global warming, Freshwater ecosystems | Pt | 1.18E-07 | 4.55E-08 | 1.44E-07 | 8.49E-08 | 4.53E-07 | 5.67E-07 |
| Stratospheric ozone depletion | Pt | 3.99E-04 | 4.16E-04 | 4.08E-04 | 4.24E-04 | 8.24E-05 | 3.99E-04 |
| Ionizing radiation | Pt | 3.80E-05 | -6.74E-05 | 4.79E-05 | -6.02E-05 | 2.37E-04 | 1.19E-04 |
| Ozone formation, Human health | Pt | 4.60E-04 | 4.07E-04 | 4.98E-04 | 4.44E-04 | 6.35E-04 | 7.15E-04 |
| Fine particulate matter formation | Pt | 1.53E-01 | 1.11E-01 | 1.69E-01 | 1.26E-01 | 2.72E-01 | 2.38E-01 |
| Ozone formation, Terrestrial ecosystems | Pt | 1.09E-03 | 9.61E-04 | 1.19E-03 | 1.06E-03 | 1.61E-03 | 1.79E-03 |
| Terrestrial acidification | Pt | 1.61E-03 | 9.71E-04 | 1.81E-03 | 1.16E-03 | 3.67E-03 | 3.17E-03 |
| Freshwater eutrophication | Pt | 5.52E-03 | -2.75E-05 | 5.95E-03 | 3.22E-05 | 5.84E-03 | 1.21E-03 |
| Marine eutrophication | Pt | 2.93E-06 | 1.23E-07 | 3.11E-06 | 2.97E-07 | 3.14E-05 | 6.24E-07 |
| Terrestrial ecotoxicity | Pt | 2.20E-04 | 1.54E-04 | 2.50E-04 | 1.84E-04 | 1.46E-03 | 1.41E-03 |
| Freshwater ecotoxicity | Pt | 4.85E-05 | 2.14E-05 | 1.09E-04 | 5.29E-05 | 3.53E-04 | 2.78E-04 |
| Marine ecotoxicity | Pt | 1.12E-05 | 4.69E-06 | 2.30E-05 | 1.03E-05 | 7.83E-05 | 6.17E-05 |
| Human carcinogenic toxicity | Pt | 4.20E-02 | 2.53E-02 | 5.35E-02 | 5.23E-02 | 2.27E-01 | 2.18E-01 |
| Human non-carcinogenic toxicity | Pt | 2.45E-02 | 5.82E-03 | 7.15E-02 | 2.64E-02 | 1.59E-01 | 9.81E-02 |
| Land use | Pt | 2.32E-02 | 2.29E-02 | 2.39E-02 | 2.36E-02 | 1.25E-03 | 9.53E-04 |
| Mineral resource scarcity | Pt | 2.20E-05 | 1.70E-05 | 2.90E-05 | 2.39E-05 | 1.39E-04 | 1.36E-04 |
| Fossil resource scarcity | Pt | 9.54E-05 | -1.01E-04 | 1.26E-04 | -7.47E-05 | 6.67E-04 | 4.57E-04 |
| Water consumption, Human health | Pt | -3.46E-04 | -3.56E-04 | -6.94E-05 | -7.74E-05 | 4.67E-03 | 3.72E-03 |
| Water consumption, Terrestrial ecosystem | Pt | -3.36E-05 | -2.53E-05 | -6.23E-06 | 2.43E-06 | 4.62E-04 | 3.80E-04 |
| Water consumption, Aquatic ecosystems | Pt | 1.22E-09 | 1.83E-09 | 4.21E-09 | 4.82E-09 | 3.43E-08 | 3.81E-08 |
| Contribution to the final score (%) |  |  |  |  |  |  |  |
| Impact category |  | NW1 | NW2 | TW1 | TW2 | RES1 | RES2 |
| Global warming |  | 26.92% | 17.56% | 25.69% | 22.37% | 34.38% | 43.91% |
| Fine particulate matter formation |  | 44.45% | 54.74% | 38.24% | 42.25% | 26.28% | 23.47% |
| Human carcinogenic toxicity |  | 12.17% | 12.43% | 12.11% | 17.55% | 21.91% | 21.49% |
| Human non-carcinogenic toxicity |  | 7.10% | 2.86% | 16.20% | 8.85% | 15.39% | 9.67% |
| Land use |  | 6.71% | 11.24% | 5.41% | 7.92% | 0.12% | 0.09% |
| TOT |  | 98.94% | 97.65% | 98.83% | 97.35% | 98.63% | 98.08% |
| AVERAGE | 98.25% |  |  |  |  |  |  |

Table S2. Final score analysis (Egalitarian perspective) for each scenario and contribution (%) of GW, PMF, HTc, HTnc, LO.

| Method: ReCiPe 2016 Endpoint (E) V1.11 / World (2010) E/A |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Impact category | Unit | NW1 | NW2 | TW1 | TW2 | RES1 | RES2 |
| Total | Pt | 2.06E+00 | 5.83E-01 | 5.00E+00 | 2.09E+00 | 1.38E+01 | 1.00E+01 |
| Global warming, Human health | Pt | 1.88E-01 | 5.91E-02 | 2.39E-01 | 1.41E-01 | 8.74E-01 | 1.21E+00 |
| Global warming, Terrestrial ecosystems | Pt | 2.05E-02 | 6.44E-03 | 2.60E-02 | 1.54E-02 | 9.52E-02 | 1.31E-01 |
| Global warming, Freshwater ecosystems | Pt | 5.59E-07 | 1.76E-07 | 7.09E-07 | 4.20E-07 | 2.60E-06 | 3.58E-06 |
| Stratospheric ozone depletion | Pt | 3.56E-04 | 3.71E-04 | 3.64E-04 | 3.79E-04 | 6.90E-05 | 3.21E-04 |
| Ionizing radiation | Pt | 2.03E-05 | -3.46E-05 | 2.56E-05 | -3.07E-05 | 1.26E-04 | 4.81E-05 |
| Ozone formation, Human health | Pt | 1.05E-04 | 9.33E-05 | 1.14E-04 | 1.02E-04 | 1.45E-04 | 1.78E-04 |
| Fine particulate matter formation | Pt | 3.51E-02 | 2.55E-02 | 3.87E-02 | 2.88E-02 | 6.23E-02 | 5.53E-02 |
| Ozone formation, Terrestrial ecosystems | Pt | 8.39E-04 | 7.39E-04 | 9.15E-04 | 8.13E-04 | 1.24E-03 | 1.48E-03 |
| Terrestrial acidification | Pt | 1.24E-03 | 7.47E-04 | 1.39E-03 | 8.89E-04 | 2.82E-03 | 2.19E-03 |
| Freshwater eutrophication | Pt | 3.35E-03 | -4.70E-05 | 3.62E-03 | -4.62E-06 | 3.63E-03 | 8.74E-04 |
| Marine eutrophication | Pt | 2.26E-06 | 9.49E-08 | 2.39E-06 | 2.28E-07 | 2.41E-05 | 4.00E-07 |
| Terrestrial ecotoxicity | Pt | 1.82E-04 | 1.27E-04 | 2.07E-04 | 1.52E-04 | 1.21E-03 | 1.47E-03 |
| Freshwater ecotoxicity | Pt | 3.48E-05 | 1.38E-05 | 8.12E-05 | 3.81E-05 | 2.73E-04 | 2.17E-04 |
| Marine ecotoxicity | Pt | 3.31E-02 | 3.40E-03 | 1.10E-01 | 3.12E-02 | 2.72E-01 | 1.59E-01 |
| Human carcinogenic toxicity | Pt | 6.92E-01 | 4.17E-01 | 8.81E-01 | 8.68E-01 | 3.71E+00 | 3.70E+00 |
| Human non-carcinogenic toxicity | Pt | 1.07E+00 | 5.05E-02 | 3.68E+00 | 9.81E-01 | 8.76E+00 | 4.76E+00 |
| Land use | Pt | 1.83E-02 | 1.81E-02 | 1.89E-02 | 1.86E-02 | 1.09E-03 | 8.81E-04 |
| Mineral resource scarcity | Pt | 2.20E-05 | 1.70E-05 | 2.90E-05 | 2.39E-05 | 1.39E-04 | 1.36E-04 |
| Fossil resource scarcity | Pt | 3.40E-03 | 1.73E-03 | 4.67E-03 | 2.97E-03 | 2.28E-02 | 2.11E-02 |
| Water consumption, Human health | Pt | -7.93E-05 | -8.15E-05 | -2.12E-05 | -2.30E-05 | 1.07E-03 | 6.50E-04 |
| Water consumption, Terrestrial ecosystem | Pt | -2.58E-05 | -1.95E-05 | -6.31E-06 | 3.44E-07 | 3.55E-04 | 2.30E-04 |
| Water consumption, Aquatic ecosystems | Pt | 9.30E-10 | 1.40E-09 | 2.40E-09 | 2.87E-09 | 2.65E-08 | 2.98E-08 |
| Global warming |  | 10% | 11% | 5% | 8% | 7% | 13% |
| Fine particulate matter formation |  | 2% | 4% | 1% | 1% | 0% | 1% |
| Human carcinogenic toxicity |  | 34% | 71% | 18% | 42% | 27% | 37% |
| Human non-carcinogenic toxicity |  | 52% | 9% | 74% | 47% | 63% | 47% |
| Land use |  | 1% | 3% | 0% | 1% | 0% | 0% |
| TOT |  | 98% | 99% | 98% | 98% | 98% | 98% |
| AVERAGE |  | 98% |  |  |  |  |  |

Table S3. Final score analysis (Individualist perspective) for each scenario and contribution (%) of GW, PMF, HTc, HTnc, LO.

| Method: ReCiPe 2016 Endpoint (I) V1.11 / World (2010) I/A |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Impact category | Unit | NW1 | NW2 | TW1 | TW2 | RES1 | RES2 |
| Total | mPt | 2.21E+02 | 1.77E+02 | 2.49E+02 | 2.05E+02 | 3.29E+02 | 3.43E+02 |
| Global warming, Human health | mPt | 2.03E+01 | 6.13E+00 | 2.49E+01 | 1.24E+01 | 7.98E+01 | 9.55E+01 |
| Global warming, Terrestrial ecosystems | mPt | 1.48E+00 | 4.45E-01 | 1.81E+00 | 9.02E-01 | 5.80E+00 | 6.94E+00 |
| Global warming, Freshwater ecosystems | mPt | 4.03E-05 | 1.21E-05 | 4.93E-05 | 2.46E-05 | 1.58E-04 | 1.89E-04 |
| Stratospheric ozone depletion | mPt | 2.45E-01 | 2.55E-01 | 2.50E-01 | 2.60E-01 | 4.96E-02 | 2.23E-01 |
| Ionizing radiation | mPt | 6.44E-02 | -1.14E-01 | 8.11E-02 | -1.02E-01 | 4.01E-01 | 1.43E-01 |
| Ozone formation, Human health | mPt | 9.91E-01 | 8.79E-01 | 1.07E+00 | 9.58E-01 | 1.37E+00 | 1.67E+00 |
| Fine particulate matter formation | mPt | 1.49E+02 | 1.31E+02 | 1.60E+02 | 1.41E+02 | 1.78E+02 | 1.93E+02 |
| Ozone formation, Terrestrial ecosystems | mPt | 1.61E+00 | 1.42E+00 | 1.75E+00 | 1.56E+00 | 2.37E+00 | 2.85E+00 |
| Terrestrial acidification | mPt | 2.37E+00 | 1.43E+00 | 2.67E+00 | 1.70E+00 | 5.41E+00 | 4.20E+00 |
| Freshwater eutrophication | mPt | 6.42E+00 | -9.02E-02 | 6.94E+00 | -8.85E-03 | 6.96E+00 | 1.68E+00 |
| Marine eutrophication | mPt | 4.32E-03 | 1.82E-04 | 4.58E-03 | 4.38E-04 | 4.62E-02 | 7.67E-04 |
| Terrestrial ecotoxicity | mPt | 1.38E-01 | 9.50E-02 | 1.58E-01 | 1.15E-01 | 9.37E-01 | 1.14E+00 |
| Freshwater ecotoxicity | mPt | 6.75E-02 | 2.87E-02 | 1.55E-01 | 7.36E-02 | 4.90E-01 | 3.79E-01 |
| Marine ecotoxicity | mPt | 4.25E-03 | 1.28E-03 | 8.89E-03 | 3.75E-03 | 2.78E-02 | 2.18E-02 |
| Human carcinogenic toxicity | mPt | 4.82E-01 | 3.29E-01 | 5.70E-01 | 4.11E-01 | 3.57E+00 | 1.68E+00 |
| Human non-carcinogenic toxicity | mPt | 1.32E+00 | 6.17E-01 | 8.89E+00 | 7.79E+00 | 8.29E+00 | 5.29E+00 |
| Land use | mPt | 3.51E+01 | 3.47E+01 | 3.62E+01 | 3.57E+01 | 2.10E+00 | 1.69E+00 |
| Mineral resource scarcity | mPt | 1.09E-02 | 8.09E-03 | 1.44E-02 | 1.16E-02 | 7.06E-02 | 6.92E-02 |
| Fossil resource scarcity | mPt | 2.99E+00 | 1.59E+00 | 4.13E+00 | 2.69E+00 | 2.03E+01 | 1.89E+01 |
| Water consumption, Human health | mPt | -1.24E+00 | -1.28E+00 | -5.23E-01 | -5.55E-01 | 1.28E+01 | 6.90E+00 |
| Water consumption, Terrestrial ecosystem | mPt | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| Water consumption, Aquatic ecosystems | mPt | 1.78E-06 | 2.69E-06 | 4.60E-06 | 5.50E-06 | 5.07E-05 | 5.71E-05 |
| Global warming |  | 10% | 4% | 11% | 6% | 26% | 30% |
| Fine particulate matter formation |  | 67% | 74% | 64% | 69% | 54% | 56% |
| Human carcinogenic toxicity |  | 0% | 0% | 0% | 0% | 1% | 0% |
| Human non-carcinogenic toxicity |  | 1% | 0% | 4% | 4% | 3% | 2% |
| Land use |  | 16% | 20% | 15% | 17% | 1% | 0% |
| TOT |  | 94% | 98% | 93% | 97% | 84% | 89% |
| AVERAGE |  | 92% |  |  |  |  |  |

Image S1: Indonesian energy mix (source https://www.iea.org/countries/indonesia/energy-mix)

Image S2: European energy mix (source https://www.iea.org/regions/europe/energy-mix)

Table S4. Source of information regarding polyurethane glue

| Source | Product | Consumption |
| --- | --- | --- |
| Utensileria Online | Not specified | 150 - 220 g/m² |
| Faichim 1 | Extrapur | 100 - 200 g/m² |
| Faichim 2 | Extrapur Plus | 100 - 200 g/m² |
| Polibond | Polibond 701 | 150 g/m² |
| Tool Market 1 | Alcacol PU-45 | 100 - 200 g/m² |
| Tool Market 2 | Bindan | 150 g/m² |
| Tool Market 3 | Not specified | Approximately 200 g/m² |

Table S5: Source of information regarding the nominal power of the sanders.

| Source | Product | Power (W) |
| --- | --- | --- |
| Leroy Merlin 1 | DeWalt DWE6423-QS | 280 W |
| Leroy Merlin 2 | Bosch PSS200AC | 200 W |
| Leroy Merlin 3 | Bosch Professional GSS23AE | 190 W |
| Direct Industry 1 | Ingersoll Rand | 200 W |
| Direct Industry 2 | Holger Clasen | 375 W |
| Data | Source | Amount |
| Time necessary for teak sanding | Expert consultation from Calzolari Snc | 30min/m2 |

Table S6: Source of information regarding wood treatment.

| Source | Product | Coverage | Density |
| --- | --- | --- | --- |
| Magellano Store | Teak Wonder Dressing & Sealer | 5-6 m²/l | Not specified |
| SVB Marine 1 | Epifanes Semi-Gloss Paint | 10 m²/l | 0.97 kg/dm³ |
| SVB Marine 2 | Not specified | 14 m²/l | Not specified |

Table S7: Source of information regarding resin composition.

| Source | Composition | Thickness | Resin Quantity |
| --- | --- | --- | --- |
| Bolidt 1 - NauticExpo (525) | Not specified | 2-4 mm | Not specified |
| Bolidt 2 - NauticExpo (700) | Epoxy resin | 5 mm | Not specified |
| Bolidt 3 - NauticExpo (Select) | Not specified | 10-15 mm | Not specified |
| Bolidt 4 - NauticExpo (FiftyFifty) | Not specified | 2-3 mm | Not specified |
| Bolidt 5 - NauticExpo (Jewel) | Not specified | 3.5 mm | Not specified |
| Bolidt 6 - NauticExpo (Sensation) | Not specified | 3-6 mm | Not specified |
| Resinpro | Not Specified | 1.5 mm | 1.6 kg/m² |
| Minirasex | Not Specified | Up to 2 mm | 1-1.5 kg/m² |
| Resin4Decor | Not Specified | 1 mm | 1 kg/m² |
| Prolance | Epoxy or Poyurethane resin | Not specifed | Not specifed |
| Patent WO2013159277A1 | Epoxy resin | Not specifed | Not specifed |
| SIRN | Epoxy or Polyurethane resin | Not specifed | Not specifed |
| EpoxyEurope | Epoxy resin | Not specifed | Not specifed |
| Reme Flooring | Epoxy or Polyurethane resin | Not specifed | Not specifed |
| ItalKem | Epoxy resin | Not specifed | Not specifed |

Image S3: Flowchart production process and life cycle of 1 m2 of Scenario 3, resin flooring

Table S8: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), WOP production system boundaries (Cradle-to-Gate), -1/+1.

| WOP PRODUCTION SYSTEM BOUNDARIES (Cradle-to-Gate) -1/+1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 – fossil (kg CO2 eq) | GWP100 – biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
| Logging and harvesting | logs | 21.074 | kg | 1.59E+00 | 1.01E-02 | 8.01E-03 | -1.38E+01 | -1.21E+0 |
| Transportation 1 | truck transportation | 2.833 | tkm | 5.55E-01 | 3.30E-03 | 2.22E-04 | -2.53E-03 | 5.56E-01 |
| Sawing and milling | energy consumption | 0.100 | kWh | 1.17E-01 | 1.89E-03 | 2.42E-04 | -1.35E-04 | 1.19E-01 |
| Drying | wooden biomass burnt | 10.110 | MJ | 9.11E-02 | 1.08E+00 | 1.67E-04 | -5.36E-04 | 1.17E+00 |
| Planning and shaping | energy consumption | 0.020 | kWh | 2.34E-02 | 3.77E-04 | 4.84E-05 | -2.69E-05 | 2.38E-02 |
| Transportation 2 | truck transportation | 0.105 | tkm | 2.06E-02 | 1.22E-04 | 8.24E-06 | -9.39E-05 | 2.06E-02 |
| Transportation 3 | ship transportation | 84.000 | tkm | 8.56E-01 | 1.81E-03 | 4.30E-04 | -1.83E-03 | 8.57E-01 |
| TOTAL IMPACT |  |  |  | 3.26E+00 | 1.10E+00 | 9.12E-03 | -1.38E+01 | -9.39E+0 |

Table S9: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), WOP production system boundaries (Cradle-to-Gate), 0/0.

| WOP PRODUCTION SYSTEM BOUNDARIES (Cradle-to-Gate) 0/0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 – fossil (kg CO2 eq) | GWP100 – biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
| Logging and harvesting | logs | 21.074 | kg | 1.59E+00 | 1.01E-02 | 8.01E-03 | 0.00E+00 | 1.61E+00 |
| Transportation 1 | truck transportation | 2.833 | tkm | 5.55E-01 | 3.30E-03 | 2.22E-04 | -2.53E-03 | 5.56E-01 |
| Sawing and milling | energy consumption | 0.100 | kWh | 1.17E-01 | 1.89E-03 | 2.42E-04 | -1.35E-04 | 1.19E-01 |
| Drying | wooden biomass burnt | 10.110 | MJ | 9.11E-02 | 1.73E-03 | 1.67E-04 | -5.36E-04 | 9.25E-02 |
| Planning and shaping | energy consumption | 0.020 | kWh | 2.34E-02 | 3.77E-04 | 4.84E-05 | -2.69E-05 | 2.38E-02 |
| Transportation 2 | truck transportation | 0.105 | tkm | 2.06E-02 | 1.22E-04 | 8.24E-06 | -9.39E-05 | 2.06E-02 |
| Transportation 3 | ship transportation | 84.000 | tkm | 8.56E-01 | 1.81E-03 | 4.30E-04 | -1.83E-03 | 8.57E-01 |
| TOTAL IMPACT |  |  |  | 3.26E+00 | 1.94E-02 | 9.12E-03 | -5.15E-03 | 3.28E+00 |

Table S10: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), Non-treated wood life cycle, LANDFILLING, SCENARIO NW1, -1/+1.

| NON-TREATED WOOD LIFE CYCLE, LANDFILLING, SCENARIO NW1, -1/+1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 – fossil (kg CO2 eq) | GWP100 – biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) | TOT ISO 14067 |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.10E+00 | 9.12E-03 | -1.38E+01 | -9.39E+00 |  |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |  |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |  |
| End of life | planks waste, landfill | 7.000 | kg | 7.97E-02 | 5.44E-01 | 3.32E-05 | -6.45E-04 | 6.23E-01 |  |
|  | glue waste, landfill | 0.220 | kg | 3.37E-02 | 2.11E-06 | 1.01E-07 | -1.55E-06 | 3.37E-02 |  |
| TOTAL IMPACT |  |  |  | 5.03E+00 | 1.73E+00 | 1.38E-02 | -1.38E+01 | -7.04E+00 | 3.11E+00 |

Table S11: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), Non-treated wood life cycle, WTE, SCENARIO NW2, -1/+1.

| NON-TREATED WOOD LIFE CYCLE, WTE, SCENARIO NW2, -1/+1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 - fossil (kg CO2 eq) | GWP100 – biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.10E+00 | 9.12E-03 | -1.38E+01 | -9.39E+00 |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |
| End of life | planks waste, WtE | 7.000 | kg | -3.20E+00 | 9.81E+00 | -6.79E-03 | 1.22E-01 | 6.73E+00 |
|  | glue waste, WtE | 0.220 | kg | 3.65E-01 | -3.00E-02 | -4.75E-04 | 8.42E-03 | 3.43E-01 |
| TOTAL IMPACT |  |  |  | 2.09E+00 | 1.10E+01 | 6.51E-03 | -1.37E+01 | -6.23E-01 |

Table S12: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), Non-treated wood life cycle, LANDFILLING, SCENARIO TW1, -1/+1.

| TREATED WOOD LIFE CYCLE, LANDFILLING, SCENARIO TW1, -1/+1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 - fossil (kg CO2 eq) | GWP100 - biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) | TOT ISO 14067 |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.10E+00 | 9.12E-03 | -1.38E+01 | -9.39E+00 |  |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |  |
|  | wood coating | 0.320 | kg | 1.02E+00 | 4.42E-02 | 1.72E-01 | -2.30E-01 | 1.01E+00 |  |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |  |
| End of life | planks waste, landfill | 7.000 | kg | 7.97E-02 | 5.44E-01 | 3.32E-05 | -6.45E-04 | 6.23E-01 |  |
|  | coating waste, landfill | 0.320 | kg | 3.03E-02 | 6.01E-05 | 1.41E-06 | -3.09E-05 | 3.03E-02 |  |
|  | glue waste, landfill | 0.220 | kg | 3.37E-02 | 2.11E-06 | 1.01E-07 | -1.55E-06 | 3.37E-02 |  |
| TOTAL IMPACT |  |  |  | 6.09E+00 | 1.78E+00 | 1.86E-01 | -1.40E+01 | -6.00E+00 | 4.14E+00 |

Table S13: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), Non-treated wood life cycle, WTE, SCENARIO TW2, -1/+1.

| TREATED WOOD LIFE CYCLE, WTE, SCENARIO TW2, -1/+1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 – fossil (kg CO2 eq) | GWP100 – biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.10E+00 | 9.12E-03 | -1.38E+01 | -9.39E+00 |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |
|  | wood coating | 0.320 | kg | 1.02E+00 | 4.42E-02 | 1.72E-01 | -2.30E-01 | 1.01E+00 |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |
| End of life | planks waste, WtE | 7.000 | kg | -3.20E+00 | 9.81E+00 | -6.79E-03 | 1.22E-01 | 6.73E+00 |
|  | coating waste, WtE | 0.320 | kg | 6.75E-01 | -1.17E-02 | -1.82E-04 | 3.28E-03 | 6.66E-01 |
|  | glue waste, WtE | 0.220 | kg | 3.65E-01 | -3.00E-02 | -4.75E-04 | 8.42E-03 | 3.43E-01 |
| TOTAL IMPACT |  |  |  | 3.78E+00 | 1.10E+01 | 1.78E-01 | -1.39E+01 | 1.05E+00 |

Table S14: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), Non-treated wood life cycle, LANDFILLING, SCENARIO NW1, 0/0.

| NON-TREATED WOOD LIFE CYCLE, LANDFILLING, SCENARIO NW1, 0/0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 – fossil (kg CO2 eq) | GWP100 – biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.94E-02 | 9.12E-03 | -5.15E-03 | 3.28E+00 |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |
| End of life | planks waste, landfill | 7.000 | kg | 7.97E-02 | 4.33E-01 | 3.32E-05 | -6.45E-04 | 5.12E-01 |
|  | glue waste, landfill | 0.220 | kg | 3.37E-02 | 2.11E-06 | 1.01E-07 | -1.55E-06 | 3.37E-02 |
| TOTAL IMPACT |  |  |  | 5.03E+00 | 5.43E-01 | 1.38E-02 | -6.40E-02 | 5.53E+00 |

Table S15: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), treated wood life cycle, WTE, SCENARIO NW2, 0/0.

| NON-TREATED WOOD LIFE CYCLE, WTE, SCENARIO NW2, 0/0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 - fossil (kg CO2 eq) | GWP100 - biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.94E-02 | 9.12E-03 | -5.15E-03 | 3.28E+00 |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |
| End of life | planks waste, WtE | 7.000 | kg | -3.20E+00 | -4.32E-01 | -6.79E-03 | 1.22E-01 | 6.73E+00 |
|  | glue waste, WtE | 0.220 | kg | 3.65E-01 | -3.00E-02 | -4.75E-04 | 8.42E-03 | 3.43E-01 |
| TOTAL IMPACT |  |  |  | 2.09E+00 | -3.52E-01 | 6.51E-03 | 6.70E-02 | 1.81E+00 |

Table S16: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), treated wood life cycle, LANDFILLING, SCENARIO TW1, 0/0.

| TREATED WOOD LIFE CYCLE, LANDFILLING, SCENARIO TW1, 0/0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 - fossil (kg CO2 eq) | GWP100 - biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.94E-02 | 9.12E-03 | -5.15E-03 | 3.28E+00 |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |
|  | wood coating | 0.320 | kg | 1.02E+00 | 4.42E-02 | 1.72E-01 | -2.30E-01 | 1.01E+00 |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |
| End of life | planks waste, landfill | 7.000 | kg | 7.97E-02 | 4.33E-01 | 3.32E-05 | -6.45E-04 | 5.12E-01 |
|  | coating waste, landfill | 0.320 | kg | 3.03E-02 | 6.01E-05 | 1.41E-06 | -3.09E-05 | 3.03E-02 |
|  | glue waste, landfill | 0.220 | kg | 3.37E-02 | 2.11E-06 | 1.01E-07 | -1.55E-06 | 3.37E-02 |
| TOTAL IMPACT |  |  |  | 6.09E+00 | 5.87E-01 | 1.86E-01 | -2.94E-01 | 6.56E+00 |

Table S17: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), treated wood life cycle, WTE, SCENARIO TW2, 0/0. SD: Standard deviation.

| TREATED WOOD LIFE CYCLE, WTE, SCENARIO TW2, 0/0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 – fossil (kg CO2 eq) | GWP100 – biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
|  | WOP PRODUCTION |  |  | 3.26E+00 | 1.94E-02 | 9.12E-03 | -5.15E-03 | 3.28E+00 |
| Use phase | electricity | 0.875 | kWh | 2.93E-01 | 5.58E-02 | 1.22E-03 | -2.40E-02 | 3.26E-01 |
|  | wood coating | 0.320 | kg | 1.02E+00 | 4.42E-02 | 1.72E-01 | -2.30E-01 | 1.01E+00 |
|  | timber glue | 0.220 | kg | 1.37E+00 | 3.47E-02 | 3.44E-03 | -3.43E-02 | 1.38E+00 |
| End of life | planks waste, WtE | 7.000 | kg | -3.20E+00 | -4.32E-01 | -6.79E-03 | 1.22E-01 | 6.73E+00 |
|  | coating waste, WtE | 0.320 | kg | 6.75E-01 | -1.17E-02 | -1.82E-04 | 3.28E-03 | 6.66E-01 |
|  | glue waste, WtE | 0.220 | kg | 3.65E-01 | -3.00E-02 | -4.75E-04 | 8.42E-03 | 3.43E-01 |
| TOTAL IMPACT |  |  |  | 3.78E+00 | -3.20E-01 | 1.78E-01 | -1.60E-01 | 3.48E+00 |
| SD |  |  |  | 21% | 7% | 59% | 11% | / |

Table S18: LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01), Resin life cycle. SD: Standard deviation.

| RESIN LIFE CYCLE BOUNDARIES |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GWP100 - fossil (kg CO2 eq) | GWP100 - biogenic (kg CO2 eq) | GWP100 - land transformation (kg CO2 eq) | GWP100 - CO2 uptake (kg CO2 eq) | TOT (100y term emiss.) |
| Production | Epoxy resin | 2.613 | kg | 1.53E+01 | 5.25E-01 | 1.28E-02 | -3.64E-01 | 1.55E+01 |
|  | Polyurethane resin | 1.120 | kg | 5.29E+00 | 3.05E-01 | 5.48E-03 | -1.66E-01 | 5.43E+00 |
| End of life | 100% WtE | 3.733 | kg | 6.19E+00 | -5.10E-01 | -8.06E-03 | 1.43E-01 | 5.81E+00 |
|  | 100% Landfilling | 3.733 | kg | 5.71E-01 | 3.57E-05 | 1.72E-06 | -2.62E-05 | 5.71E-01 |
| TOTAL IMPACT, LANDFILLING, RES1 |  |  |  | 2.12E+01 | 8.30E-01 | 1.83E-02 | -5.31E-01 | 2.15E+01 |
| TOTAL IMPACT, WTE, RES2 |  |  |  | 2.68E+01 | 3.21E-01 | 1.02E-02 | -3.88E-01 | 2.67E+01 |
| SD |  |  |  | 14% | 38% | 37% | 28% | / |

Table S19: Impact of WoP. LCIA Method: ReCiPe 2016 Midpoint (H) V1.08 / World (2010) H. Ozone depletion (OD), Ionizing radiation (IR), Ozone formation human health (OFH), Particulate matter formation (PMF), Ozone formation terrestrial ecosystems (OFT), Terrestrial acidification (TA), Freshwater eutrophication (FE), Marine eutrophication (ME), Terrestrial ecotoxicity (TET), Freshwater ecotoxicity (FET), Marine ecotoxicity (MET), Human toxicity (cancer effects, HTc), Human toxicity (non-cancer effects, HTnc), Land occupation (LO), Mineral resource scarcity (MRS), Fossil resource scarcity (FRS), Water consumption (WC), SD: Standard deviation.

| WoP PRODUCTION SYSTEM BOUNDARIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
| Logging and harvesting | logs | 21,074 | kg | 4,08E-05 | 6,80E-03 | 4,25E-03 | 9,31E-04 | 4,60E-03 | 2,79E-03 | 3,07E-04 | 2,58E-04 | 6,76E+00 | 1,29E-02 | 2,29E-02 | 1,14E-01 | 3,01E-01 | 9,57E+00 | 1,93E-03 | 1,56E-02 | 2,65E-03 |
| transportation 1 | truck transportation | 2,833 | tkm | 8,47E-08 | 5,44E-03 | 3,44E-03 | 7,59E-04 | 3,56E-03 | 1,73E-03 | 6,17E-05 | 5,06E-06 | 1,46E+01 | 1,48E-02 | 3,06E-02 | 1,07E-01 | 4,68E-01 | 2,25E-02 | 1,43E-03 | 1,37E-02 | 1,06E-03 |
| sawing and milling | energy consumption | 0,100 | kWh | 2,82E-08 | 1,92E-04 | 3,18E-04 | 2,74E-03 | 3,22E-04 | 3,94E-04 | 2,17E-04 | 1,35E-05 | 1,74E-01 | 5,94E-03 | 8,21E-03 | 1,26E-02 | 2,20E-01 | 4,87E-04 | 3,83E-05 | 2,07E-02 | 5,47E-04 |
| drying | wooden biomass burnt | 10,110 | MJ | 3,5E-07 | 7,37E-04 | 1,55E-03 | 1,77E-03 | 1,55E-03 | 7,99E-04 | 1,11E-04 | 6,75E-06 | 1,66E+00 | 7,17E-03 | 1,13E-02 | 4,15E-02 | 3,42E-01 | 2,55E-03 | 4,68E-04 | 1,13E-02 | 5,52E-04 |
| planning and shaping | energy consumption | 0,020 | kWh | 5,64E-09 | 3,84E-05 | 6,36E-05 | 5,47E-04 | 6,43E-05 | 7,87E-05 | 4,34E-05 | 2,70E-06 | 3,47E-02 | 1,19E-03 | 1,64E-03 | 2,52E-03 | 4,39E-02 | 9,75E-05 | 7,67E-06 | 4,14E-03 | 1,09E-04 |
| transportation 2 | truck transportation | 0,105 | tkm | 3,14E-09 | 2,02E-04 | 1,27E-04 | 2,81E-05 | 1,32E-04 | 6,40E-05 | 2,29E-06 | 1,88E-07 | 5,40E-01 | 5,49E-04 | 1,14E-03 | 3,97E-03 | 1,74E-02 | 8,35E-04 | 5,32E-05 | 5,07E-04 | 3,91E-05 |
| transportation 3 | ship transportation | 84,000 | tkm | 4,15E-07 | 4,00E-03 | 1,64E-02 | 5,12E-03 | 1,66E-02 | 1,59E-02 | 3,72E-05 | 4,82E-06 | 8,83E+00 | 8,83E-03 | 2,01E-02 | 1,36E-01 | 1,23E-01 | 6,22E-03 | 1,67E-03 | 7,32E-03 | 7,75E-04 |
| TOTAL IMPACT |  |  |  | 4,17E-05 | 1,74E-02 | 2,62E-02 | 1,19E-02 | 2,69E-02 | 2,18E-02 | 7,80E-04 | 2,91E-04 | 3,26E+01 | 5,14E-02 | 9,60E-02 | 4,17E-01 | 1,51E+00 | 9,60E+00 | 5,60E-03 | 7,33E-02 | 5,73E-03 |
| NON-TREATED WOOD LIFE CYCLE, LANDFILLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase | Parameter | Amount | Unit | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
|  | WoP PRODUCTION |  |  | 4,17E-05 | 1,74E-02 | 2,62E-02 | 1,19E-02 | 2,69E-02 | 2,18E-02 | 7,80E-04 | 2,91E-04 | 3,26E+01 | 5,14E-02 | 9,60E-02 | 4,17E-01 | 1,51E+00 | 9,60E+00 | 5,60E-03 | 7,33E-02 | 5,73E-03 |
| use phase | Electricity | 0,875 | kWh | 1,66E-07 | 1,72E-01 | 5,69E-04 | 4,89E-04 | 5,89E-04 | 1,22E-03 | 3,07E-04 | 2,59E-05 | 2,89E+00 | 4,38E-02 | 5,83E-02 | 4,81E-02 | 6,81E-01 | 1,57E-02 | 1,39E-03 | 3,77E-02 | 7,02E-03 |
|  | timber glue | 0,220 | kg | 3,17E-06 | 7,77E-02 | 2,99E-03 | 2,08E-03 | 3,22E-03 | 4,76E-03 | 7,05E-04 | 7,03E-05 | 3,47E+01 | 8,82E-02 | 1,34E-01 | 2,72E-01 | 1,82E+00 | 3,86E-02 | 6,11E-03 | 1,23E-01 | 1,27E-02 |
| end of life | planks waste, landfill | 7,000 | kg | 3,96E-08 | 1,48E-03 | 5,68E-04 | 1,56E-04 | 5,94E-04 | 3,15E-04 | 2,73E-02 | 2,02E-03 | 1,12E+00 | 3,31E-02 | 4,67E-02 | 2,07E-02 | 1,30E+00 | 7,60E-03 | 2,53E-04 | 2,75E-03 | -2,59E-02 |
|  | glue waste, landfill | 0,220 | kg | 3,95E-10 | 5,39E-06 | 1,22E-05 | 2,65E-06 | 1,25E-05 | 5,14E-06 | 1,37E-03 | 3,97E-03 | 9,62E-03 | 4,27E-02 | 5,77E-02 | 7,55E-04 | 1,13E+00 | 3,82E-04 | 1,75E-06 | 8,74E-06 | 1,03E-06 |
| TOTAL IMPACT |  |  |  | 4,51E-05 | 2,69E-01 | 3,03E-02 | 1,46E-02 | 3,13E-02 | 2,81E-02 | 3,04E-02 | 6,37E-03 | 7,13E+01 | 2,59E-01 | 3,93E-01 | 7,59E-01 | 6,44E+00 | 9,66E+00 | 1,34E-02 | 2,37E-01 | -4,75E-04 |
| NON-TREATED WOOD LIFE CYCLE, WTE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase | Parameter | Amount | Unit | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
|  | WoP PRODUCTION |  |  | 4,17E-05 | 1,74E-02 | 2,62E-02 | 1,19E-02 | 2,69E-02 | 2,18E-02 | 7,80E-04 | 2,91E-04 | 3,26E+01 | 5,14E-02 | 9,60E-02 | 4,17E-01 | 1,51E+00 | 9,60E+00 | 5,60E-03 | 7,33E-02 | 5,73E-03 |
| use phase | Electricity | 0,875 | kWh | 1,66E-07 | 1,72E-01 | 5,69E-04 | 4,89E-04 | 5,89E-04 | 1,22E-03 | 3,07E-04 | 2,59E-05 | 2,89E+00 | 4,38E-02 | 5,83E-02 | 4,81E-02 | 6,81E-01 | 1,57E-02 | 1,39E-03 | 3,77E-02 | 7,02E-03 |
|  | timber glue | 0,220 | kg | 3,17E-06 | 7,77E-02 | 2,99E-03 | 2,08E-03 | 3,22E-03 | 4,76E-03 | 7,05E-04 | 7,03E-05 | 3,47E+01 | 8,82E-02 | 1,34E-01 | 2,72E-01 | 1,82E+00 | 3,86E-02 | 6,11E-03 | 1,23E-01 | 1,27E-02 |
| end of life | planks waste, WtE | 7,000 | kg | -2,24E-07 | -6,94E-01 | -3,19E-03 | -3,66E-03 | -3,42E-03 | -1,03E-02 | -1,81E-03 | -1,51E-04 | -1,93E+01 | -8,79E-02 | -1,47E-01 | -2,71E-01 | -2,66E+00 | -1,01E-01 | -2,71E-03 | -4,04E-01 | -3,88E-02 |
|  | glue waste, WtE | 0,220 | kg | 2,11E-06 | -4,92E-02 | 3,23E-04 | -1,89E-04 | 3,10E-04 | -5,02E-04 | -1,32E-04 | 3,20E-05 | -9,04E-01 | 1,90E-02 | 2,32E-02 | -8,82E-03 | 1,82E-01 | -6,89E-03 | -9,86E-05 | -2,76E-02 | -1,98E-03 |
| TOTAL IMPACT |  |  |  | 4,69E-05 | -4,76E-01 | 2,68E-02 | 1,06E-02 | 2,76E-02 | 1,69E-02 | -1,53E-04 | 2,68E-04 | 4,99E+01 | 1,14E-01 | 1,65E-01 | 4,57E-01 | 1,53E+00 | 9,55E+00 | 1,03E-02 | -1,98E-01 | -1,54E-02 |
| TREATED WOOD LIFE CYCLE, LANDFILLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase | Parameter | Amount | Unit | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
|  | WoP PRODUCTION |  |  | 4,17E-05 | 1,74E-02 | 2,62E-02 | 1,19E-02 | 2,69E-02 | 2,18E-02 | 7,80E-04 | 2,91E-04 | 3,26E+01 | 5,14E-02 | 9,60E-02 | 4,17E-01 | 1,51E+00 | 9,60E+00 | 5,60E-03 | 7,33E-02 | 5,73E-03 |
| use phase | Electricity | 0,875 | kWh | 1,66E-07 | 1,72E-01 | 5,69E-04 | 4,89E-04 | 5,89E-04 | 1,22E-03 | 3,07E-04 | 2,59E-05 | 2,89E+00 | 4,38E-02 | 5,83E-02 | 4,81E-02 | 6,81E-01 | 1,57E-02 | 1,39E-03 | 3,77E-02 | 7,02E-03 |
|  | wood coating | 0,320 | kg | 9,92E-07 | 6,92E-02 | 2,48E-03 | 1,47E-03 | 2,79E-03 | 3,47E-03 | 3,67E-04 | 3,82E-04 | 9,86E+00 | 9,48E-02 | 9,64E-02 | 1,93E-01 | 1,89E+00 | 2,97E-01 | 4,20E-03 | 6,33E-02 | 1,13E-02 |
|  | timber glue | 0,220 | kg | 3,17E-06 | 7,77E-02 | 2,99E-03 | 2,08E-03 | 3,22E-03 | 4,76E-03 | 7,05E-04 | 7,03E-05 | 3,47E+01 | 8,82E-02 | 1,34E-01 | 2,72E-01 | 1,82E+00 | 3,86E-02 | 6,11E-03 | 1,23E-01 | 1,27E-02 |
| end of life | planks waste, landfill | 7,000 | kg | 3,96E-08 | 1,48E-03 | 5,68E-04 | 1,56E-04 | 5,94E-04 | 3,15E-04 | 2,73E-02 | 2,02E-03 | 1,12E+00 | 3,31E-02 | 4,67E-02 | 2,07E-02 | 1,30E+00 | 7,60E-03 | 2,53E-04 | 2,75E-03 | -2,59E-02 |
|  | coating waste, landfill | 0,320 | kg | 1,14E-09 | 8,57E-05 | 2,55E-05 | 6,65E-06 | 2,68E-05 | 1,35E-05 | 2,03E-03 | 6,93E-08 | 4,75E-02 | 2,27E-01 | 3,19E-01 | 1,33E-02 | 1,05E+01 | 3,45E-04 | 1,05E-05 | 9,70E-05 | -7,34E-04 |
|  | glue waste, landfill | 0,220 | kg | 3,95E-10 | 5,39E-06 | 1,22E-05 | 2,65E-06 | 1,25E-05 | 5,14E-06 | 1,37E-03 | 3,97E-03 | 9,62E-03 | 4,27E-02 | 5,77E-02 | 7,55E-04 | 1,13E+00 | 3,82E-04 | 1,75E-06 | 8,74E-06 | 1,03E-06 |
| TOTAL IMPACT |  |  |  | 4,61E-05 | 3,38E-01 | 3,28E-02 | 1,61E-02 | 3,41E-02 | 3,15E-02 | 3,28E-02 | 6,75E-03 | 8,12E+01 | 5,81E-01 | 8,09E-01 | 9,65E-01 | 1,88E+01 | 9,96E+00 | 1,76E-02 | 3,00E-01 | 1,01E-02 |
| TREATED WOOD LIFE CYCLE, WTE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase | Parameter | Amount | Unit | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
|  | WoP PRODUCTION |  |  | 4,17E-05 | 1,74E-02 | 2,62E-02 | 1,19E-02 | 2,69E-02 | 2,18E-02 | 7,80E-04 | 2,91E-04 | 3,26E+01 | 5,14E-02 | 9,60E-02 | 4,17E-01 | 1,51E+00 | 9,60E+00 | 5,60E-03 | 7,33E-02 | 5,73E-03 |
| use phase | Electricity | 0,875 | kWh | 1,66E-07 | 1,72E-01 | 5,69E-04 | 4,89E-04 | 5,89E-04 | 1,22E-03 | 3,07E-04 | 2,59E-05 | 2,89E+00 | 4,38E-02 | 5,83E-02 | 4,81E-02 | 6,81E-01 | 1,57E-02 | 1,39E-03 | 3,77E-02 | 7,02E-03 |
|  | wood coating | 0,320 | kg | 9,92E-07 | 6,92E-02 | 2,48E-03 | 1,47E-03 | 2,79E-03 | 3,47E-03 | 3,67E-04 | 3,82E-04 | 9,86E+00 | 9,48E-02 | 9,64E-02 | 1,93E-01 | 1,89E+00 | 2,97E-01 | 4,20E-03 | 6,33E-02 | 1,13E-02 |
|  | timber glue | 0,220 | kg | 3,17E-06 | 7,77E-02 | 2,99E-03 | 2,08E-03 | 3,22E-03 | 4,76E-03 | 7,05E-04 | 7,03E-05 | 3,47E+01 | 8,82E-02 | 1,34E-01 | 2,72E-01 | 1,82E+00 | 3,86E-02 | 6,11E-03 | 1,23E-01 | 1,27E-02 |
| end of life | planks waste, WtE | 7,000 | kg | -2,24E-07 | -6,94E-01 | -3,19E-03 | -3,66E-03 | -3,42E-03 | -1,03E-02 | -1,81E-03 | -1,51E-04 | -1,93E+01 | -8,79E-02 | -1,47E-01 | -2,71E-01 | -2,66E+00 | -1,01E-01 | -2,71E-03 | -4,04E-01 | -3,88E-02 |
|  | coating waste, WtE | 0,320 | kg | -3,65E-08 | -1,87E-02 | -4,67E-05 | -9,37E-05 | -5,25E-05 | -2,64E-04 | -3,79E-05 | -4,65E-06 | -2,97E-01 | 7,28E-02 | 1,02E-01 | 2,94E-01 | 3,51E+00 | -2,69E-03 | -4,11E-05 | -1,10E-02 | -9,45E-04 |
|  | glue waste, WtE | 0,220 | kg | 2,11E-06 | -4,92E-02 | 3,23E-04 | -1,89E-04 | 3,10E-04 | -5,02E-04 | -1,32E-04 | 3,20E-05 | -9,04E-01 | 1,90E-02 | 2,32E-02 | -8,82E-03 | 1,82E-01 | -6,89E-03 | -9,86E-05 | -2,76E-02 | -1,98E-03 |
| TOTAL IMPACT |  |  |  | 4,79E-05 | -4,26E-01 | 2,93E-02 | 1,20E-02 | 3,03E-02 | 2,01E-02 | 1,76E-04 | 6,46E-04 | 5,95E+01 | 2,82E-01 | 3,63E-01 | 9,44E-01 | 6,93E+00 | 9,84E+00 | 1,44E-02 | -1,46E-01 | -5,01E-03 |
| SD |  |  |  | 34% | >50% | 27% | 22% | 26% | 33% | >50% | 37% | >50% | >50% | 40% | >50% | >50% | >50% | 28% | >50% | >50% |

Table S20: Impact of resin flooring. LCIA Method: ReCiPe 2016 Midpoint (H) V1.08 / World (2010) H. Ozone depletion (OD), Ionizing radiation (IR), Ozone formation human health (OFH), Particulate matter formation (PMF), Ozone formation terrestrial ecosystems (OFT), Terrestrial acidification (TA), Freshwater eutrophication (FE), Marine eutrophication (ME), Terrestrial ecotoxicity (TET), Freshwater ecotoxicity (FET), Marine ecotoxicity (MET), Human toxicity (cancer effects, HTc), Human toxicity (non-cancer effects, HTnc), Land occupation (LO), Mineral resource scarcity (MRS), Fossil resource scarcity (FRS), Water consumption (WC), SD: Standard deviation.

| RESIN LIFE CYCLE BOUNDARIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
| production | Epoxy resin | 2,613 | kg | 3,70E-06 | 1,00E+00 | 3,19E-02 | 1,84E-02 | 3,50E-02 | 4,07E-02 | 6,88E-03 | 4,12E-04 | 4,13E+02 | 8,24E-01 | 1,29E+00 | 2,95E+00 | 1,62E+01 | 3,67E-01 | 5,90E-02 | 1,17E+00 | 1,05E-01 |
|  | Polyurethane resin | 1,120 | kg | 5,60E-06 | 6,70E-01 | 9,70E-03 | 7,56E-03 | 1,09E-02 | 2,32E-02 | 2,05E-03 | 4,02E-04 | 5,92E+01 | 3,40E-01 | 4,85E-01 | 1,13E+00 | 6,49E+00 | 1,47E-01 | 2,49E-02 | 2,41E-01 | 7,46E-02 |
| end of life | resin, WtE | 3,733 | kg | 3,58E-05 | -8,34E-01 | 5,48E-03 | -3,21E-03 | 5,25E-03 | -8,52E-03 | -2,24E-03 | 5,43E-04 | ######### | 3,22E-01 | 3,94E-01 | -1,50E-01 | 3,09E+00 | -1,17E-01 | -1,67E-03 | -4,69E-01 | -3,35E-02 |
|  | resin, landfilling | 3,733 | kg | 6,70E-09 | 9,15E-05 | 2,07E-04 | 4,49E-05 | 2,12E-04 | 8,71E-05 | 2,33E-02 | 6,73E-02 | 1,63E-01 | 7,24E-01 | 9,79E-01 | 1,28E-02 | 1,92E+01 | 6,48E-03 | 2,96E-05 | 1,48E-04 | 1,75E-05 |
| TOTAL IMPACT, WTE |  |  |  | 4,51E-05 | 8,39E-01 | 4,71E-02 | 2,27E-02 | 5,12E-02 | 5,54E-02 | 6,70E-03 | 1,36E-03 | 4,56E+02 | 1,49E+00 | 2,17E+00 | 3,93E+00 | 2,58E+01 | 3,97E-01 | 8,23E-02 | 9,42E-01 | 1,46E-01 |
| TOTAL IMPACT, LANDFILLING |  |  |  | 9,30E-06 | 1,67E+00 | 4,18E-02 | 2,60E-02 | 4,62E-02 | 6,40E-02 | 3,22E-02 | 6,82E-02 | 4,72E+02 | 1,89E+00 | 2,75E+00 | 4,10E+00 | 4,19E+01 | 5,21E-01 | 8,40E-02 | 1,41E+00 | 1,80E-01 |
| SD |  |  |  | >50% | >50% | 35% | 24% | 34% | 26% | 32% | 23% | >50% | 35% | 34% | 48% | >50% | 36% | 41% | 41% | >50% |

Table S21: Impact of 100% epoxy resin floor. LCIA Method: ReCiPe 2016 Midpoint (H) V1.08 / World (2010) H. Ozone depletion (OD), Ionizing radiation (IR), Ozone formation human health (OFH), Particulate matter formation (PMF), Ozone formation terrestrial ecosystems (OFT), Terrestrial acidification (TA), Freshwater eutrophication (FE), Marine eutrophication (ME), Terrestrial ecotoxicity (TET), Freshwater ecotoxicity (FET), Marine ecotoxicity (MET), Human toxicity (cancer effects, HTc), Human toxicity (non-cancer effects, HTnc), Land occupation (LO), Mineral resource scarcity (MRS), Fossil resource scarcity (FRS), Water consumption (WC). LCIA Method: IPCC2021 GWP100 (incl. CO2 uptake version 1.01) for Global Warming (GW).

| 100% EPOXY RESIN LIFE CYCLE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase | Parameter | Amount | Unit | GW (kg CO2eq) | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
| production | Epoxy resin | 3,733 | kg | 2.21E+01 | 4.87E-06 | 1.43E+00 | 4.56E-02 | 2.62E-02 | 5.01E-02 | 5.82E-02 | 8.54E-03 | 5.89E-04 | 5.89E+02 | 1.18E+00 | 1.84E+00 | 4.22E+00 | 2.32E+01 | 6.03E-01 | 8.43E-02 | 8.97E+00 | 1.50E-01 |
| end of life | resin, WtE | 3,733 | kg | 5.81E+00 | 3,58E-05 | -8,34E-01 | 5,48E-03 | -3,21E-03 | 5,25E-03 | -8,52E-03 | -2,24E-03 | 5,43E-04 | ######### | 3,22E-01 | 3,94E-01 | -1,50E-01 | 3,09E+00 | -1,17E-01 | -1,67E-03 | -4,69E-01 | -3,35E-02 |
|  | resin, landfilling | 3,733 | kg | 5.71E-01 | 6,70E-09 | 9,15E-05 | 2,07E-04 | 4,49E-05 | 2,12E-04 | 8,71E-05 | 2,33E-02 | 6,73E-02 | 1,63E-01 | 7,24E-01 | 9,79E-01 | 1,28E-02 | 1,92E+01 | 6,48E-03 | 2,96E-05 | 1,48E-04 | 1,75E-05 |
| TOTAL IMPACT, WTE |  |  |  | 2.79E+01 | 4.51E-05 | 8.39E-01 | 4.71E-02 | 2.27E-02 | 5.12E-02 | 5.54E-02 | 6.70E-03 | 1.36E-03 | 4.56E+02 | 1.49E+00 | 2.17E+00 | 3.93E+00 | 2.58E+01 | 3.97E-01 | 8.23E-02 | 9.42E-01 | 1.46E-01 |
| TOTAL IMPACT, LANDFILLING |  |  |  | 2.27E+01 | 9.30E-06 | 1.67E+00 | 4.18E-02 | 2.60E-02 | 4.62E-02 | 6.40E-02 | 3.22E-02 | 6.82E-02 | 4.72E+02 | 1.89E+00 | 2.75E+00 | 4.10E+00 | 4.19E+01 | 5.21E-01 | 8.40E-02 | 1.41E+00 | 1.80E-01 |

Image S4: Network visualization of HTnc impact category resin flooring, WtE.

Table S22: Impacts divided by geographic region. LCIA Method: ReCiPe 2016 Midpoint (H) V1.08 / World (2010) H. Ozone depletion (OD), Ionizing radiation (IR), Ozone formation human health (OFH), Particulate matter formation (PMF), Ozone formation terrestrial ecosystems (OFT), Terrestrial acidification (TA), Freshwater eutrophication (FE), Marine eutrophication (ME), Terrestrial ecotoxicity (TET), Freshwater ecotoxicity (FET), Marine ecotoxicity (MET), Human toxicity (cancer effects, HTc), Human toxicity (non-cancer effects, HTnc), Land occupation (LO), Mineral resource scarcity (MRS), Fossil resource scarcity (FRS), Water consumption (WC).

| NON-TREATED WOOD LIFE CYCLE, LANDFILLING NW1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Region | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
| Indonesia | 4,13E-05 | 1,34E-02 | 9,74E-03 | 6,78E-03 | 1,02E-02 | 5,85E-03 | 7,43E-04 | 2,87E-04 | 2,37E+01 | 4,26E-02 | 7,59E-02 | 2,81E-01 | 1,39E+00 | 9,60E+00 | 3,93E-03 | 6,60E-02 | 4,96E-03 |
| Ocean | 4,15E-07 | 4,00E-03 | 1,64E-02 | 5,12E-03 | 1,66E-02 | 1,59E-02 | 3,72E-05 | 4,82E-06 | 8,83E+00 | 8,83E-03 | 2,01E-02 | 1,36E-01 | 1,23E-01 | 6,22E-03 | 1,67E-03 | 7,32E-03 | 7,75E-04 |
| Europe | 3,38E-06 | 2,51E-01 | 4,13E-03 | 2,73E-03 | 4,42E-03 | 6,30E-03 | 2,96E-02 | 6,08E-03 | 3,87E+01 | 2,08E-01 | 2,97E-01 | 3,42E-01 | 4,93E+00 | 6,22E-02 | 7,76E-03 | 1,64E-01 | -6,21E-03 |
| Total | 4,51E-05 | 2,69E-01 | 3,03E-02 | 1,46E-02 | 3,13E-02 | 2,81E-02 | 3,04E-02 | 6,37E-03 | 7,13E+01 | 2,59E-01 | 3,93E-01 | 7,59E-01 | 6,44E+00 | 9,66E+00 | 1,34E-02 | 2,37E-01 | -4,75E-04 |
| % Indonesia | 91,58% | 4,99% | 32,16% | 46,32% | 32,71% | 20,85% | 2,44% | 4,50% | 33,29% | 16,44% | 19,30% | 37,08% | 21,61% | 99,29% | 29,44% | 27,84% | -1043,38% |
| % Ocean | 0,92% | 1,49% | 54,19% | 35,01% | 53,17% | 56,71% | 0,12% | 0,08% | 12,38% | 3,41% | 5,13% | 17,89% | 1,90% | 0,06% | 12,48% | 3,09% | -163,13% |
| % Europe | 7,50% | 93,52% | 13,65% | 18,68% | 14,12% | 22,44% | 97,44% | 95,43% | 54,33% | 80,16% | 75,57% | 45,03% | 76,49% | 0,64% | 58,09% | 69,07% | 1306,51% |
| NON-TREATED WOOD LIFE CYCLE, WTE NW2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Region | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
| Indonesia | 4,13E-05 | 1,34E-02 | 9,74E-03 | 6,78E-03 | 1,02E-02 | 5,85E-03 | 7,43E-04 | 2,87E-04 | 2,37E+01 | 4,26E-02 | 7,59E-02 | 2,81E-01 | 1,39E+00 | 9,60E+00 | 3,93E-03 | 6,60E-02 | 4,96E-03 |
| Ocean | 4,15E-07 | 4,00E-03 | 1,64E-02 | 5,12E-03 | 1,66E-02 | 1,59E-02 | 3,72E-05 | 4,82E-06 | 8,83E+00 | 8,83E-03 | 2,01E-02 | 1,36E-01 | 1,23E-01 | 6,22E-03 | 1,67E-03 | 7,32E-03 | 7,75E-04 |
| Europe | 5,22E-06 | -4,94E-01 | 6,85E-04 | -1,27E-03 | 7,00E-04 | -4,82E-03 | -9,33E-04 | -2,30E-05 | 1,74E+01 | 6,31E-02 | 6,91E-02 | 3,99E-02 | 1,48E-02 | -5,36E-02 | 4,69E-03 | -2,71E-01 | -2,11E-02 |
| Total | 4,69E-05 | -4,76E-01 | 2,68E-02 | 1,06E-02 | 2,76E-02 | 1,69E-02 | -1,53E-04 | 2,68E-04 | 4,99E+01 | 1,14E-01 | 1,65E-01 | 4,57E-01 | 1,53E+00 | 9,55E+00 | 1,03E-02 | -1,98E-01 | -1,54E-02 |
| % Indonesia | 87,98% | -2,82% | 36,29% | 63,76% | 37,12% | 34,53% | -484,97% | 106,79% | 47,50% | 37,22% | 45,95% | 61,56% | 91,02% | 100,50% | 38,21% | -33,36% | -32,16% |
| % Ocean | 0,88% | -0,84% | 61,16% | 48,20% | 60,34% | 93,92% | -24,32% | 1,80% | 17,67% | 7,71% | 12,21% | 29,71% | 8,01% | 0,07% | 16,20% | -3,70% | -5,03% |
| % Europe | 11,13% | 103,65% | 2,55% | -11,96% | 2,54% | -28,45% | 609,29% | -8,59% | 34,83% | 55,07% | 41,84% | 8,74% | 0,97% | -0,56% | 45,59% | 137,06% | 137,19% |
| TREATED WOOD LIFE CYCLE, LANDFILLING TW1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Region | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
| Indonesia | 4,13E-05 | 1,34E-02 | 9,74E-03 | 6,78E-03 | 1,02E-02 | 5,85E-03 | 7,43E-04 | 2,87E-04 | 2,37E+01 | 4,26E-02 | 7,59E-02 | 2,81E-01 | 1,39E+00 | 9,60E+00 | 3,93E-03 | 6,60E-02 | 4,96E-03 |
| Ocean | 4,15E-07 | 4,00E-03 | 1,64E-02 | 5,12E-03 | 1,66E-02 | 1,59E-02 | 3,72E-05 | 4,82E-06 | 8,83E+00 | 8,83E-03 | 2,01E-02 | 1,36E-01 | 1,23E-01 | 6,22E-03 | 1,67E-03 | 7,32E-03 | 7,75E-04 |
| Europe | 4,37E-06 | 3,21E-01 | 6,64E-03 | 4,21E-03 | 7,24E-03 | 9,78E-03 | 3,20E-02 | 6,46E-03 | 4,86E+01 | 5,30E-01 | 7,13E-01 | 5,48E-01 | 1,73E+01 | 3,59E-01 | 1,20E-02 | 2,27E-01 | 4,40E-03 |
| Total | 4,61E-05 | 3,38E-01 | 3,28E-02 | 1,61E-02 | 3,41E-02 | 3,15E-02 | 3,28E-02 | 6,75E-03 | 8,12E+01 | 5,81E-01 | 8,09E-01 | 9,65E-01 | 1,88E+01 | 9,96E+00 | 1,76E-02 | 3,00E-01 | 1,01E-02 |
| % Indonesia | 89,61% | 3,97% | 29,70% | 42,07% | 30,00% | 18,55% | 2,26% | 4,24% | 29,22% | 7,33% | 9,38% | 29,14% | 7,41% | 96,33% | 22,39% | 21,97% | 48,92% |
| % Ocean | 0,90% | 1,18% | 50,06% | 31,80% | 48,77% | 50,45% | 0,11% | 0,07% | 10,87% | 1,52% | 2,49% | 14,06% | 0,65% | 0,06% | 9,49% | 2,44% | 7,65% |
| % Europe | 9,49% | 94,85% | 20,24% | 26,13% | 21,23% | 31,00% | 97,62% | 95,69% | 59,90% | 91,15% | 88,13% | 56,80% | 91,94% | 3,61% | 68,12% | 75,60% | 43,44% |
| TREATED WOOD LIFE CYCLE, WTE TW2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Region | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | OFH (kg NOx eq) | PMF (kg PM2.5 eq) | OFT (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRS (kg Cu eq) | FRS (kg oil eq) | WCP (m3) |
| Indonesia | 4,13E-05 | 1,34E-02 | 9,74E-03 | 6,78E-03 | 1,02E-02 | 5,85E-03 | 7,43E-04 | 2,87E-04 | 2,37E+01 | 4,26E-02 | 7,59E-02 | 2,81E-01 | 1,39E+00 | 9,60E+00 | 3,93E-03 | 6,60E-02 | 4,96E-03 |
| Ocean | 4,15E-07 | 4,00E-03 | 1,64E-02 | 5,12E-03 | 1,66E-02 | 1,59E-02 | 3,72E-05 | 4,82E-06 | 8,83E+00 | 8,83E-03 | 2,01E-02 | 1,36E-01 | 1,23E-01 | 6,22E-03 | 1,67E-03 | 7,32E-03 | 7,75E-04 |
| Europe | 6,18E-06 | -4,43E-01 | 3,12E-03 | 1,06E-04 | 3,44E-03 | -1,62E-03 | -6,04E-04 | 3,54E-04 | 2,70E+01 | 2,31E-01 | 2,67E-01 | 5,27E-01 | 5,42E+00 | 2,40E-01 | 8,85E-03 | -2,19E-01 | -1,07E-02 |
| Total | 4,79E-05 | -4,26E-01 | 2,93E-02 | 1,20E-02 | 3,03E-02 | 2,01E-02 | 1,76E-04 | 6,46E-04 | 5,95E+01 | 2,82E-01 | 3,63E-01 | 9,44E-01 | 6,93E+00 | 9,84E+00 | 1,44E-02 | -1,46E-01 | -5,01E-03 |
| % Indonesia | 86,23% | -3,15% | 33,28% | 56,45% | 33,76% | 29,04% | 422,88% | 44,39% | 39,87% | 15,10% | 20,89% | 29,80% | 20,08% | 97,50% | 27,22% | -45,34% | -98,86% |
| % Ocean | 0,87% | -0,94% | 56,08% | 42,67% | 54,88% | 78,99% | 21,20% | 0,75% | 14,83% | 3,13% | 5,55% | 14,38% | 1,77% | 0,06% | 11,54% | -5,03% | -15,46% |
| % Europe | 12,90% | 104,09% | 10,64% | 0,88% | 11,36% | -8,03% | -344,08% | 54,87% | 45,30% | 81,77% | 73,56% | 55,82% | 78,15% | 2,44% | 61,24% | 150,37% | 214,32% |

Table S23: Global warming impact variation considering the corresponding CO2 uptake of the biogenic CH4 emissions within the system following the 0/0 approach. The calculation was only performed on the biogenic methane emitting step of the system such as wood chips incineration for the drying step, wood landfilling and incineration demonstrating that this aspect is negligible.

| Process | Global Warming contribution (kg CO2eq) | CH4 biogenic emission (kg) | Corresponding CO2 uptake (kg) | GW contr. Variation (%) |
| --- | --- | --- | --- | --- |
| Wood chips incineration | 9.25E-02 | 1.25E-04 | 3.43E-04 | 0.37% |
| Waste wood landfilling | 5.12E-01 | 1.44E-02 | 3.94E-02 | 7.69% |
| Waste wood incineration | -3.51E+00 | 3.55E-06 | 9.75E-06 | 0.0003% |

# Life Cycle Inventory

Table LCI1: LCI of Wood flooring, untreated, WtE

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| Wood flooring, untreated, WtE | 1.00 | m2 |  | To produce 1 m² of wooden flooring, 7 kg of wood is required. This calculation is based on a plank thickness of 1 cm and a wood density of 700 kg/m³ |
| Waste to treatment |  |  |  |  |
| Waste wood, untreated {GLO} municipal WtE + electrical energy and heat recovery | 7.00 | kg | ecoinvent process modified, inventory presented in page 'waste' |  |
| Waste polyurethane {GLO} municipal WtE + electrical energy and heat recovery | 0.22 | kg | ecoinvent process modified, inventory presented in page 'waste' |  |
| input |  |  |  |  |
| Materials/fuels |  |  |  |  |
| cleft timber, market for, measured as dry mass RoW | 21.07 | kg | ecoinvent process modified | market for cleft timber, measured as dry mass RoW was used, but transportation was excluded since precise transportation data were available as primary data. |
| Wood chips, from forest, hardwood, burned in furnace 50kW/ID U | 10.11 | MJ | ecoinvent process modified | Wood chips, from forest, hardwood, burned in furnace 50kW/CH U was used, excluding transportation and harvesting as these were available as primary data. Additionally, the electricity mix for Indonesia was applied instead of the Swiss one. |
| Transport, freight, lorry 16-32 metric ton, EURO3 {RoW}¦ market for transport | 2.94 | tkm | from ecoinvent 3.9.1 database |  |
| Transport, freight, sea, container ship {GLO} | 84.00 | tkm | from ecoinvent 3.9.1 database |  |
| Polyurethane adhesive {GLO} market for | 0.22 | kg | from ecoinvent 3.9.1 database |  |
| Electricity/heat |  |  |  |  |
| Electricity, medium voltage {ID}¦ market for electricity | 0.12 | kWh | from ecoinvent 3.9.1 database |  |
| Electricity, low voltage {RER}¦ market group for electricity | 0.88 | kWh | from ecoinvent 3.9.1 database |  |
| Emissions to air |  |  |  |  |
| Carbon dioxide, biogenic | 21.29 | kg | from ecoinvent 3.9.1 database |  |

Table LCI2: LCI of Wood flooring, untreated, landfilling

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| Wood flooring, untreated, landfilling | 1.00 | m2 |  | To produce 1 m² of wooden flooring, 7 kg of wood is required. This calculation is based on a plank thickness of 1 cm and a wood density of 700 kg/m³ |
| Waste to treatment |  |  |  |  |
| Waste wood, untreated {RoW} sanitary landfill | 7.00 | kg | from ecoinvent 3.9.1 database | Landfill gas capture and utilisation (34.98%) generates 0.004568 kWh/kg of electricity and 0.008238 MJ/kg of usefull heat. |
| Waste polyurethane {GLO} unsanitary landfill, moist infiltration class (300mm) | 0.22 | kg | from ecoinvent 3.9.1 database | Percentage of landfill gas directly emitted is 100w%. |
| input |  |  |  |  |
| Materials/fuels |  |  |  |  |
| cleft timber, market for, measured as dry mass RoW | 21.07 | kg | ecoinvent process modified | market for cleft timber, measured as dry mass RoW was used, but transportation was excluded since precise transportation data were available as primary data. |
| Wood chips, from forest, hardwood, burned in furnace 50kW/ID U | 10.11 | MJ | ecoinvent process modified | Wood chips, from forest, hardwood, burned in furnace 50kW/CH U was used, excluding transportation and harvesting as these were available as primary data. Additionally, the electricity mix for Indonesia was applied instead of the Swiss one. |
| Transport, freight, lorry 16-32 metric ton, EURO3 {RoW}¦ market for transport | 2.94 | tkm | from ecoinvent 3.9.1 database |  |
| Transport, freight, sea, container ship {GLO} | 84.00 | tkm | from ecoinvent 3.9.1 database |  |
| Average Wood Coating Mix (Alkyd paint, Acrylic varnish,Wood coating) [RER] | 0.32 | kg | inventory presented in page 'coating' |  |
| Polyurethane adhesive {GLO} market for | 0.22 | kg | from ecoinvent 3.9.1 database |  |
| Electricity/heat |  |  |  |  |
| Electricity, medium voltage {ID}¦ market for electricity | 0.12 | kWh | from ecoinvent 3.9.1 database |  |
| Electricity, low voltage {RER}¦ market group for electricity | 0.88 | kWh | from ecoinvent 3.9.1 database |  |
| Emissions to air |  |  |  |  |
| Carbon dioxide, biogenic | 21.29 | kg | from ecoinvent 3.9.1 database |  |

Table LCI3: LCI of Wood flooring, treated, WtE

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| Wood flooring, treated, WtE | 1.00 | m2 |  | To produce 1 m² of wooden flooring, 7 kg of wood is required. This calculation is based on a plank thickness of 1 cm and a wood density of 700 kg/m³ |
| Waste to treatment |  |  |  |  |
| Waste wood, untreated {GLO} municipal WtE + electrical energy and heat recovery | 7.00 | kg | ecoinvent process modified, inventory presented in page 'waste' |  |
| Waste polyurethane {GLO} municipal WtE + electrical energy and heat recovery | 0.22 | kg | ecoinvent process modified, inventory presented in page 'waste' |  |
| Waste paint {Europe without Switzerland} municipal WtE + electrical energy and heat recovery | 0.32 | kg | ecoinvent process modified, inventory presented in page 'waste' |  |
| input |  |  |  |  |
| Materials/fuels |  |  |  |  |
| cleft timber, market for, measured as dry mass RoW | 21.07 | kg | ecoinvent process modified | market for cleft timber, measured as dry mass RoW was used, but transportation was excluded since precise transportation data were available as primary data. |
| Wood chips, from forest, hardwood, burned in furnace 50kW/ID U | 10.11 | MJ | ecoinvent process modified | Wood chips, from forest, hardwood, burned in furnace 50kW/CH U was used, excluding transportation and harvesting as these were available as primary data. Additionally, the electricity mix for Indonesia was applied instead of the Swiss one. |
| Transport, freight, lorry 16-32 metric ton, EURO3 {RoW}¦ market for transport | 2.94 | tkm | from ecoinvent 3.9.1 database |  |
| Transport, freight, sea, container ship {GLO} | 84.00 | tkm | from ecoinvent 3.9.1 database |  |
| Average Wood Coating Mix (Alkyd paint, Acrylic varnish, Wood coating) [RER] | 0.32 | kg | inventory presented in page 'coating' |  |
| Polyurethane adhesive {GLO} market for | 0.22 | kg | from ecoinvent 3.9.1 database |  |
| Electricity/heat |  |  |  |  |
| Electricity, medium voltage {ID}¦ market for electricity | 0.12 | kWh | from ecoinvent 3.9.1 database |  |
| Electricity, low voltage {RER}¦ market group for electricity | 0.88 | kWh | from ecoinvent 3.9.1 database |  |
| Emissions to air |  |  |  |  |
| Carbon dioxide, biogenic | 21.29 | kg | from ecoinvent 3.9.1 database |  |

Table LCI4: LCI of Wood flooring, treated, landfilling

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| Wood flooring, treated, landfilling | 1.00 | m2 |  | To produce 1 m² of wooden flooring, 7 kg of wood is required. This calculation is based on a plank thickness of 1 cm and a wood density of 700 kg/m³ |
| Waste to treatment |  |  |  |  |
| Waste wood, untreated {RoW} sanitary landfill | 7.00 | kg | from ecoinvent 3.9.1 database | Landfill gas capture and utilisation (34.98%) generates 0.004568 kWh/kg of electricity and 0.008238 MJ/kg of usefull heat. |
| Waste paint {Europe without Switzerland} sanitary landfill | 0.32 | kg | from ecoinvent 3.9.1 database | Landfill gas capture and utilisation (34.98%) generates 0.004939 kWh/kg of electricity and 0.008908 MJ/kg of usefull heat. |
| Waste polyurethane {GLO} unsanitary landfill, moist infiltration class (300mm) | 0.22 | kg | from ecoinvent 3.9.1 database | Percentage of landfill gas directly emitted is 100w%. |
| input |  |  |  |  |
| Materials/fuels |  |  |  |  |
| cleft timber, market for, measured as dry mass RoW | 21.07 | kg | ecoinvent process modified | market for cleft timber, measured as dry mass RoW was used, but transportation was excluded since precise transportation data were available as primary data. |
| Wood chips, from forest, hardwood, burned in furnace 50kW/ID U | 10.11 | MJ | ecoinvent process modified | Wood chips, from forest, hardwood, burned in furnace 50kW/CH U was used, excluding transportation and harvesting as these were available as primary data. Additionally, the electricity mix for Indonesia was applied instead of the Swiss one. |
| Transport, freight, lorry 16-32 metric ton, EURO3 {RoW}¦ market for transport | 2.94 | tkm | from ecoinvent 3.9.1 database |  |
| Transport, freight, sea, container ship {GLO} | 84.00 | tkm | from ecoinvent 3.9.1 database |  |
| Average Wood Coating Mix (Alkyd paint, Acrylic varnish, Wood coating) [RER] | 0.32 | kg | inventory presented in page 'coating' |  |
| Polyurethane adhesive {GLO} market for | 0.22 | kg | from ecoinvent 3.9.1 database |  |
| Electricity/heat |  |  |  |  |
| Electricity, medium voltage {ID}¦ market for electricity | 0.12 | kWh | from ecoinvent 3.9.1 database |  |
| Electricity, low voltage {RER}¦ market group for electricity | 0.88 | kWh | from ecoinvent 3.9.1 database |  |
| Emissions to air |  |  |  |  |
| Carbon dioxide, biogenic | 21.29 | kg | from ecoinvent 3.9.1 database |  |

Table LCI5: LCI of resin flooring, WtE

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| resin flooring, WtE | 1.00 | m2 |  | To produce 1 m² of resin flooring, 3.733 kg of resin is required. This is calculated based on a resin consumption of 1.6 kg/m² for a 1.5 mm thickness, adjusted proportionally to the yacht deck's average thickness of 3.5 mm. |
| Waste to treatment |  |  |  |  |
| Waste polyurethane {GLO} municipal WtE + electrical energy and heat recovery | 3.73 | kg | ecoinvent process modified to include energy and heat recovery following documentation available on simapro |  |
| input |  |  |  |  |
| Materials/fuels |  |  |  |  |
| Epoxy resin, liquid {RER}¦ market for | 2.61 | kg | from ecoinvent 3.9.1 database |  |
| Methylene diphenyl diisocyanate {RER}¦ market | 1.12 | kg | from ecoinvent 3.9.1 database | Methylene diphenyl diisocyanate was chosen as a proxy for polyurethane resin as it is its key component. |

Table LCI6: LCI of resin flooring, landfilling

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| resin flooring, WtE | 1.00 | m2 |  | To produce 1 m² of resin flooring, 3.733 kg of resin is required. This is calculated based on a resin consumption of 1.6 kg/m² for a 1.5 mm thickness, adjusted proportionally to the yacht deck's average thickness of 3.5 mm. |
| Waste to treatment |  |  |  |  |
| Waste polyurethane {GLO} unsanitary landfill, moist infiltration class (300mm) | 3.733 | kg | ecoinvent process modified to include energy and heat recovery following documentation available on simapro |  |
| input |  |  |  |  |
| Materials/fuels |  |  |  |  |
| Epoxy resin, liquid {RER}¦ market for | 2.61 | kg | from ecoinvent 3.9.1 database |  |
| Methylene diphenyl diisocyanate {RER}¦ market | 1.12 | kg | from ecoinvent 3.9.1 database | Methylene diphenyl diisocyanate was chosen as a proxy for polyurethane resin as it is its key component. |

Table LCI7: LCI of Waste wood, untreated {GLO}, municipal WtE + electrical energy and heat recovery

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| Waste wood, untreated {GLO}, municipal WtE + electrical energy and heat recovery | 1.00 | kg |  | Net energy production: 1.74MJ/kg electric energy and 3.5MJ/kg thermal energy. |
| Avoided products |  |  |  |  |
| Electricity, medium voltage {RER}¦ market group for electricity, medium voltage ¦ APOS, U | 1.74 | MJ | from ecoinvent 3.9.1 database |  |
| Heat, central or small-scale, other than natural gas {Europe without Switzerland} ¦ market for heat, central or small-scale, other than natural gas ¦ APOS, U | 2.23 | MJ | from ecoinvent 3.9.1 database | According to Eurostat data, 63.73% of gross derived heat generation in Europe is sourced from fuels other than natural gas. For further details, refer to the Eurostat statistics https://ec.europa.eu/eurostat/statistics-explained/index.php?title=File:Gross_derived_heat_generation_by_fuel,_EU,_2000-2021_(GWh)_07-07-2023.png |
| Heat, district or industrial, natural gas {Europe without Switzerland} ¦ market for heat, district or industrial, natural gas ¦ APOS, U | 1.27 | MJ | from ecoinvent 3.9.1 database | According to Eurostat, 36.27% of gross derived heat generation in Europe is sourced from natural gas. https://ec.europa.eu/eurostat/statistics-explained/index.php?title=File:Gross_derived_heat_generation_by_fuel,_EU,_2000-2021_(GWh)_07-07-2023.png |
| input |  |  |  |  |
| Waste wood, untreated {GLO}, municipal WtE | 1.00 | kg | from ecoinvent 3.9.1 database |  |

Table LCI8: LCI of Waste paint {Europe without Switzerland} municipal WtE + electrical energy and heat recovery

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| Waste paint {Europe without Switzerland} municipal WtE + electrical energy and heat recovery | 1.00 | kg |  | Net energy production: 1.03MJ/kg electric energy and 2.12MJ/kg thermal energy. |
| Avoided products |  |  |  |  |
| Electricity, medium voltage {RER}¦ market group for electricity, medium voltage ¦ APOS, U | 1.03 | MJ | from ecoinvent 3.9.1 database |  |
| Heat, central or small-scale, other than natural gas {Europe without Switzerland} ¦ market for heat, central or small-scale, other than natural gas ¦ APOS, U | 1.35 | MJ | from ecoinvent 3.9.1 database | According to Eurostat data, 63.73% of gross derived heat generation in Europe is sourced from fuels other than natural gas. For further details, refer to the Eurostat statistics https://ec.europa.eu/eurostat/statistics-explained/index.php?title=File:Gross_derived_heat_generation_by_fuel,_EU,_2000-2021_(GWh)_07-07-2023.png |
| Heat, district or industrial, natural gas {Europe without Switzerland} ¦ market for heat, district or industrial, natural gas ¦ APOS, U | 0.77 | MJ | from ecoinvent 3.9.1 database | According to Eurostat, 36.27% of gross derived heat generation in Europe is sourced from natural gas. https://ec.europa.eu/eurostat/statistics-explained/index.php?title=File:Gross_derived_heat_generation_by_fuel,_EU,_2000-2021_(GWh)_07-07-2023.png |
| input |  |  |  |  |
| Waste paint {Europe without Switzerland} municipal WtE | 1.00 | kg | from ecoinvent 3.9.1 database |  |

Table LCI9: LCI of Waste polyurethane {GLO} municipal WtE + electrical energy and heat recovery

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| output |  |  |  |  |
| Product |  |  |  |  |
| Waste polyurethane {GLO} municipal WtE + electrical energy and heat recovery | 1.00 | kg |  | Net energy production: 3.94MJ/kg electric energy and 7.7MJ/kg thermal energy. |
| Avoided products |  |  |  |  |
| Electricity, medium voltage {RER}¦ market group for electricity, medium voltage ¦ APOS, U | 3.94 | MJ | from ecoinvent 3.9.1 database |  |
| Heat, central or small-scale, other than natural gas {Europe without Switzerland} ¦ market for heat, central or small-scale, other than natural gas ¦ APOS, U | 4.91 | MJ | from ecoinvent 3.9.1 database | According to Eurostat data, 63.73% of gross derived heat generation in Europe is sourced from fuels other than natural gas. For further details, refer to the Eurostat statistics https://ec.europa.eu/eurostat/statistics-explained/index.php?title=File:Gross_derived_heat_generation_by_fuel,_EU,_2000-2021_(GWh)_07-07-2023.png |
| Heat, district or industrial, natural gas {Europe without Switzerland} ¦ market for heat, district or industrial, natural gas ¦ APOS, U | 2.79 | MJ | from ecoinvent 3.9.1 database | According to Eurostat, 36.27% of gross derived heat generation in Europe is sourced from natural gas. https://ec.europa.eu/eurostat/statistics-explained/index.php?title=File:Gross_derived_heat_generation_by_fuel,_EU,_2000-2021_(GWh)_07-07-2023.png |
| input |  |  |  |  |
| Waste polyurethane {GLO} municipal WtE | 1.00 | kg | from ecoinvent 3.9.1 database |  |

Table LCI10: LCI of Average Wood Coating Mix (Alkyd paint, Acrylic varnish, Wood coating) [RER]

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| Product |  |  |  |  |
| Average Wood Coating Mix (Alkyd paint, Acrylic varnish,Wood coating) [RER] | 3.00 | kg |  | wet mass= 7.88 kg dry mass= 3 kg |
| Materials/fuels |  |  |  |  |
| Alkyd paint, colorless, without solvent, in 60% solution state {RER} market | 1.00 | kg | ecoinvent process modified |  |
| Acrylic varnish, colorless, with water, in 53% solution state {RER} market | 1.89 | kg | ecoinvent process modified |  |
| Wood coating, yatch deck (teak) {RER} market | 5.00 | kg |  |  |

Table LCI11: LCI of Acrylic varnish, colorless, with water, in 53% solution state {RER} production

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| Acrylic varnish, colorless, with water, in 53% solution state {RER} production |  |  | ecoinvent process modified | The Ecoinvent process acrylic varnish production, with water, in 53% solution state RER was modified to enhance its colorless nature by removing the contribution of titanium dioxide (0.20366 kg per kg of product). |

Table LCI12: LCI of Alkyd paint, colorless, without solvent, in 60% solution state {RER} production

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| Alkyd paint, colorless, without solvent, in 60% solution state {RER} production |  |  | ecoinvent process modified | The Ecoinvent process alkyd paint production, white, solvent-based, product in 60% solution state RER was modified to enhance its colorless nature by removing the contribution of titanium dioxide (0.505 kg per kg of product). |

Table LCI13: LCI of Wood coating, yatch deck (teak) {RER} production

| Process | Quantity | Unit | Database | Notes |
| --- | --- | --- | --- | --- |
| Wood coating, yatch deck (teak) {RER} production | 1 | kg |  | Data from "Life Cycle Assessment in Green Chemistry: A Comparison of Various Industrial Wood Surface Coatings" by Linda M. Gustafsson and Pål Börjesson; "Life Cycle Assessment of UV-Curable Biobased Wood Flooring Coatings" by Mahdokht Montazeri and Matthew J. Eckelman. |
| Materials/fuels |  |  |  |  |
| Water, deionised {Europe without Switzerland} ¦ market for water, deionised ¦ APOS, U | 0.8 | kg | from ecoinvent 3.9.1 database |  |
| Petroleum slack wax {GLO}¦ market for petroleum slack wax ¦ APOS, U | 0.12 | kg | from ecoinvent 3.9.1 database |  |
| Acrylic acid {RER}¦ market for acrylic acid ¦ APOS, U | 0.04 | kg | from ecoinvent 3.9.1 database |  |
| Methyl methacrylate {RER}¦ market for methyl methacrylate ¦ APOS, U | 0.04 | kg | from ecoinvent 3.9.1 database |  |