Electronic Supporting Information

Methanol to Polypropylene: Life Cycle Assessment and a Preliminary Social Impact Analysis

Alice Vardaro¹,a, Francesco Arfelli¹,a*,  Fabrizio Passarini¹,² , Daniele Cespi¹,²

1Department of Industrial Chemistry “Toso Montanari”, University of Bologna, via Piero Gobetti 85, 40129 Bologna, Italy.

2Interdepartmental Centre of Industrial Research “Renewable Resources, Environment, Sea and Energy”, University of Bologna, via Angherà 22, 47922 Rimini, Italy.

aThe authors equally contributed to the article

Corresponding author: francesco.arfelli3@unibo.it

# METHODOLOGICAL CHOICES

- The choice of adopting the “avoided products” approach to deal with the multifunctionality of the system is supported by ISO 14040 (International Organization for Standardization (ISO) 2006). The ISO suggests to rely on a “system expansion” approach when, in multifunctional systems, it is not possible to directly allocate inputs and outputs involved in a unit process to a specific product. In addition, the ISO ranks this option as preferable with respect to physical and economic allocation.

- The literature lacks representative and comprehensive data to account for the material and energy flows associated with membrane gas separation phase in LCA. In the absence of complete data, assumptions are often necessary (Cellura et al. 2011).When input data are not fully supported by reliable references, the impact of such assumptions can be evaluated through sensitivity analysis, which helps quantify the uncertainty and assess its influence on the results (De Souza et al. 2016). Accordingly, the authors adopted a conservative yet arbitrary separation efficiency of 95% to ensure the separation stage was not entirely omitted. This assumption was subsequently tested through a dedicated sensitivity analysis (see Sensitivity and uncertainty analysis section).

- The biomass gasification process (latter one) specific to China is not available in the ecoinvent database and the available alternatives were between "CH" and "RoW". The choice of "CH" was motivated by the fact that, although "RoW" represents the same sequence of processes as "CH", it includes a series of geographically specific variables that are more complex to control and interpret. The use of "CH" allowed us to keep the focus on the technological stages characterising the biomass-to-methanol conversion, rather than on the geographic assumptions behind the background data. However, it is important to specify that the CC impact between the RoW version is 2.67 times higher. Such a difference is not due to the technology, since the material flow remains identical between the two options, but to the type of electricity consumed. In particular, the RoW version shows a 45.5% contribution of electricity, against the 3.67% in the case of CH. The discrepancy is accordingly attributable to the geographical variability intrinsically included in the two proxies.

Figure S1: Flowsheet of the MTP synthesis according to Lurgi’s technology, adapted from (Gogate 2019).

Figure S2: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by methane steam reforming (CO2tM-dmCO2 methane H2)

Table S1: Inventory of methanol (FU = 1 kg of propylene) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by methane steam reforming (CO2tM-dmCO2 methane H2)

| Process | Required amount | Unit |
| --- | --- | --- |
| MeOH synthesis | 7.28E-01 | kg |
| Hydrogen production, steam methane reforming ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 9.16E-02 | kg |
| Market group for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - Europe without Switzerland | 3.87E-01 | m3 |
| Chemical factory construction, organics ¦ chemical factory, organics ¦ APOS, U - RER | 4.57E-11 | Item(s) |
| Market for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - CH | 2.70E-03 | m3 |
| Market for zeolite, powder ¦ zeolite, powder ¦ APOS, U - GLO | 7.54E-05 | kg |
| Market for nickel, class 1 ¦ nickel, class 1 ¦ APOS, U - GLO | 1.73E-05 | kg |
| Market for copper oxide ¦ copper oxide ¦ APOS, U - GLO | 3.09E-05 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - Europe without Switzerland | 3.77E-01 | kg |
| Market for aluminium oxide, metallurgical ¦ aluminium oxide, metallurgical ¦ APOS, U - IAI Area, EU27 & EFTA | 4.55E-05 | kg |
| Market for zinc oxide ¦ zinc oxide ¦ APOS, U - GLO | 3.17E-05 | kg |
| Market for chromium trioxide, flakes ¦ chromium trioxide, flakes ¦ APOS, U - GLO | 3.07E-06 | kg |
| Market for molybdenum trioxide ¦ molybdenum trioxide ¦ APOS, U - GLO | 1.43E-06 | kg |
| Market for magnesium oxide ¦ magnesium oxide ¦ APOS, U - GLO | 2.39E-06 | kg |
| Market for quicklime, milled, packed ¦ quicklime, milled, packed ¦ APOS, U - RER | 4.10E-06 | kg |
| Market for portafer ¦ portafer ¦ APOS, U - GLO | 2.67E-05 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - CH | 1.36E-03 | kg |
| Market for silica sand ¦ silica sand ¦ APOS, U - GLO | 9.90E-07 | kg |
| RWGS | 6.36E-01 | kg |
| Hydrogen production, steam methane reforming ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 4.58E-02 | kg |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - RoW | 4.09E-04 | m3 |
| Purified CO2 | 1.00E+00 | kg |
| Cogeneration of dedicated biomass | 1.05E+00 | kg |
| _heat production, hardwood chips from forest, at furnace 50kW ¦ heat, central or small-scale, other than natural gas ¦ APOS, U - CH | 9.78E-01 | MJ |
| Heat production, natural gas, at boiler modulating <100kW ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |

Figure S3: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by coal gasification (CO2tM- dmCO2 coal H2

Table S2: Inventory of methanol (FU = 1 kg of propylene) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by coal gasification (CO2tM- dmCO2 coal H2)

| Process | Required amount | Unit |
| --- | --- | --- |
| Meoh synthesis | 7.28E-01 | kg |
| Hydrogen production, coal gasification ¦ hydrogen, gaseous, low pressure ¦ APOS, U - CN | 9.16E-02 | kg |
| Market group for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - CN | 1.32E+00 | MJ |
| Market for hard coal ¦ hard coal ¦ APOS, U - CN | 5.66E-01 | kg |
| Market for chemical factory, organics ¦ chemical factory, organics ¦ APOS, U - GLO | 4.11E-11 | Item(s) |
| Market for methanol ¦ methanol ¦ APOS, U - CN | 3.71E-04 | kg |
| Market for non-ionic surfactant ¦ non-ionic surfactant ¦ APOS, U - GLO | 2.84E-04 | kg |
| Market for hard coal ash ¦ hard coal ash ¦ APOS, U - row | 6.22E-02 | kg |
| Market for cobalt ¦ cobalt ¦ APOS, U - GLO | 1.51E-05 | kg |
| Market for waste gypsum ¦ waste gypsum ¦ APOS, U - row | 2.30E-02 | kg |
| Market for molybdenum trioxide ¦ molybdenum trioxide ¦ APOS, U - GLO | 4.09E-05 | kg |
| Market for aluminium oxide, metallurgical ¦ aluminium oxide, metallurgical ¦ APOS, U - row | 8.19E-05 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - row | 5.00E-01 | kg |
| Market for wastewater, average ¦ wastewater, average ¦ APOS, U - row | 5.00E-04 | m3 |
| Market for limestone, crushed, washed ¦ limestone, crushed, washed ¦ APOS, U - row | 1.34E-02 | kg |
| Market for hazardous waste, for underground deposit ¦ hazardous waste, for underground deposit ¦ APOS, U - row | 5.60E-05 | kg |
| RWGS | 6.36E-01 | kg |
| Hydrogen production, coal gasification ¦ hydrogen, gaseous, low pressure ¦ APOS, U - row | 4.58E-02 | kg |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - row | 4.09E-04 | m3 |
| Purified CO2 | 1.00E+00 | kg |
| Cogeneration of dedicated biomass (1) | 1.05E+00 | kg |
| _heat production, hardwood chips from forest, at furnace 50kw ¦ heat, central or small-scale, other than natural gas ¦ APOS, U - CH | 9.78E-01 | MJ |
| Heat production, natural gas, at boiler modulating <100kw ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |

Figure S4: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by petroleum refinery (CO2tM- dmCO2 petroleum H2)

Table S3: Inventory of methanol (1 kg of propylene) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by petroleoum refinery (CO2tM- dmCO2 petroleum H2)

| Process | Required amount | Unit |
| --- | --- | --- |
| MeOH synthesis | 7.28E-01 | kg |
| Hydrogen production, gaseous, petroleum refinery operation ¦ hydrogen, gaseous ¦ APOS, U - Europe without Switzerland | 9.16E-02 | kg |
| Market for petroleum ¦ petroleum ¦ APOS, U - GLO | 2.78E-01 | kg |
| Market for lime, hydrated, packed ¦ lime, hydrated, packed ¦ APOS, U - RER | 3.53E-04 | kg |
| Market for refinery sludge ¦ refinery sludge ¦ APOS, U - Europe without Switzerland | 1.38E-04 | kg |
| Market for hazardous waste, for incineration ¦ hazardous waste, for incineration ¦ APOS, U - Europe without Switzerland | 7.75E-05 | kg |
| Market for quicklime, milled, packed ¦ quicklime, milled, packed ¦ APOS, U - RER | 7.89E-05 | kg |
| Market for platinum ¦ platinum ¦ APOS, U - GLO | 1.28E-09 | kg |
| Petroleum refinery construction ¦ petroleum refinery ¦ APOS, U - RER | 1.52E-12 | Item(s) |
| Market for sodium hydroxide, without water, in 50% solution state ¦ sodium hydroxide, without water, in 50% solution state ¦ APOS, U - GLO | 1.09E-05 | kg |
| Market for hydrochloric acid, without water, in 30% solution state ¦ hydrochloric acid, without water, in 30% solution state ¦ APOS, U - RER | 2.17E-05 | kg |
| Market for zeolite, powder ¦ zeolite, powder ¦ APOS, U - GLO | 2.39E-06 | kg |
| Market for aluminium oxide, non-metallurgical ¦ aluminium oxide, non-metallurgical ¦ APOS, U - IAI Area, EU27 & EFTA | 4.09E-06 | kg |
| Market for waste gypsum ¦ waste gypsum ¦ APOS, U - Europe without Switzerland | 2.85E-04 | kg |
| Market for cobalt ¦ cobalt ¦ APOS, U - GLO | 8.53E-08 | kg |
| Market for lanthanum oxide ¦ lanthanum oxide ¦ APOS, U - GLO | 8.53E-08 | kg |
| Market for silicone product ¦ silicone product ¦ APOS, U - RER | 8.53E-07 | kg |
| Market for hazardous waste, for underground deposit ¦ hazardous waste, for underground deposit ¦ APOS, U - RER | 8.50E-06 | kg |
| Market for nickel, class 1 ¦ nickel, class 1 ¦ APOS, U - GLO | 8.53E-08 | kg |
| Market for molybdenum trioxide ¦ molybdenum trioxide ¦ APOS, U - GLO | 8.53E-08 | kg |
| Market for kaolin ¦ kaolin ¦ APOS, U - GLO | 8.53E-07 | kg |
| RWGS | 6.36E-01 | kg |
| Hydrogen production, gaseous, petroleum refinery operation ¦ hydrogen, gaseous, low pressure ¦ APOS, U - Europe without Switzerland | 4.58E-02 | kg |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - row | 4.09E-04 | m3 |
| Purified CO2 | 1.00E+00 | kg |
| Cogeneration of dedicated biomass | 1.05E+00 | kg |
| _heat production, hardwood chips from forest, at furnace 50kw ¦ heat, central or small-scale, other than natural gas ¦ APOS, U - CH | 9.78E-01 | MJ |
| Heat production, natural gas, at boiler modulating <100kw ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |

Figure S5: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by chloro-alcaline electrolysis (CO2tM- dmCO2 electrolysis H2)

Table S4: Inventory of methanol (FU = 1 kg of propylene) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by chloro-alcaline electrolysis (CO2tM- dmCO2 electrolysis H2)

| Process | Required amount |  | Unit |  |
| --- | --- | --- | --- | --- |
| MeOH synthesis | 7.28E-01 | kg |  |
| Chlor-alkali electrolysis, membrane cell ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 9.16E-02 | kg |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - DE | 1.60E+00 | MJ |  |
| Market for sodium chloride, powder ¦ sodium chloride, powder ¦ APOS, U - GLO | 7.25E-01 | kg |  |
| Market for heat, from steam, in chemical industry ¦ heat, from steam, in chemical industry ¦ APOS, U - RER | 8.90E-01 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - PL | 1.94E-01 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - NL | 3.50E-01 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - BE | 5.06E-01 | MJ |  |
| Chemical factory construction, organics ¦ chemical factory, organics ¦ APOS, U - RER | 1.55E-10 | Item(s) |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - HU | 1.78E-01 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - GB | 2.44E-01 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - ES | 1.49E-01 | MJ |  |
| Market for hydrochloric acid, without water, in 30% solution state ¦ hydrochloric acid, without water, in 30% solution state ¦ APOS, U - RER | 1.28E-02 | kg |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - CZ | 3.55E-02 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - FR | 2.09E-01 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - PT | 5.82E-02 | MJ |  |
| Market for soda ash, dense ¦ soda ash, dense ¦ APOS, U - GLO | 3.23E-03 | kg |  |
| Market for potassium chloride ¦ potassium chloride ¦ APOS, U - RER | 2.20E-03 | kg |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - FI | 3.50E-02 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - IT | 9.72E-03 | MJ |  |
| Market for nitrogen, liquid ¦ nitrogen, liquid ¦ APOS, U - RER | 5.03E-03 | kg |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - SE | 6.68E-02 | MJ |  |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - NO | 1.70E-01 | MJ |  |
| Market for sodium hydrogen sulfite ¦ sodium hydrogen sulfite ¦ APOS, U - GLO | 6.34E-04 | kg |  |
| Market for sulfuric acid ¦ sulfuric acid ¦ APOS, U - RER | 6.37E-03 | kg |  |
| Market for compressed air, 600 kpa gauge ¦ compressed air, 600 kpa gauge ¦ APOS, U - RER | 1.35E-02 | m3 |  |
| Market for chemical, inorganic ¦ chemical, inorganic ¦ APOS, U - GLO | 2.22E-04 | kg |  |
| Market for wastewater, average ¦ wastewater, average ¦ APOS, U - Europe without Switzerland | 9.92E-04 | m3 |  |
| Market for hazardous waste, for incineration ¦ hazardous waste, for incineration ¦ APOS, U - Europe without Switzerland | 1.36E-04 | kg |  |
| Market for carbon dioxide, liquid ¦ carbon dioxide, liquid ¦ APOS, U - RER | 3.71E-04 | kg |  |
| Market for hazardous waste, for underground deposit ¦ hazardous waste, for underground deposit ¦ APOS, U - RER | 1.03E-03 | kg |  |
| Market for chemical, organic ¦ chemical, organic ¦ APOS, U - GLO | 7.72E-05 | kg |  |
| Market group for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - Europe without Switzerland | 3.56E-04 | m3 |  |
| Market for hydrogen peroxide, without water, in 50% solution state ¦ hydrogen peroxide, without water, in 50% solution state ¦ APOS, U - RER | 1.15E-04 | kg |  |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - Europe without Switzerland | 7.20E-01 | kg |  |
| Market for platinum ¦ platinum ¦ APOS, U - GLO | 2.02E-09 | kg |  |
| Market for calcium chloride ¦ calcium chloride ¦ APOS, U - RER | 2.46E-04 | kg |  |
| Market for hydrogen, gaseous, low pressure ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 1.16E-05 | kg |  |
| Market for sodium sulfite ¦ sodium sulfite ¦ APOS, U - RER | 1.72E-04 | kg |  |
| Market for sodium sulfide ¦ sodium sulfide ¦ APOS, U - GLO | 2.10E-05 | kg |  |
| Market for potassium carbonate ¦ potassium carbonate ¦ APOS, U - GLO | 1.30E-05 | kg |  |
| Market for disodium disulphite ¦ disodium disulphite ¦ APOS, U - GLO | 3.27E-05 | kg |  |
| Market for refrigerant R134a ¦ refrigerant R134a ¦ APOS, U - GLO | 1.95E-06 | kg |  |
| Market for barium carbonate ¦ barium carbonate ¦ APOS, U - GLO | 1.92E-05 | kg |  |
| Market for inert waste ¦ inert waste ¦ APOS, U - RER | 1.16E-03 | kg |  |
| Market for anionic resin ¦ anionic resin ¦ APOS, U - RER | 4.10E-06 | kg |  |
| Market for titanium dioxide ¦ titanium dioxide ¦ APOS, U - RER | 2.02E-06 | kg |  |
| Market for hazardous waste, for incineration ¦ hazardous waste, for incineration ¦ APOS, U - CH | 3.55E-06 | kg |  |
| Market for activated carbon, granular ¦ activated carbon, granular ¦ APOS, U - GLO | 2.32E-06 | kg |  |
| Market for wastewater, average ¦ wastewater, average ¦ APOS, U - CH | 2.49E-05 | m3 |  |
| Market for cationic resin ¦ cationic resin ¦ APOS, U - RER | 4.10E-06 | kg |  |
| Market for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - CH | 2.49E-06 | m3 |  |
| Market for perfluoropentane ¦ perfluoropentane ¦ APOS, U - GLO | 4.65E-08 | kg |  |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - CH | 2.59E-03 | kg |  |
| Market for ammonia, anhydrous, liquid ¦ ammonia, anhydrous, liquid ¦ APOS, U - RER | 9.32E-08 | kg |  |
| Market for oxygen, liquid ¦ oxygen, liquid ¦ APOS, U - RER | 3.67E-07 | kg |  |
| Market for ethylene glycol ¦ ethylene glycol ¦ APOS, U - RER | 6.52E-08 | kg |  |
| RWGS | 6.36E-01 | kg |  |
| Chlor-alkali electrolysis, membrane cell ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 4.58E-02 | kg |  |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - row | 4.09E-04 | m3 |  |
| Purified CO2 | 1.00E+00 | kg |  |
| Cogeneration of dedicated biomass | 1.05E+00 | kg |  |
| _heat production, hardwood chips from forest, at furnace 50kw ¦ heat, central or small-scale, other than natural gas ¦ APOS, U (3) - CH | 9.78E-01 | MJ |  |
| Heat production, natural gas, at boiler modulating <100kw ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |  |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |  |

Figure S6: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by wind electrolysis (CO2tM- dmCO2 wind electrolysis H2)

Table S5: Inventory of methanol (FU = 1 kg of propylene) starting from biomass from dedicated crops as raw material and involving the employment of hydrogen derived by wind electrolysis (CO2tM- dmCO2 wind electrolysis H2)

| Process | Required amount | Unit |
| --- | --- | --- |
| MeOH synthesis | 7.28E-01 | kg |
| r-H2 production | 9.16E-02 | kg |
| Electricity production, wind, >3MW turbine, onshore ¦ electricity, high voltage ¦ APOS, U - IT | 9.09E-01 | MJ |
| RWGS | 6.36E-01 | kg |
| R-H2 production | 4.58E-02 | kg |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - row | 4.09E-04 | m3 |
| Purified CO2 | 1.00E+00 | kg |
| Cogeneration of dedicated biomass | 1.05E+00 | kg |
| _heat production, hardwood chips from forest, at furnace 50kw ¦ heat, central or small-scale, other than natural gas ¦ APOS, U - CH | 9.78E-01 | MJ |
| Heat production, natural gas, at boiler modulating <100kw ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |

Figure S7: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from waste biomass raw material and involving the employment of hydrogen derived by methane steam reforming (CO2tM- wcwCO2 methane H2)

Table S6: Inventory of methanol (FU = 1 kg of propylene) starting from biomass from waste biomass raw material and involving the employment of hydrogen derived by methane steam reforming (CO2tM- wcwCO2 methane H2)

| Process | Required amount | Unit |
| --- | --- | --- |
| MeOH synthesis | 7.28E-01 | kg |
| Hydrogen production, steam methane reforming ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 9.16E-02 | kg |
| Market group for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - Europe without Switzerland | 3.87E-01 | m3 |
| Chemical factory construction, organics ¦ chemical factory, organics ¦ APOS, U - RER | 4.57E-11 | Item(s) |
| Market for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - CH | 2.70E-03 | m3 |
| Market for zeolite, powder ¦ zeolite, powder ¦ APOS, U - GLO | 7.54E-05 | kg |
| Market for nickel, class 1 ¦ nickel, class 1 ¦ APOS, U - GLO | 1.73E-05 | kg |
| Market for copper oxide ¦ copper oxide ¦ APOS, U - GLO | 3.09E-05 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - Europe without Switzerland | 3.77E-01 | kg |
| Market for aluminium oxide, metallurgical ¦ aluminium oxide, metallurgical ¦ APOS, U - IAI Area, EU27 & EFTA | 4.55E-05 | kg |
| Market for zinc oxide ¦ zinc oxide ¦ APOS, U - GLO | 3.17E-05 | kg |
| Market for chromium trioxide, flakes ¦ chromium trioxide, flakes ¦ APOS, U - GLO | 3.07E-06 | kg |
| Market for molybdenum trioxide ¦ molybdenum trioxide ¦ APOS, U - GLO | 1.43E-06 | kg |
| Market for magnesium oxide ¦ magnesium oxide ¦ APOS, U - GLO | 2.39E-06 | kg |
| Market for quicklime, milled, packed ¦ quicklime, milled, packed ¦ APOS, U - RER | 4.10E-06 | kg |
| Market for portafer ¦ portafer ¦ APOS, U - GLO | 2.67E-05 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - CH | 1.36E-03 | kg |
| Market for silica sand ¦ silica sand ¦ APOS, U - GLO | 9.90E-07 | kg |
| RWGS | 6.36E-01 | kg |
| Hydrogen production, steam methane reforming ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 4.58E-02 | kg |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - row | 4.09E-04 | m3 |
| Purified CO2 | 1.00E+00 | kg |
| Cogeneration of dedicated biomass | 1.05E+00 | kg |
| _heat production, hardwood chips from forest, at furnace 50kw ¦ heat, central or small-scale, other than natural gas ¦ APOS, U (5) - CH | 9.78E-01 | MJ |
| Heat production, natural gas, at boiler modulating <100kw ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |

Figure S8: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from biomass obtained by a sustainably managed forest as raw material and involving the employment of hydrogen derived by methane steam reforming (CO2tM- smCO2 methane H2)

Table S7: Inventory of methanol (FU = 1 kg of propylene) starting from biomass obtained by a sustainably managed forest as raw material and involving the employment of hydrogen derived by methane steam reforming (CO2tM- smCO2 methane H2)

| Process | Required amount | Unit |
| --- | --- | --- |
| MeOH synthesis | 7.28E-01 | kg |
| Hydrogen production, steam methane reforming ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 9.16E-02 | kg |
| Market group for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - Europe without Switzerland | 3.87E-01 | m3 |
| Chemical factory construction, organics ¦ chemical factory, organics ¦ APOS, U - RER | 4.57E-11 | Item(s) |
| Market for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - CH | 2.70E-03 | m3 |
| Market for zeolite, powder ¦ zeolite, powder ¦ APOS, U - GLO | 7.54E-05 | kg |
| Market for nickel, class 1 ¦ nickel, class 1 ¦ APOS, U - GLO | 1.73E-05 | kg |
| Market for copper oxide ¦ copper oxide ¦ APOS, U - GLO | 3.09E-05 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - Europe without Switzerland | 3.77E-01 | kg |
| Market for aluminium oxide, metallurgical ¦ aluminium oxide, metallurgical ¦ APOS, U - IAI Area, EU27 & EFTA | 4.55E-05 | kg |
| Market for zinc oxide ¦ zinc oxide ¦ APOS, U - GLO | 3.17E-05 | kg |
| Market for chromium trioxide, flakes ¦ chromium trioxide, flakes ¦ APOS, U - GLO | 3.07E-06 | kg |
| Market for molybdenum trioxide ¦ molybdenum trioxide ¦ APOS, U - GLO | 1.43E-06 | kg |
| Market for magnesium oxide ¦ magnesium oxide ¦ APOS, U - GLO | 2.39E-06 | kg |
| Market for quicklime, milled, packed ¦ quicklime, milled, packed ¦ APOS, U - RER | 4.10E-06 | kg |
| Market for portafer ¦ portafer ¦ APOS, U - GLO | 2.67E-05 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - CH | 1.36E-03 | kg |
| Market for silica sand ¦ silica sand ¦ APOS, U - GLO | 9.90E-07 | kg |
| RWGS | 6.36E-01 | kg |
| Hydrogen production, steam methane reforming ¦ hydrogen, gaseous, low pressure ¦ APOS, U - RER | 4.58E-02 | kg |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - row | 4.09E-04 | m3 |
| Purified CO2 | 1.00E+00 | kg |
| Cogeneration of dedicated biomass | 1.05E+00 | kg |
| _heat production, hardwood chips from forest, at furnace 50kw ¦ heat, central or small-scale, other than natural gas ¦ APOS, U - CH | 9.78E-01 | MJ |
| Heat production, natural gas, at boiler modulating <100kw ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |

Figure S9: Product system representing the synthesis of methanol (Level I) and propylene (Level II) starting from waste biomass raw material and involving the employment of hydrogen derived by wind electrolysis (CO2tM- wcwCO2 wind electrolysis H2)

Table S8: Inventory of methanol (FU = 1 kg of propylene) starting from waste biomass raw material and involving the employment of hydrogen derived by wind electrolysis (CO2tM- wcwCO2 wind electrolysis H2)

| Process | Required amount | Unit |
| --- | --- | --- |
| MeOH synthesis | 7.28E-01 | kg |
| r-H2 production | 9.16E-02 | kg |
| Electricity production, wind, >3MW turbine, onshore ¦ electricity, high voltage ¦ APOS, U - IT | 9.09E-01 | MJ |
| RWGS | 6.36E-01 | kg |
| R-H2 production | 4.58E-02 | kg |
| Treatment of wastewater, unpolluted, wastewater treatment ¦ wastewater, unpolluted ¦ APOS, U - row | 4.09E-04 | m3 |
| Purified CO2 | 1.00E+00 | kg |
| Cogeneration of dedicated biomass | 1.05E+00 | kg |
| _heat production, hardwood chips from forest, at furnace 50kw ¦ heat, central or small-scale, other than natural gas ¦ APOS, U - CH | 9.78E-01 | MJ |
| Heat production, natural gas, at boiler modulating <100kw ¦ heat, central or small-scale, natural gas ¦ APOS, U - Europe without Switzerland | -3.52E+00 | MJ |
| Electricity, high voltage, production mix ¦ electricity, high voltage ¦ APOS, U - IT | -2.56E+00 | MJ |

Figure S10: Product system representing the Natural Gas to Methanol (NGtM) scenario.

Table S9: Inventory of methanol (FU = 1 kg of propylene) representing the Natural Gas to Methanol (NGtM) scenario

| Process | Required amount | Unit |
| --- | --- | --- |
| Methanol production, natural gas reforming ¦ methanol ¦ APOS, U - CN | 7.28E-01 | kg |
| Market for natural gas, high pressure ¦ natural gas, high pressure ¦ APOS, U - CN | 6.47E-01 | m3 |
| Market group for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - CN | 2.32E-01 | MJ |
| Market for nickel, class 1 ¦ nickel, class 1 ¦ APOS, U - GLO | 1.46E-05 | kg |
| Market for copper oxide ¦ copper oxide ¦ APOS, U - GLO | 6.55E-05 | kg |
| Market for methanol factory ¦ methanol factory ¦ APOS, U - GLO | 2.71E-11 | Item(s) |
| Market for aluminium oxide, non-metallurgical ¦ aluminium oxide, non-metallurgical ¦ APOS, U - row | 1.75E-04 | kg |
| Market for molybdenum trioxide ¦ molybdenum trioxide ¦ APOS, U - GLO | 7.28E-06 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - row | 2.58E-01 | kg |
| Market for wastewater, average ¦ wastewater, average ¦ APOS, U - row | 2.15E-04 | m3 |
| Market for heat, district or industrial, natural gas ¦ heat, district or industrial, natural gas ¦ APOS, U - row | 7.28E-03 | MJ |
| Market for zinc oxide ¦ zinc oxide ¦ APOS, U - GLO | 2.18E-05 | kg |

Figure S11: Product system representing the Coal to Methanol (CtM) scenario

Table S10: Inventory of methanol (Level I, FU = 1 kg of propylene) representing the Coal to Methanol (CtM) scenario

| Process | Required amount | Unit |
| --- | --- | --- |
| Methanol production, coal gasification ¦ methanol ¦ APOS, U (copy) - CN | 7.28E-01 | kg |
| Market for oxygen, liquid ¦ oxygen, liquid ¦ APOS, U - row | 9.76E-01 | kg |
| Market for hard coal ¦ hard coal ¦ APOS, U - CN | 1.11E+00 | kg |
| Market for nitrogen, liquid ¦ nitrogen, liquid ¦ APOS, U - row | 4.15E-01 | kg |
| Market for heat, from steam, in chemical industry ¦ heat, from steam, in chemical industry ¦ APOS, U - row | 1.32E+00 | MJ |
| Market group for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - CN | 2.88E-01 | MJ |
| Market for chemical factory, organics ¦ chemical factory, organics ¦ APOS, U - GLO | 2.91E-10 | Item(s) |
| Market for heat, district or industrial, natural gas ¦ heat, district or industrial, natural gas ¦ APOS, U - row | 2.04E-01 | MJ |
| Market for hard coal ash ¦ hard coal ash ¦ APOS, U - row | 1.16E-01 | kg |
| Market for tap water ¦ tap water ¦ APOS, U - row | 1.42E-01 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - row | 4.62E-02 | kg |
| Market for wastewater, average ¦ wastewater, average ¦ APOS, U - row | 4.62E-05 | m3 |

Figure S12: Product system representing the Biomass to Methanol (BMtM) scenario.

Table S11: Inventory of methanol (Level I, FU = 1 kg of propylene) representing the Biomass to Methanol (BMtM) scenario

| Process | Required amount | Unit |
| --- | --- | --- |
| Methanol production, biomass gasification ¦ methanol, from biomass ¦ APOS, U (copy) - CH | 7.28E-01 | kg |
| Market for synthetic gas ¦ synthetic gas ¦ APOS, U - CH | 5.19E+00 | m3 |
| Market for electricity, medium voltage ¦ electricity, medium voltage ¦ APOS, U - CH | 7.25E-01 | MJ |
| Market for methanol factory ¦ methanol factory ¦ APOS, U - GLO | 2.70E-11 | Item(s) |
| Market for wastewater, average ¦ wastewater, average ¦ APOS, U - CH | 3.87E-03 | m3 |
| Market for copper oxide ¦ copper oxide ¦ APOS, U - GLO | 6.55E-05 | kg |
| Market for aluminium oxide, non-metallurgical ¦ aluminium oxide, non-metallurgical ¦ APOS, U - row | 1.75E-04 | kg |
| Market for nickel, class 1 ¦ nickel, class 1 ¦ APOS, U - GLO | 1.46E-05 | kg |
| Market for molybdenum ¦ molybdenum ¦ APOS, U - GLO | 7.28E-06 | kg |
| Market for water, deionised ¦ water, deionised ¦ APOS, U - CH | 6.19E-01 | kg |
| Market for zinc ¦ zinc ¦ APOS, U - GLO | 2.18E-05 | kg |

Table S12: Inventory of Methanol to Propylene synthesis (Propylene production, from methanol-to-propylene conversion | propylene | APOS, U -CN)

| Process | Required amount | Unit |
| --- | --- | --- |
| Propylene production, from methanol-to-propylene conversion ¦ propylene ¦ APOS, U (copy) - CN | 1.00E+00 | kg |
| Market for heat, from steam, in chemical industry ¦ heat, from steam, in chemical industry ¦ APOS, U - row | 1.67E+00 | MJ |
| Steam production, as energy carrier, in chemical industry ¦ heat, from steam, in chemical industry ¦ APOS, U - row | 1.67E+00 | MJ |
| Market for chemical factory, organics ¦ chemical factory, organics ¦ APOS, U - GLO | 3.92E-10 | Item(s) |
| Chemical factory construction, organics ¦ chemical factory, organics ¦ APOS, U - row | 2.63E-10 | Item(s) |
| Chemical factory construction, organics ¦ chemical factory, organics ¦ APOS, U - RER | 1.30E-10 | Item(s) |
| Market for cooling energy, at -25 °C ¦ cooling energy, at -25 °C ¦ APOS, U - GLO | 2.66E-02 | MJ |
| Cooling energy production, at -25 °C, propylene compression refrigeration system 1 MW ¦ cooling energy, at -25 °C ¦ APOS, U - GLO | 1.33E-02 | MJ |
| Cooling energy production, at -25 °C, ammonia compression refrigeration system 1 MW ¦ cooling energy, at -25 °C ¦ APOS, U - GLO | 1.33E-02 | MJ |
| Market for wastewater, average ¦ wastewater, average ¦ APOS, U - row | 1.26E-03 | m3 |
| Treatment of wastewater, average, wastewater treatment ¦ wastewater, average ¦ APOS, U - row | 5.19E-04 | m3 |
| Sewer grid construction, urban ¦ sewer grid, urban ¦ APOS, U - GLO | 1.04E-03 | m3 |
| Sewer grid construction, rural ¦ sewer grid, rural ¦ APOS, U - GLO | 2.17E-04 | m3 |

Table S13: Pedigree Matrix

|  | Amount of methanol produced | Energy (credit) | CO2 purified (to rWGS) | Hydrogen and CO2 input in rWGS | CO in output in rWGS |
| --- | --- | --- | --- | --- | --- |
| Reliability | 2 | 3 | 2 | 3 | 3 |
| Completeness | 1 | 1 | 2 | 1 | 1 |
| Temporal correlation | 1 | 1 | 3 | 1 | 1 |
| Geographical correlation | 1 | 1 | 1 | 1 | 1 |
| Technological correlation | 1 | 1 | 4 | 1 | 1 |
| Ub | 1.05 | 1.05 | 1.05 | 1.05 | 1.05 |

Table S14: Contribution analysis of the investigated scenarios of the CO2tM reaction in terms of kg CO2eq, Level I.

| GWP100 [kgCO2eq] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO2tM-wcCO2mH2 | CO2tM-wcCO2cH2 | CO2tM-wcCO2pH2 | CO2tM-wcCO2eH2 | CO2tM-wcCO2weH2 | CO2tM-wc0CO2mH2 | CO2tM-sCO2mH2 | CO2tM-wc0CO2weH2 |
| CO2 source | -4.10E-01 | -4.10E-01 | -4.10E-01 | -4.10E-01 | -4.10E-01 | -4.11E-01 | -4.09E-01 | -4.11E-01 |
| rWGS | 3.68E-01 | 6.89E-01 | 9.53E-02 | 2.64E-01 | 2.82E-03 | 3.68E-01 | 3.68E-01 | 2.82E-03 |
| H2 source | 7.35E-01 | 1.58E+00 | 1.05E-01 | 5.28E-01 | 5.56E-03 | 7.35E-01 | 7.35E-01 | 5.56E-03 |
| net | 6.93E-01 | 1.86E+00 | -2.10E-01 | 3.82E-01 | -4.02E-01 | 6.92E-01 | 6.93E-01 | -4.02E-01 |

Table S15: Standard Deviation associated with the midpoint categories of the ReCiPe 2016 method, Level I.

|  | IR (kBq Co-60-Eq) | LO (m2*a crop-Eq) | MET (kg 1,4-DCB-Eq) | WC (m3) | OD (kg CFC-11-Eq) | ME (kg N-Eq) | SO (kg Cu-Eq) | HOF (kg NOx-Eq) | FF (kg oil-Eq) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO2tM-wcCO2mH2 | 1.82E-03 | 1.94E-02 | 6.34E-03 | 1.07E-03 | -2.11E-08 | 4.61E-06 | 2.27E-03 | 3.27E-04 | 2.71E-01 |
| CO2tM-wcCO2cH2 | 2.43E-02 | 4.08E-02 | 4.33E-02 | 2.92E-04 | 3.79E-08 | 3.41E-05 | 1.34E-03 | 1.46E-03 | 2.73E-01 |
| CO2tM-wcCO2pH2 | 5.35E-03 | 1.93E-02 | -6.46E-03 | -2.27E-03 | 9.36E-08 | -5.82E-06 | -6.87E-04 | 2.75E-04 | 1.86E-01 |
| CO2tM-wcCO2eH2 | 2.19E-01 | 5.69E-02 | 9.90E-02 | 5.51E-03 | 2.77E-07 | 8.69E-05 | 8.23E-03 | 1.23E-03 | 7.06E-02 |
| CO2tM-wcCO2weH2 | -1.40E-03 | 1.90E-02 | -7.00E-03 | -2.59E-03 | -1.03E-07 | -9.02E-06 | -1.10E-03 | -3.21E-04 | -1.40E-01 |
| CO2tM-wc0CO2mH2 | 1.52E-03 | -7.08E-03 | 6.32E-03 | 1.05E-03 | -2.17E-08 | 4.55E-06 | 2.26E-03 | 3.23E-04 | 2.70E-01 |
| CO2tM-sCO2mH2 | 1.55E-03 | 2.40E-02 | 6.34E-03 | 1.06E-03 | -2.06E-08 | 4.67E-06 | 2.27E-03 | 3.30E-04 | 2.71E-01 |
| CO2tM-wc0CO2weH2 | -1.70E-03 | -7.49E-03 | -7.02E-03 | -2.60E-03 | -1.03E-07 | -9.08E-06 | -1.10E-03 | -3.25E-04 | -1.40E-01 |
| NGtM | 3.89E-03 | 2.07E-03 | 1.34E-02 | 1.00E-03 | 6.49E-08 | 1.70E-05 | 3.74E-03 | 1.00E-03 | 4.61E-01 |
| CtM | 8.39E-02 | 4.36E-02 | 9.39E-02 | 2.50E-02 | 3.57E-07 | 5.76E-05 | 5.97E-03 | 4.12E-03 | 7.11E-01 |
| BMtM | 4.42E-02 | 8.52E-01 | 2.23E-02 | 5.98E-03 | 9.34E-08 | 2.53E-05 | 2.47E-03 | 7.42E-04 | 3.87E-02 |
|  | FE (kg P-Eq) | FET (kg 1,4-DCB-Eq) | TET (kg 1,4-DCB-Eq) | HTc (kg 1,4-DCB-Eq) | HTnc (kg 1,4-DCB-Eq) | EOF (kg NOx-Eq) | TA kg SO2-Eq | PMF (kg PM2.5-Eq) |
| CO2tM-wcCO2mH2 | -1.97E-05 | 3.38E-03 | 2.00E+00 | 3.88E-02 | 4.11E-02 | 4.12E-04 | -2.72E-05 | 1.02E-04 |
| CO2tM-wcCO2cH2 | 5.63E-04 | 3.14E-02 | 1.05E+00 | 1.58E-01 | 1.44E+00 | 1.43E-03 | 2.22E-03 | 1.33E-03 |
| CO2tM-wcCO2pH2 | -3.20E-05 | -4.01E-03 | -8.38E-01 | -1.54E-02 | -5.09E-02 | 3.18E-04 | 7.66E-04 | 3.43E-04 |
| CO2tM-wcCO2eH2 | 5.40E-04 | 7.02E-02 | 8.27E+00 | 1.92E-01 | 1.46E+00 | 1.24E-03 | 2.03E-03 | 9.53E-04 |
| CO2tM-wcCO2weH2 | -3.30E-05 | -3.72E-03 | -1.82E+0 | -2.42E-02 | -2.18E-02 | -3.67E-04 | -4.20E-04 | -5.46E-05 |
| CO2tM-wc0CO2mH2 | -2.00E-05 | 3.37E-03 | 1.99E+00 | 3.86E-02 | 4.06E-02 | 4.07E-04 | -3.00E-05 | 1.01E-04 |
| CO2tM-sCO2mH2 | -1.95E-05 | 3.38E-03 | 2.00E+00 | 3.88E-02 | 4.10E-02 | 4.16E-04 | -2.60E-05 | 1.03E-04 |
| CO2tM-wc0CO2weH2 | -3.33E-05 | -3.73E-03 | -1.83E+0 | -2.43E-02 | -2.24E-02 | -3.72E-04 | -4.23E-04 | -5.58E-05 |
| NGtM | 2.77E-05 | 6.85E-03 | 3.97E+00 | 6.93E-02 | 1.38E-01 | 1.21E-03 | 6.54E-04 | 2.75E-04 |
| CtM | 8.16E-04 | 6.41E-02 | 7.92E+00 | 3.37E-01 | 2.38E+00 | 4.58E-03 | 5.03E-03 | 3.19E-03 |
| BMtM | 6.65E-05 | 1.51E-02 | 2.87E+00 | 6.30E-02 | 6.68E-01 | 8.10E-04 | 6.13E-04 | 2.54E-04 |

Table S16: Impacts associated with the midpoint categories of the ReCiPe 2016 method, Level I.

|  | IR (kBq Co-60-Eq) | LO (m2*a crop-Eq) | MET (kg 1,4-DCB-Eq) | WC (m3) | OD (kg CFC-11-Eq) | ME (kg N-Eq) | MRD (kg Cu-Eq) | HOF (kg NOx-Eq) | FD (kg oil-Eq) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NGtM | 1.5E-01 | 6.6E-03 | 2.7E-02 | 1.8E-03 | 9.7E-08 | 1.3E-05 | 3.2E-03 | 3.4E-04 | 6.6E-02 |
| CtM | 2.7E+00 | 8.7E-02 | 4.2E-01 | 2.1E-02 | 8.7E-07 | 1.3E-04 | 8.1E-03 | 2.6E-03 | 3.7E-01 |
| BMtM | 3.2E-01 | 6.1E-01 | 4.1E-02 | 4.4E-03 | 1.9E-07 | 2.8E-05 | 1.8E-03 | 6.0E-04 | 7.8E-02 |
| CO2tM-wcCO2mH2 | 3.5E-01 | 2.0E-02 | 3.3E-02 | 4.2E-03 | 2.5E-07 | 2.4E-05 | 2.4E-03 | 5.1E-04 | 7.6E-02 |
| CO2tM-wcCO2cH2 | 3.1E-01 | 3.5E-02 | 2.0E-01 | 4.6E-03 | 3.4E-07 | 3.1E-05 | 1.9E-03 | 6.3E-04 | 9.9E-02 |
| CO2tM-wcCO2pH2 | 5.1E-01 | 2.3E-02 | 4.9E-02 | 4.8E-03 | 3.8E-07 | 3.2E-05 | 1.8E-03 | 9.4E-04 | 9.4E-02 |
| CO2tM-wcCO2eH2 | 1.6E+00 | 7.1E-02 | 2.6E-01 | 1.8E-02 | 7.7E-07 | 1.1E-04 | 9.7E-03 | 2.2E-03 | 3.2E-01 |
| CO2tM-wcCO2weH2 | 5.0E-01 | 3.0E-02 | 7.2E-02 | 7.1E-03 | 3.8E-07 | 4.8E-05 | 2.6E-03 | 9.3E-04 | 1.4E-01 |
| CO2tM-wc0CO2mH2 | 9.1E-02 | 7.2E-03 | 3.1E-02 | 2.3E-03 | 1.8E-07 | 1.2E-05 | 1.8E-03 | 2.5E-04 | 3.9E-02 |
| CO2tM-sCO2mH2 | 2.0E-01 | 3.8E-02 | 5.4E-02 | 5.8E-03 | 3.3E-07 | 3.9E-05 | 2.7E-03 | 7.3E-04 | 1.1E-01 |
| CO2tM-wc0CO2weH2 | 6.6E-01 | 2.8E-02 | 7.9E-02 | 9.7E-03 | 4.4E-07 | 5.6E-05 | 3.1E-03 | 1.1E-03 | 1.6E-01 |
| MtP | 2.0E+00 | 1.7E-01 | 1.0E+00 | 4.3E-02 | 1.6E-06 | 2.8E-04 | 1.7E-02 | 5.0E-03 | 7.7E-01 |
|  | FE (kg P-Eq) | CC (kg CO2-Eq) | FET (kg 1,4-DCB-Eq) | TET (kg 1,4-DCB-Eq) | HTc (kg 1,4-DCB-Eq) | HTnc (kg 1,4-DCB-Eq) | EOF (kg NOx-Eq) | TA kg SO2-Eq | PMF (kg PM2.5-Eq) |
| NGtM | 2.0E-04 | 1.6E-01 | 1.9E-02 | 8.2E+00 | 8.7E-02 | 2.0E+00 | 4.3E-04 | 4.7E-04 | 2.0E-04 |
| CtM | 2.1E-03 | 1.4E+00 | 2.9E-01 | 2.1E+01 | 9.2E-01 | 2.8E+01 | 2.7E-03 | 4.7E-03 | 2.4E-03 |
| BMtM | 7.5E-04 | 2.9E-01 | 2.8E-02 | 5.7E+00 | 1.2E-01 | 2.3E+00 | 6.3E-04 | 1.0E-03 | 4.1E-04 |
| CO2tM-wcCO2mH2 | 6.7E-04 | 2.7E-01 | 2.3E-02 | 6.3E+00 | 1.5E-01 | 2.4E+00 | 5.4E-04 | 9.3E-04 | 4.1E-04 |
| CO2tM-wcCO2cH2 | 1.8E-03 | 5.9E-01 | 1.4E-01 | 3.6E+00 | 8.7E-01 | 8.4E+01 | 6.5E-04 | 1.2E-03 | 8.1E-04 |
| CO2tM-wcCO2pH2 | 6.0E-04 | 3.6E-01 | 3.3E-02 | 5.5E+00 | 1.1E-01 | 5.1E+00 | 9.6E-04 | 1.2E-03 | 5.3E-04 |
| CO2tM-wcCO2eH2 | 1.7E-03 | 1.2E+00 | 1.8E-01 | 2.2E+01 | 3.7E+00 | 9.0E+00 | 2.3E-03 | 4.3E-03 | 1.8E-03 |
| CO2tM-wcCO2weH2 | 7.0E-04 | 5.0E-01 | 5.0E-02 | 5.7E+00 | 1.8E-01 | 3.3E+00 | 9.5E-04 | 1.8E-03 | 7.4E-04 |
| CO2tM-wc0CO2mH2 | 3.1E-04 | 1.5E-01 | 2.2E-02 | 4.4E+00 | 7.5E-02 | 1.0E+00 | 2.8E-04 | 4.3E-04 | 1.8E-04 |
| CO2tM-sCO2mH2 | 2.3E-03 | 4.1E-01 | 3.7E-02 | 6.1E+00 | 1.6E-01 | 1.8E+00 | 7.6E-04 | 1.4E-03 | 5.9E-04 |
| CO2tM-wc0CO2weH2 | 7.5E-04 | 5.9E-01 | 5.5E-02 | 5.8E+00 | 1.6E-01 | 5.0E+00 | 1.1E-03 | 2.1E-03 | 8.5E-04 |
| MtP | 4.0E-03 | 2.8E+00 | 7.3E-01 | 4.2E+01 | 2.4E+00 | 1.3E+02 | 5.2E-03 | 9.5E-03 | 4.5E-03 |

Table S17: Social impact of the category Labor right. Dark red: very high risk, red: high risk, yellow: medium risk, green: low risk, blue: no evidence, white: no data.

| LABOR RIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| subcategory | issue | China | Italy | Germany |  |
| Wage assessments | Overall country-sector risk that average wage is below the benchmarks |  |  |  |  |
|  | Risk that Sector average wage is below Living Wage |  |  |  |  |
|  | Risk that Sector average wage is below Sweatfree Wage |  |  |  |  |
|  | Risk that Sector average wage is below country Minimum Wage |  |  |  |  |
| Workers in poverty | Overall risk employed population is living below international poverty line |  |  |  |  |
| child labor | Overall country-sector risk child labor |  |  |  |  |
|  | Risk of children missing out of childhood (Save the Children) |  |  |  |  |
|  | Risk related to children's rights in the workplace (index) |  |  |  |  |
|  | Overall country risk of child labor |  |  |  |  |
| Forced labor | Overall country-sector risk forced labor |  |  |  |  |
|  | Risk of trafficking in persons (US DoL) |  |  |  |  |
|  | Forced labor risk (Global Slavery Index) |  |  |  |  |
|  | Overall country risk of forced labor |  |  |  |  |
| Excessive working time | Overall risk excessive working time |  |  |  |  |
|  | Risk of working >48 hrs per week (ILO) |  |  |  |  |
|  | Lack of regulatory framework for working hours and its implementation (US DoS) |  |  |  |  |
| Freedom of association, collective bargaining, right to strike | Overall risk related to freedom of association, collective bargaining, right to strike |  |  |  |  |
|  | Lack of association & collective bargaining rights & right to strike (ITUC) |  |  |  |  |
|  | Collective bargaining coverage (ILO) |  |  |  |  |
| Migrant labor | Overall risk migrant workers |  |  |  |  |
|  | Net migration rate |  |  |  |  |
|  | Immigrants as a percentage of the population |  |  |  |  |
|  | Workers’ remittances and compensation paid per immigrant |  |  |  |  |
|  | Ratified international conventions or set up policies for immigrants |  |  |  |  |
|  | Evidence of risk to migrant Workers - qualitative |  |  |  |  |
|  | People of concern |  |  |  |  |
| Social benefits | Overall risk social benefits |  |  |  |  |
|  | Paid annual leave available to workers |  |  |  |  |
|  | Sick leave (pay, start, duration) |  |  |  |  |
|  | Other types of leave (child, adult, educational and health) |  |  |  |  |
|  | Maternal leave (pay & duration) |  |  |  |  |
|  | Parental leave (pay & duration) |  |  |  |  |
|  | Paternal leave (pay & duration) |  |  |  |  |
|  | Proportion of population covered by social assistance programs |  |  |  |  |
| Labor laws & conventions | Overall risk labor laws & conventions |  |  |  |  |
|  | Year minimum wage was last updated |  |  |  |  |
|  | Number of labor laws in country |  |  |  |  |
|  | Ratification of ILO Conventions |  |  |  |  |
|  | Number of labor laws by sector |  |  |  |  |
|  | Ratification of conventions by sector |  |  |  |  |
| Discrimination, equal opportunity | Overall prevalence of discrimination in the workplace (qualitative) |  |  |  |  |
| Unemployment | Overall (un)employment risk |  |  |  |  |
|  | Unemployment level |  |  |  |  |
|  | Vulnerable employment |  |  |  |  |
|  | Unemployment programs in place (ISSA) |  |  |  |  |

Table S18: Social impact of the category Health and safety. Dark red: very high risk, red: high risk, yellow: medium risk, green: low risk, blue: no evidence, white: no data.

| HEALTH AND SAFETY |  |  |  |  |
| --- | --- | --- | --- | --- |
| subcategory | issue | China | Italy | Germany |
| Occupational toxics & hazards | Overall risk occupational toxics & hazards |  |  |  |
|  | Occupational noise related risk |  |  |  |
|  | Occupational cancer related risk |  |  |  |
|  | Occupational asthma related risk |  |  |  |
|  | Occupational COPD related risk |  |  |  |
|  | Occupational Pneuma related risk |  |  |  |
|  | Occupational infectious disease related risk |  |  |  |
| Occupational injuries & fatalities | Overall risk occupational injuries & fatalities |  |  |  |
|  | Occupational road injuries & fatalities related risk |  |  |  |
|  | Occupational poison related injuries & fatalities |  |  |  |
|  | Occupational related fall injuries & fatalities |  |  |  |
|  | Occupational related fire/heat injuries & fatalities |  |  |  |
|  | Occupational related drowning injuries & fatalities |  |  |  |
|  | Occupational related exposure to mechanical forces injuries & fatalities |  |  |  |

Table S19: Social impact of the category Society. Dark red: very high risk, red: high risk, yellow: medium risk, green: low risk, blue: no evidence, white: no data.

| SOCIETY |  |  |  |  |
| --- | --- | --- | --- | --- |
| subcategory | issue | China | Italy | Germany |
| Indigenous rights | Overall country-sector risk related to indigenous rights infringement |  |  |  |
|  | Overall country risk of indigenous rights being infringed |  |  |  |
|  | Percent of population that is indigenous |  |  |  |
|  | Ratification conventions to protect indigenous rights |  |  |  |
|  | Number of laws to protect indigenous rights (ILO) |  |  |  |
| Gender Equality | Overall gender equality risk |  |  |  |
|  | Social Institutions and Gender Index (SIGI) |  |  |  |
|  | The Global Gender Gap Index (World Economic Forum) |  |  |  |
|  | Gender Inequality Index (GII) (UNDP) |  |  |  |
|  | Woman, Business and Law Index (World Bank) |  |  |  |
| High conflict zones | Overall risk high conflict |  |  |  |
|  | High conflict (Heidelberg Institute) |  |  |  |
|  | State Fragility Index (Center for Systemic Peace) |  |  |  |
|  | People under Threat (Minority Rights Group) |  |  |  |
|  | Global Peace Index |  |  |  |
| Human health issues - Non-Communicable Diseases (NCD) and other health risk | Overall risk non-communicable diseases and other health risk |  |  |  |
|  | Life expectancy at birth (years) |  |  |  |
|  | Under-five mortality rate |  |  |  |
|  | Age-standardized mortality rates for non-communicable diseases |  |  |  |
|  | Age-standardized mortality rates for injuries |  |  |  |
|  | Proportion of undernourished in population |  |  |  |
|  | Age-standardized mortality for cardiovascular diseases |  |  |  |
|  | Mortality rate attributed to household and ambient air pollution and exposure to unsafe WASH services |  |  |  |
|  | Population affected by natural disaster |  |  |  |
| Human health issues - Communicable diseases | Overall risk Human health issues - Communicable diseases Age standardized mortality rates from communicable diseases. |  |  |  |
|  | Cases of HIV |  |  |  |
|  | Cases of Tuberculosis |  |  |  |
|  | Notified cases of Malaria |  |  |  |
|  | Dengue Fever, incidence rate |  |  |  |

Table S20: Social impact of the category Governance. Dark red: very high risk, red: high risk, yellow: medium risk, green: low risk, blue: no evidence, white: no data.

| GOVERNANCE |  |  |  |  |
| --- | --- | --- | --- | --- |
| subcategory | issue | China | Italy | Germany |  |
| Legal system | Overall risk fragility in legal system |  |  |  |  |
|  | Worldwide Governance Rule of Law Indicator (World Bank) |  |  |  |  |
|  | Rule of Law - Independent Judiciary (Bertelsmann Transformation Index) |  |  |  |  |
|  | Rule of Law Index (World Justice Project) |  |  |  |  |
|  | Rule of Law (African Integrity Indicators) |  |  |  |  |
| Corruption | Overall risk corruption |  |  |  |  |
|  | Worldwide Governance Indicators Corruption Index (World Bank) |  |  |  |  |
|  | Corruption (Competitiveness report WEF) |  |  |  |  |
|  | Corruption Perception Index (CPI) (Transparency International) |  |  |  |  |
|  | CPI 3 year trend (2017-2020) (Transparency International) |  |  |  |  |
| Democracy & freedom of speech | Overall risk related to democracy & freedom of speechc |  |  |  |  |
|  | Democracy Index (EUI) |  |  |  |  |
|  | Freedom of speech (Freedom House) |  |  |  |  |
|  | Global State of Democracy indices (IDEA) |  |  |  |  |

Table S21: Social impact of the category Community. Dark red: very high risk, red: high risk, yellow: medium risk, green: low risk, blue: no evidence, white: no data.

| COMMUNITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| subcategory | issue | China | Italy | Germany |
| Access to improved source of drinking water | Overall access to improved source of drinking water |  |  |  |
|  | Urban access to improved source of drinking water |  |  |  |
|  | Rural access to improved source of drinking water |  |  |  |
| Access to improved source of sanitation | Overall access to improved source of sanitation |  |  |  |
|  | Urban access to improved source of sanitation |  |  |  |
| Children out of school | Overall risk children out of school |  |  |  |
|  | Children out of primary school, male |  |  |  |
|  | Children out of primary school, female |  |  |  |
| Access to health care | Overall risk related to access to health care |  |  |  |
|  | Hospital beds |  |  |  |
|  | Nurses/Midwives |  |  |  |
| Smallholders versus commercial farms (only agriculture sectors) | Overall risk related to (agri) smallholders |  |  |  |
|  | Percentage of family-owned farms in country |  |  |  |
|  | Percentage of commercially-owned farms in country |  |  |  |
|  | Smallholdings Land % < x hectares |  |  |  |
|  | Smallholdings Land % > x hectares |  |  |  |
| Access to electricity | Overall access to electricity |  |  |  |
| Property rights | Overall property rights (citizen's perceptions) |  |  |  |

REFERENCES

International Organization for Standardization (ISO). 2006. ISO 14040:2006/Amd 1:2020 Environmental management - Life cycle assessment - Requirements and guidelines. https://www.iso.org/standard/37456.html

Cellura M, Longo S, Mistretta M. 2011. Sensitivity analysis to quantify uncertainty in Life Cycle Assessment: The case study of an Italian tile. Renewable and Sustainable Energy Reviews. 15(9):4697–4705 [accessed 2025 Jul 26]. https://linkinghub.elsevier.com/retrieve/pii/S1364032111003273. https://doi.org/10.1016/j.rser.2011.07.082

De Souza RJ, Eisen RB, Perera S, Bantoto B, Bawor M, Dennis BB, Samaan Z, Thabane L. 2016. Best (but oft-forgotten) practices: sensitivity analyses in randomized controlled trials. The American Journal of Clinical Nutrition. 103(1):5–17 [accessed 2025 Jul 26]. https://linkinghub.elsevier.com/retrieve/pii/S0002916523120958. https://doi.org/10.3945/ajcn.115.121848

Gogate MR. 2019. Methanol-to-olefins process technology: current status and future prospects. Petroleum Science and Technology. 37(5):559–565 [accessed 2024 Aug 7]. https://www.tandfonline.com/doi/full/10.1080/10916466.2018.1555589. https://doi.org/10.1080/10916466.2018.1555589