Life Cycle Assessment of a DC-LINK capacitor used for automotive applications

Francesco Arfelli¹, Daniele Cespi¹,²*, Fabio Codecàc, Fabrizio Passarini¹,²

aDepartment of Industrial Chemistry “Toso Montanari”, University of Bologna, via Piero Gobetti 85, 40129 Bologna, Italy.

bInterdepartmental Centre of Industrial Research “Renewable Resources, Environment, Sea and Energy”, University of Bologna, via Angherà 22, 47922 Rimini, Italy.

cKemet Electronics Italia S.r.l.; Via Sagittario 1/3, 40037, Sasso Marconi (Bologna), Italy

Mail of the corresponding author: daniele.cespi2@unibo.it

KEYWORDS: Life Cycle Assessment, Electronic components, Automotive sustainability, Secondary materials sustainability, LCA applied to capacitors

Figure S 1: DC-Link C4EEGMX6560ABTJ.

Figure S 2: Electricity consumption allocated to the processes involved in the system.

Table S 1: Summarization of electricity consumption of the system.

| Site | Unit process number | Unit process | Electricity consumed (kWh/FU) |
| --- | --- | --- | --- |
| BG | I | Metalization | 1.81 |
| MK | II | Winder & flattering | 0.12 |
|  | III | Masking & spraying | 0.43 |
|  | IV | Element treatment | 0.90 |
|  | V | Element measurement | 0.12 |
|  | VI | Soldering | 0.39 |
|  | VII | Resing filling, Post & Final curing | 3.93 |
|  | VIII | Statistical quality control | 0.12 |
|  | IX | End Of Line | 0.20 |
|  | Total |  | 6.21 |

Table S 2: North macedonia, Bulgary and North Macedonia (no fossil) electricity mixes

|  | MK | BG | MK (no fossil) |
| --- | --- | --- | --- |
| Coal | 39.7% | 36.0% | / |
| Hydroelectric | 26.4% | 10.7% | 88.9% |
| Solar PV | 0.5% | 3.1% | 1.5% |
| Wind | 1.9% | 3.0% | 6.3% |
| Biofuels | 1.0% | 5.4% | 3.2% |
| Natural gas | 28.1% | 6.4% | / |
| Oil | 2.5% | 0.8% | / |
| Nuclear | / | 34.7% | / |

Table S 3: List of packaging materials involved in the system and transportation distances.

| Unit process | Material | Amount (kg) | Truck Distance (km) |
| --- | --- | --- | --- |
| (I) | Polystyrene | 6.69E-03 | 1935 |
| (I) | LDPE | 3.01E-04 | 1200 |
| (I) | LDPE | 1.95E-03 | 1650 |
| (I) | Plastic / aluminum | 2.62E-03 | 1500 |
| (I) | Corrugated board box | 1.05E-02 | 200 |
| (V) | Adhesive label | 5.12E-02 | 1252 |
| (IX) | PET | 3.20E-02 | 1252 |
| (IX) | Steel | 3.20E-04 | 1327 |
| (IX) | Polyethylene | 1.03E-02 | 5.2 |
| (IX) | Polyethylene | 1.66E-03 | 5.2 |

Table S 4: Inventory for the production of secondary coppe (Zhang et al., 2021)

| Material inventory of the smelting process. |  |  |  |
| --- | --- | --- | --- |
| Type | Material | Unit | Quantity |
| Input | Copper byproduct | kg | 3.4 |
|  | Copper slag concentrate | kg | 464.4 |
|  | Dust | kg | 1.4 |
|  | Hard coal | kg | 710.1 |
|  | Quicklime | kg | 162 |
|  | Limestone powder | kg | 162 |
|  | Oxygen (gaseous) | kg | 137.5 |
|  | Quartz sand | kg | 135 |
|  | Copper-based mixed waste | kg | 3240 |
|  | Copper scrap | kg | 8.1 |
|  | Scrap | kg | 483.9 |
|  | Slag and ash | kg | 125763.4 |
|  | Transport | kg·km | 275.1 |
|  | Electricity | kwh | 194.4 |
| Output | Ash 1 | kg | 94.5 |
|  | Black copper | kg | 1350 |
|  | Copper-containing slag | kg | 2932.7 |
|  | Ash 2 | kg | 0.4 |
|  | Carbon dioxide | kg | 977.4 |
| Material inventory of the refining process |  |  |  |
| Type | Material | Unit | Quantity |
| Input | Activated coke | kg | 2.4 |
|  | Anodes (used) | kg | 210 |
|  | Black copper | kg | 1350 |
|  | Clay bricks | kg | 5.9 |
|  | Hard coal | kg | 23.8 |
|  | Heavy fuel oil | kg | 85.5 |
|  | Magnesium powder | kg | 5.9 |
|  | Oxygen (gaseous) | kg | 0.3 |
|  | Quartz sand | kg | 48.8 |
|  | Sodium silicate | kg | 9.6 |
|  | Transport | kg·km | 13082.8 |
| Output | Ash | kg | 31.9 |
|  | Blister copper | kg | 1200 |
|  | Dust | kg | 1.4 |
|  | Slag | kg | 190.2 |
|  | Ash | kg | 0.1 |
|  | Carbon dioxide | kg | 151.7 |
| Material inventory of the electrolysis process. |  |  |  |
| Type | Material | Unit | Quantity |
| Input | Activated coke | kg | 2.4 |
|  | Anodes (used) | kg | 210 |
|  | Black copper | kg | 1350 |
|  | Clay bricks | kg | 5.9 |
|  | Hard coal | kg | 23.8 |
|  | Heavy fuel oil | kg | 85.5 |
|  | Magnesium powder | kg | 5.9 |
|  | Oxygen (gaseous) | kg | 0.3 |
|  | Quartz sand | kg | 48.8 |
|  | Sodium silicate | kg | 9.6 |
|  | Transport | kg·km | 13082.8 |
| Output | Ash | kg | 31.9 |
|  | Blister copper | kg | 1200 |
|  | Dust | kg | 1.4 |
|  | Slag | kg | 190.2 |
|  | Ash | kg | 0.1 |
|  | Carbon dioxide | kg | 151.7 |

Table S 5: Pedigree matrix.

|  | Material |  | Energy MK |  | Energy BG |  | Waste |  | Secondary Cu and Al |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U1 (reliability) | 1 | 0 | 1.10 | 9.1E-03 | 1 | 0 | 1.10 | 9.1E-03 | 1.05 | 2.4E-03 |
| U2 (completeness) | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1.10 | 9.1E-03 |
| U3 (temporal c.) | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| U4 (geographical c.) | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1.05 | 2.4E-03 |
| U5 (technological c.) | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 2.00 | 4.8E-01 |
| Ub | 1.05 | 2.4E-03 | 1.05 | 2.4E-03 | 1.05 | 2.4E-03 | 1.05 | 2.4E-03 | 1.05 | 2.4E-03 |
| SD |  | 1.05E+00 |  | 1.11E+00 |  | 1.05E+00 |  | 1.11E+00 |  | 2.02E+00 |

- Reliability: The information related to the amount of materials involved in the system process and the energy consumed in the BG plant has been provided by the company as high-quality data (score = 1). The electricity derived from BG was calculated by normalising the annual flow (directly measured by the company) to the functional unit (score = 1). The electricity consumed by MK plant refers to monthly consumption data (from April to August 2023), considered representative for the reference year in consistency with primary information, and allocated to the amount of DC-LINKS produced. Consistently, a score of 3 has been assigned. Same for the waste generated within the system boundaries, a score of 3 was assigned since the data have been estimated and have not been verified. Finally, since the modelling of components containing secondary materials (i.e., aluminium and copper) required the modification of the ecoinvent proxy (aluminium) and extraction af a new inventory from literature (copper), a score of 2 has been assigned.

- Completeness: The data used for materials, energy and waste were considered representative for the site analyzed in the case study; for this reason, the selected score was 1. For secondary copper and aluminium, the more severe score 4 has been assigned, since it was not certain of the representativeness.

- Temporal correlation: all the data used have been sourced in the last three years, so score 1 has been assigned to all the involved flows.

- Geographical correlation: The data used for materials, energy and waste were considered representative for the site under study (score = 1). For secondary copper and alluminium, the more severe score 4 has been assigned, since the information derived from areas with slightly similar production conditions.

- Technological correlation: The data used for materials, energy and waste were considered representative for the site under study (score = 1). Score 5 has been conservatively assigned to secondary copper and aluminium, since the data used were not certain to derive from the same technology.

Table S 6: Life Cycle Impact Assessment of Baseline Scenario

|  | CC (kg CO2 eq) | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | POFh (kg NOx eq) | PMF (kg PM2.5 eq) | POFe (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | ME (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRD (kg Cu eq) | FD (kg oil eq) | WD (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Materials | 88.2% | 91.6% | 84.0% | 92.0% | 97.9% | 92.3% | 98.1% | 98.7% | 96.8% | 99.9% | 99.9% | 99.9% | 100.0% | 100.0% | 52.9% | 99.7% | 87.8% | 99.4% |
| Energy | 0.8% | 4.0% | 0.7% | 1.4% | 0.4% | 1.4% | 0.2% | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 43.4% | 0.1% | 0.6% | 0.0% |
| N2 | 1.3% | 0.6% | 12.8% | 0.5% | 0.3% | 0.5% | 0.3% | 0.6% | 1.7% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 2.0% | 0.0% | 1.8% | 0.4% |
| Transport | 3.6% | 2.2% | 1.4% | 5.4% | 1.3% | 5.2% | 1.3% | 0.2% | 0.4% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 1.6% | 0.1% | 5.8% | 0.0% |
| Waste | 6.1% | 1.6% | 1.1% | 0.6% | 0.2% | 0.6% | 0.1% | 0.4% | 0.7% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 4.0% | 0.0% |
| Spray recovery | -2.0% | -1.1% | -7.3% | -2.3% | -0.7% | -2.2% | -0.7% | -1.1% | -2.5% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -2.0% | -1.8% | -2.7% | -0.4% |
| Total | 9.5E+00 | 1.1E-05 | 4.7E-01 | 4.6E-02 | 6.6E-02 | 4.8E-02 | 1.9E-01 | 2.2E-02 | 5.6E-04 | 4.3E+02 | 1.5E+01 | 1.9E+01 | 4.8E+00 | 2.3E+02 | 1.1E+00 | 9.9E-01 | 1.9E+00 | 1.6E+00 |
| SD | 11% | 14% | >100% | 20% | 14% | 21% | 14% | 18% | 15% | 51% | 25% | 22% | 62% | >100% | 25% | 26% | 27% | >100% |

Table S 7: Life Cycle Impact Assessment of Scenario 1.

|  | CC (kg CO2 eq) | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | POFH (kg NOx eq) | PMF (kg PM2.5 eq) | POFE (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | MEP (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRD (kg Cu eq) | FD (kg oil eq) | WD (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Materials | 51.4% | 80.0% | 40.8% | 69.1% | 85.3% | 70.0% | 85.4% | 86.6% | 71.2% | 98.9% | 99.9% | 99.8% | 100% | 100% | 48.0% | 99.5% | 46.0% | 97.8% |
| Energy | 42.2% | 16.2% | 51.8% | 26.0% | 13.2% | 25.2% | 13.2% | 12.4% | 26.7% | 1.0% | 0.1% | 0.1% | 0.0% | 0.0% | 48.6% | 0.3% | 47.9% | 1.7% |
| N2 | 0.8% | 0.5% | 6.2% | 0.4% | 0.3% | 0.4% | 0.2% | 0.5% | 1.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 1.8% | 0.0% | 0.9% | 0.4% |
| Transport | 2.1% | 2.0% | 0.7% | 4.1% | 1.1% | 4.0% | 1.1% | 0.1% | 0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 1.4% | 0.1% | 3.0% | 0.0% |
| Waste | 3.6% | 1.4% | 0.5% | 0.5% | 0.1% | 0.5% | 0.1% | 0.3% | 0.5% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 2.1% | 0.0% |
| Spray recovery | -1.2% | -1.0% | -3.6% | -1.7% | -0.6% | -1.7% | -0.6% | -1.0% | -1.8% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -1.8% | -1.8% | -1.4% | -0.4% |
| Total | 1.6E+01 | 1.3E-05 | 1.0E+00 | 6.2E-02 | 7.6E-02 | 6.4E-02 | 2.2E-01 | 2.5E-02 | 7.6E-04 | 4.3E+02 | 1.5E+01 | 1.9E+01 | 4.8E+00 | 2.3E+02 | 1.3E+00 | 9.9E-01 | 3.7E+00 | 1.6E+00 |
| SD | 11% | 14% | >100% | 20% | 14% | 22% | 14% | 15% | 16% | 45% | 25% | 23% | 40% | >100% | 24% | 26% | 24% | >100% |

Table S 8: Life Cycle Impact Assessment of Scenario 2.

|  | CC (kg CO2 eq) | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | POFH (kg NOx eq) | PMF (kg PM2.5 eq) | POFE (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | MEP (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRD (kg Cu eq) | FD (kg oil eq) | WD (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Materials | 86.5% | 89.6% | 77.0% | 87.2% | 95.7% | 87.9% | 95.8% | 97.2% | 95.4% | 99.8% | 99.8% | 99.8% | 100.0% | 100.0% | 36.7% | 99.5% | 87.4% | 99.4% |
| Energy | 1.0% | 4.9% | 1.0% | 2.3% | 0.7% | 2.1% | 0.5% | 0.3% | 0.6% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 58.2% | 0.2% | 0.6% | 0.0% |
| N2 | 1.5% | 0.8% | 18.4% | 0.8% | 0.6% | 0.8% | 0.6% | 1.3% | 2.4% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 2.7% | 0.0% | 1.9% | 0.5% |
| Transport | 4.1% | 2.8% | 2.1% | 8.7% | 2.6% | 8.3% | 2.8% | 0.3% | 0.5% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 2.1% | 0.1% | 6.0% | 0.0% |
| Waste | 6.9% | 2.0% | 1.5% | 1.0% | 0.3% | 0.9% | 0.3% | 0.9% | 1.0% | 0.1% | 0.2% | 0.2% | 0.0% | 0.0% | 0.2% | 0.1% | 4.1% | 0.0% |
| Spray recovery | -2.3% | -1.4% | -10.6% | -3.6% | -1.4% | -3.5% | -1.5% | -2.4% | -3.5% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -2.7% | -3.3% | -2.8% | -0.4% |
| Total | 8.3E+00 | 9.1E-06 | 3.2E-01 | 2.9E-02 | 3.2E-02 | 3.0E-02 | 8.3E-02 | 1.0E-02 | 3.9E-04 | 1.9E+02 | 5.4E+00 | 6.7E+00 | 2.2E+00 | 8.0E+01 | 8.4E-01 | 5.3E-01 | 1.9E+00 | 1.5E+00 |
| SD | 20% | 23% | >100% | 17% | 23% | 17% | 22% | 41% | 17% | 47% | 39% | 37% | 65% | >100% | 40% | 21% | 34% | >100% |

Table S 9: Life Cycle Impact Assessment of Scenario 3.

|  | CC (kg CO2 eq) | OD (kg CFC11 eq) | IR (kBq Co-60 eq) | POFH (kg NOx eq) | PMF (kg PM2.5 eq) | POFE (kg NOx eq) | TA (kg SO2 eq) | FE (kg P eq) | MEP (kg N eq) | TET (kg 1,4-DCB) | FET (kg 1,4-DCB) | MET (kg 1,4-DCB) | HTc (kg 1,4-DCB) | HTnc (kg 1,4-DCB) | LO (m2a crop eq) | MRD (kg Cu eq) | FD (kg oil eq) | WD (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Materials | 87.0% | 91.5% | 83.0% | 91.6% | 97.9% | 92.0% | 98.1% | 98.7% | 96.7% | 99.9% | 99.9% | 99.9% | 100.0% | 100.0% | 53.5% | 99.7% | 86.8% | 99.4% |
| Energy | 0.9% | 4.0% | 0.8% | 1.5% | 0.4% | 1.4% | 0.2% | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 42.8% | 0.1% | 0.7% | 0.0% |
| N2 | 1.5% | 0.6% | 13.6% | 0.5% | 0.3% | 0.5% | 0.3% | 0.6% | 1.7% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 2.0% | 0.0% | 2.0% | 0.4% |
| Transport | 3.9% | 2.3% | 1.5% | 5.7% | 1.3% | 5.5% | 1.3% | 0.2% | 0.4% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 1.5% | 0.1% | 6.3% | 0.0% |
| Waste | 6.7% | 1.6% | 1.1% | 0.7% | 0.2% | 0.6% | 0.1% | 0.4% | 0.7% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 4.3% | 0.0% |
| Spray recovery | -2.2% | -1.1% | -7.8% | -2.4% | -0.7% | -2.3% | -0.7% | -1.1% | -2.5% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -1.9% | -1.8% | -2.9% | -0.4% |
| Total | 8.7E+00 | 1.1E-05 | 4.4E-01 | 4.4E-02 | 6.4E-02 | 4.6E-02 | 1.9E-01 | 2.2E-02 | 5.5E-04 | 4.3E+02 | 1.5E+01 | 1.9E+01 | 4.7E+00 | 2.3E+02 | 1.2E+00 | 9.9E-01 | 1.8E+00 | 1.6E+00 |
| SD | 11% | 14% | >100% | 21% | 15% | 22% | 15% | 15% | 15% | 49% | 26% | 24% | 39% | >100% | 25% | 26% | 26% | >100% |

Table S 10: Comparison between ReCiPe 2016 and CML impacts

|  | ReCipe 2016 v 1.08 | CML v 3.11 | SD (ReCiPe) |
| --- | --- | --- | --- |
| CC | 9.50E+00 | 9.18E+00 | 11% |
| OD | 1.10E-05 | 3.41E-06 | 14% |
| TA | 1.90E-01 | 2.28E-01 | 14% |
| TET | 4.30E+02 | 9.05E-01 | 51% |
| FET | 1.50E+01 | 1.18E+02 | 25% |
| MET | 1.90E+01 | 1.46E+05 | 22% |
| HTc | 4.80E+00 | 2.87E+02 | 62% |
| HTnc | 2.30E+02 | 3.07E+02 | >100% |

The CC category shows a difference of 3% between the ReCiPe and CML method. The other categories show more or less significant differences. The largest discrepancies are observed in the toxicity-related categories, which are known to exhibit higher standard deviations due to the intrinsic uncertainty of the LCIA method (Chen et al., 2021).

Freshwater eutrophication and marine eutrophication are not comparable due to the use of different units (kg P eq and kg N eq vs kg PO₄ eq). Similarly, mineral and fossil resource scarcity cannot be directly compared to abiotic depletion for the same reason.

Bibliography

Chen, X., Matthews, H. S., & Griffin, W. M. (2021). Uncertainty caused by life cycle impact assessment methods: Case studies in process-based LCI databases. Resources, Conservation and Recycling, 172. https://doi.org/10.1016/j.resconrec.2021.105678

Zhang, W., Li, Z., Dong, S., Qian, P., Ye, S., Hu, S., Xia, B., & Wang, C. (2021). Analyzing the environmental impact of copper-based mixed waste recycling-a LCA case study in China. Journal of Cleaner Production, 284. https://doi.org/10.1016/j.jclepro.2020.125256