# Annex A. Techno-economic uncertainty and cost allocation methodology

## A.1 Overview of the economic assessment framework

This annex provides the detailed methodology supporting the economic assessment presented in Section 4.4. The objective is to quantify the uncertainty associated with the production cost of sustainable aviation fuel (SAF) from camelina oil and used cooking oil (UCO), using a mass-based co-product allocation approach and a Monte Carlo simulation framework. The approach evaluates variability in feedstock price, conversion costs, operating expenditures and capital recovery, and propagates their combined uncertainty through the SAF cost model.

## A.2 Co-product mass allocation

Hydroprocessing of lipid feedstocks yields multiple fuel fractions. Per tonne of feedstock, the process produces 53.5% kerosene, 14.0% diesel, 12.7% naphtha and 8.8% LPG, corresponding to a total output of 0.89 t. Costs were allocated across these product streams according to their respective mass shares, reflecting the multi-output nature of the hydroprocessing pathway and avoiding over-attribution of costs to the SAF fraction.

The unit production cost of SAF was calculated as:

CSAF=Cfeed+Cconv+CO&M+CCAPEXYtot

where:
Cfeed is the feedstock cost per tonne of oil;
Cconv represents variable conversion costs;
CO&M denotes fixed operating costs;
CCAPEX is the annualised capital charge;
and Ytot=0.89t products per t feedstock is the mass yield from hydroprocessing (53.5% kerosene, 14% diesel, 12.7% naphtha and 8.8% LPG).

Because hydroprocessing delivers multiple fuel fractions, all costs were allocated on a mass basis:

Ci=CSAF×mijmj

where miis the mass of product stream i. This ensures that SAF carries only its proportional share of the total cost rather than being attributed all feedstock and processing expenses.

## A.3 Cost structure and parameter uncertainties

Feedstock price was represented as the principal source of uncertainty. Operating expenditures were calibrated such that feedstock accounts for approximately 80% of OPEX, consistent with published HEFA techno-economic literature. Feedstock prices were modelled using lognormal distributions centered on 1,326 € t⁻¹ for camelina oil and 950 € t⁻¹ for UCO, with coefficients of variation between 20% and 25%, derived from the feedstock price ranges reported in de Jong et al. (2015).

Conversion costs, fixed O&M expenditures and capital recovery parameters were assigned probability distributions derived from inter-study variability reported in de Jong et al. (2015; 2017), and NREL (2022). To enable a transparent uncertainty analysis, the coefficients of variation (CV) and standard deviations (SD) used for the Monte Carlo simulation were derived from the inter-study variability and ±% ranges reported in these TEA sources. Therefore, coefficients of variation and standard deviations were inferred from their reported ±20–50% variability. Conversion costs were modelled as lognormal variables with CV = 0.30, fixed O&M costs as truncated normal distributions with SD = 20 € t⁻¹, and annualised CAPEX as truncated normal distributions with SD = 40–50 € t⁻¹.

A consolidated overview of the distributions and parameter values used in the Monte Carlo simulation is provided in Table A1.

### Table A1. Probability distributions used for Monte Carlo simulation

| Cost component | Distribution type | Mean value (€/t feedstock) | Variability parameter | Motivation / Source |
| --- | --- | --- | --- | --- |
| Feedstock price – Camelina oil | Lognormal | 1,326 | CV = 0.20 | Price range for vegetable oils (de Jong et al., 2015) |
| Feedstock price – UCO | Lognormal | 950 | CV = 0.25 | Higher market volatility for waste lipids (de Jong et al., 2015) |
| Conversion cost | Lognormal | 400 | CV = 0.30 | ±20–50% range for energy and hydrogen inputs (de Jong et al., 2015; NREL, 2022) |
| Fixed O&M cost | Truncated normal | 100 | SD = 20 | Based on NREL SOT O&M variability (~20%) |
| Annualised CAPEX | Truncated normal | 150 | SD = 40–50 | Inter-study capital investment variability (de Jong et al., 2015; NREL, 2022) |

All values refer to costs per tonne of feedstock processed.

## A.4 Monte Carlo simulation approach

A Monte Carlo simulation with 50,000 iterations was implemented to propagate uncertainty through the cost model. Each iteration draws random values for feedstock, conversion, operating and capital costs according to their respective probability distributions.

Ck=fCfeed,kCconv,kCO&M,kCCAPEX,k,k=1,...,50,000

where each component C⋅,krepresents one stochastic draw. Input distributions for conversion and O&M costs followed recommended approaches in TEA literature using empirical variability and expert-informed ranges.

To quantify the relative influence of each uncertain parameter on total SAF production cost, the Spearman rank correlation coefficient was calculated after completing the simulation for both feedstocks. The Spearman coefficient measures the strength of the monotonic relationship between each model input and the resulting SAF cost. Unlike linear correlation, Spearman correlation does not assume linearity and is therefore appropriate for skewed, non-normal or non-linear distributions such as feedstock prices. Parameters with coefficients closer to ±1 exert the strongest influence on total cost variability.

## A.5 Results of the uncertainty analysis

Figure A1 presents the probability density functions of SAF production cost for camelina oil and UCO derived from the Monte Carlo simulation. Figures A2 and A3 present the corresponding Spearman sensitivity analysis for each pathway. Table A2 reports descriptive statistics for both cost distributions.

Figure A1. Monte Carlo distribution (50,000 iterations) of SAF production cost for camelina oil and UCO.

Figure A2. Sensitivity of SAF production cost to uncertain parameters for the camelina pathway (Spearman rank correlation).

Figure A3. Sensitivity of SAF production cost to uncertain parameters for the UCO pathway (Spearman rank correlation).

### Table A2. Summary statistics of simulated SAF production cost (€ t-1)

| Statistic | Camelina Feedstock | Camelina Process | Camelina SAF | UCO Feedstock | UCO Process | UCO SAF |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 1326 | 650 | 2220 | 948 | 651 | 1797 |
| Median | 1299 | 636 | 2193 | 919 | 636 | 1768 |
| 5th percentile | 938 | 464 | 1723 | 635 | 464 | 1350 |
| 95th percentile | 1804 | 886 | 2810 | 1343 | 889 | 2335 |
| Standard deviation | 266 | 130 | 333 | 233 | 131 | 305 |

The results show that the mean SAF cost from camelina oil is approximately 2,220 € t⁻¹, with a 5–95% uncertainty range of 1,723–2,810 € t⁻¹, while SAF derived from UCO exhibits a mean cost of 1,797 € t⁻¹, ranging from 1,350–2,335 € t⁻¹. In both cases, feedstock price remains the dominant source of cost variability, accounting for over 85% of total variance, whereas conversion and O&M costs contribute marginally. This refined approach demonstrates that simple linear scaling of feedstock price substantially understates the inherent techno-economic uncertainty associated with SAF production.

## A.6 Boxplot analysis of cost components

To provide an additional visual comparison of the magnitude and variability of each cost category, boxplots were generated for three aggregated variables derived from each Monte Carlo iteration: feedstock cost, the grouped process costs (conversion + O&M + CAPEX), and the resulting SAF production cost after mass allocation. Figure A4 displays these distributions for both camelina and UCO. The boxplots clearly show that feedstock cost dominates both the level and the variance of total SAF production cost, while process-related costs exhibit considerably narrower distributions.

Figure A4. Boxplots of feedstock cost, aggregated process costs (conversion + O&M + CAPEX), and total SAF production cost for camelina oil and UCO (50,000 Monte Carlo iterations). Each box includes 25-75% of the quantile, and the whiskers are the min-max value.